| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0147-v1 |
| --- | --- |

| Title: | EE2-1.13 and 1.14: CCCM using non-downsampled luma samples |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Che-Wei Kuo, Xiaoyu Xiu, Wei Chen, Ning Yan, Changyue Ma, Xianglin Wang, Vadim Seregin, Yao-Jen Chang, Bappaditya Ray, Marta Karczewicz | Email: | jhuhong-jheng@kwai.com, vseregin@qti.qualcomm.com |
| Source: | Kwai Inc. and Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports results of EE2 Test 1.13 and Test 1.14, where CCCM using non-downsampled luma samples, i.e. chroma samples are directly predicted from the reconstructed luma samples without downsampling, is tested. The tested methods were implemented on top of ECM-7.0, and the simulation results are summarized as follows:

| Class A to E | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.13 | -0.03% | -0.19% | -0.24% | 100% | 100% | -0.03% | -0.19% | -0.31% | 100% | 100% |
| 1.14 | -0.03% | 0.01% | -0.04% | 101% | 100% | 0.01% | -0.09% | -0.20% | 101% | 99% |

| Class F | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.13 | -0.22% | -0.96% | -0.64% | 100% | 99% | -0.26% | -0.88% | -0.68% | 100% | 100% |
| 1.14 | -0.23% | -0.67% | -0.34% | 101% | 99% | -0.23% | -0.61% | -0.38% | 101% | 100% |

| Class TGM | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.13 | -1.27% | -1.43% | -1.79% | 100% | 100% | -0.33% | -0.58% | -0.59% | 100% | 99% |
| 1.14 | -1.16% | -1.18% | -1.57% | 101% | 101% | -0.29% | -0.50% | -0.40% | 101% | 100% |

# Introduction

At JVET-AB meeting, CCCM using non-downsampled luma samples [1] and no luma subsampling for CCCM [2] were proposed to predict the chroma samples from the reconstructed luma samples, i.e., without downsampling. The two methods were included in Exploration Experiment (EE) 2 as Test 1.13 and Test 1.14 for further evaluation. This contribution reports the corresponding results of the two EE tests.

# Proposal

In Test 1.13 and Test 1.14, a CCCM mode with 3x2 filter using non-downsampled luma samples is proposed, which consists of 6-tap spatial terms, four nonlinear terms and a bias term. The 6-tap spatial terms correspond to 6 neighboring luma samples (i.e., L0, L1, …, L5) around the chroma sample (i.e., C) to be predicted, the four non-linear terms are derived from the samples L0, L1, L2, and L3.

C=i=05αi∙(Li-offsetLuma)+i=69αi∙(((Li-4-offsetLuma)2+β)≫bitDepth)+α10∙β+ offsetChroma

Fig. 1 Non-downsampled luma samples.

where αi is the coefficient, β is the offset. Same to the existing CCCM design, up to 6 lines/columns of chroma samples above and left to the current CU are applied to derive the filter coefficients. The filter coefficients are derived based on the same LDL decomposition method used in CCCM. The proposed method is signaled as an additional CCCM model besides the existing one, when the CCCM is selected, one single flag is signaled and used for both two chroma components to indicate whether the default CCCM model or the proposed CCCM model is applied. Additionally, SPS signaling is introduced to indicate whether the CCCM using non-downsampled luma samples is enabled. At encoder side, in Test 1.13, one extra encoding speed-up logic is involved which adaptively adjusts the number of RD calculations according to the SATD values of the normal intra modes, the default CCCM and the proposed CCCM with non-downsampled luma samples.

# Simulation results

The proposed techniques were implemented on ECM-7.0 reference software [3] and tested by following the common test condition (CTC) as specified in [4].

## Test 1.13

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.06% | -0.08% | 100% | 99% |
| Class A2 | -0.23% | -0.35% | -0.28% | 101% | 100% |
| Class B | 0.01% | -0.27% | -0.33% | 100% | 100% |
| Class C | -0.02% | -0.27% | -0.35% | 100% | 100% |
| Class E | 0.04% | 0.09% | -0.06% | 100% | 99% |
| Overall | -0.03% | -0.19% | -0.24% | 100% | 100% |
| Class D | 0.00% | -0.46% | -0.33% | 100% | 99% |
| Class F | -0.22% | -0.96% | -0.64% | 100% | 99% |
| Class TGM | -1.27% | -1.43% | -1.79% | 100% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.16% | -0.22% | 100% | 100% |
| Class A2 | -0.23% | -0.23% | -0.49% | 100% | 100% |
| Class B | 0.01% | -0.09% | -0.34% | 100% | 100% |
| Class C | 0.00% | -0.31% | -0.21% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.19% | -0.31% | 100% | 100% |
| Class D | 0.00% | -0.17% | -0.13% | 100% | 100% |
| Class F | -0.26% | -0.88% | -0.68% | 100% | 100% |
| Class TGM | -0.33% | -0.58% | -0.59% | 100% | 99% |

## Test1.14

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.11% | -0.12% | 101% | 99% |
| Class A2 | -0.23% | -0.07% | 0.07% | 102% | 100% |
| Class B | 0.02% | -0.04% | -0.08% | 101% | 100% |
| Class C | 0.00% | -0.12% | -0.21% | 101% | 100% |
| Class E | 0.02% | 0.25% | 0.23% | 101% | 100% |
| Overall | -0.03% | 0.01% | -0.04% | 101% | 100% |
| Class D | 0.01% | -0.25% | -0.23% | 101% | 98% |
| Class F | -0.23% | -0.67% | -0.34% | 101% | 99% |
| Class TGM | -1.16% | -1.18% | -1.57% | 101% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | -0.21% | -0.28% | 101% | 101% |
| Class A2 | -0.20% | -0.05% | -0.33% | 99% | 94% |
| Class B | 0.06% | -0.03% | -0.17% | 101% | 100% |
| Class C | 0.03% | -0.10% | -0.10% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.09% | -0.20% | 101% | 99% |
| Class D | 0.00% | 0.10% | 0.01% | 101% | 100% |
| Class F | -0.23% | -0.61% | -0.38% | 101% | 100% |
| Class TGM | -0.29% | -0.50% | -0.40% | 101% | 100% |

# References

- H.-J. Jhu, C.-W. Kuo, X. Xiu, W. Chen, N. Yan, C. Ma, X. Wang, V. Seregin, Y.-J. Chang, B. Ray, M. Karczewicz, “ Non-EE2: CCCM using non-downsampled luma samples”, JVET-AB0180, Mainz, DE, 20–28 October 2022.

- V. Seregin, Y.-J. Chang, B. Ray, M. Karczewicz, “ Non-EE2: No luma subsampling for CCCM”, JVET-AB0187, Mainz, DE, 20–28 October 2022.

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Apr. 2022.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).