| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0137-v1 |
| --- | --- |

| Title: | [AHG8] SEI for encoder optimization information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Dong-Gyu Gwak Hendry Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | chulkeun.kim@lge.com |
| Source: | LG electronics |  |  |

_____________________________

- Abstract

At the last meeting, AHG8 provided a description of optimization techniques for video encoders and receiving systems [1]. However, a method for identifying optimization has not been described. This contribution proposes the SEI message to indicate whether encoding optimization is applied and what optimization methods is used.

Problem description

The purpose of user or machine while exploiting an encoded video can change, and the appropriate optimization method for that purpose can change accordingly. Since the encoded video is used for multiple purposes, there can be a mismatch between the optimized method and the intended use. Information to indicate whether the optimization is applied for any purpose and also what optimization method is utilized needs to be identified to preven the abovementioned mismatch.

Proposed solution

This contribution defines information identifying whether optimization is applied, what optimization methods is used, and the scope of optimization that is applied.

Table 1. SEI for encoder optimization

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| optimization_cancel_flag | u(1) |
| if(!optimization_cancel_flag){ |  |
| optimization_method | u(16) |
| optimization_persistence_flag | u(1) |
| } |  |
| } |  |

optimization_cancel_flag equal to 1 specifies that the persistence of the previous applied optimization is cancelled. optimization_cancel_flag equal to 0 indicates that optimization_persistence_flag and optimization_method follow.

optimization_method indicates the optimization method as specified in Table 2.(The optimization methods specified in Table 2 are described in a draft technical report [1])

Table 2 – Definition of optimization_method

| Value | Interpretation |
| --- | --- |
| optimization_method = = 0 | May be used as determined by the application |
| optimization_method > 0 && ( optimization_method & 0x01 ) = = 0 | No pre-processing methods of ROI |
| ( optimization_method & 0x01 ) != 0 | With pre-processing methods of ROI |
| optimization_method > 0 && ( optimization_method & 0x02 ) = = 0 | No pre-processing methods of foreground and background |
| ( optimization_method & 0x02 ) != 0 | With pre-processing methods of foreground and background |
| optimization_method > 0 && ( optimization_method & 0x04 ) = = 0 | No temporal subsampling |
| ( optimization_method & 0x04 ) != 0 | With temporal subsampling |
| optimization_method > 0 && ( optimization_method & 0x08 ) = = 0 | No spatial subsampling |
| ( optimization_method & 0x08 ) != 0 | With spatial subsampling |
| optimization_method > 0 && ( optimization_method & 0x10 ) = = 0 | No quantization parameter adaption for Region of Interest coding |
| ( optimization_method & 0x10 ) != 0 | With quantization parameter adaption for Region of Interest coding |
| optimization_method > 0 && ( optimization_method & 0x20 ) = = 0 | No quantization step changes for temporal layers |
| ( optimization_method & 0x20 ) != 0 | With quantization step changes for temporal layers |

optimization_persistence_flag specifies the persistence of the optimization method that is indicated by the optimization_method. optimization_persistence_flag equal to 0 specifies that the optimization method identified by the optimization_method may be applied for the current picture only. optimization_persistence_flag equal to 1 specifies that the optimization method identified by the optimization_method may be applied for the current picture and all subsequent pictures.

References

- ISO/IEC JTC 1/SC 29/WG 5, “Optimization of encoders and receiving systems for machine analysis of coded video content (draft 1),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 output document JVET-AC2030, 29th JVET meeting, by teleconference, January 2023

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).