| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting: Antalya, TR, 21–28 April 2023 | Document: JVET-AD0198-v2 |
| --- | --- |

| Title: | EE2-1.20c: Combination of Test1.14 and Test1.19 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Po-Han Lin, Jian-Liang Lin, Vadim Seregin, Marta Karczewicz (Qualcomm Incorporated) Fan Wang, Lai Zhang, Yue Yu, Haoping Yu, Dong Wang (OPPO) Yanzhuo Ma, Junyan Huo, Fuzheng Yang (Xidian Univ.) | Email: | pohanlin@qti.qualcomm.com wangfan6@oppo.com yzma@mail.xidian.edu.cn |
| Source: | Qualcomm Incorporated, OPPO, Xidian Univ. |  |  |

_____________________________

# Abstract

This document reports the results of Test1.20c which is the combination of Test1.14 [1] and Test1.19 [2] in EE2 [3] on intra template matching (IntraTMP). In, Test1.19, it is proposed to add multiple template type (Left template and above template) and fusion candidate. In Test1.14, it is proposed to extend the search area in intra template matching process.

# Introduction

In this proposal, the simulation results of EE2-1.20c on IntraTMP are reported. Test1.20c combines the method of Test1.19 which proposed to use multiple template type and fusion, and Test1.14 which proposed to extend the search area in intra template matching process.

# Experimental results

The tables below summarize the coding efficiency and runtime impact of the EE2-1.20c test in ECM 8.0 following the common test conditions [4].

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM8 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.02% | -0.07% | 104% | 110% |
| Class A2 | -0.84% | -0.94% | -0.94% | 103% | 112% |
| Class B | -0.69% | -0.69% | -0.68% | 104% | 112% |
| Class C | -0.28% | -0.29% | -0.31% | 102% | 106% |
| Class E | -1.27% | -1.16% | -1.24% | 103% | 111% |
| Overall | -0.62% | -0.60% | -0.63% | 103% | 110% |
| Class D | -0.45% | -0.58% | -0.63% | 102% | 105% |
| Class F | -0.19% | -0.19% | -0.12% | 101% | 106% |
| Class TGM | -0.58% | -0.38% | -0.39% | 100% | 111% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM8 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.27% | -0.50% | -0.25% | 102% | 101% |
| Class C | -0.12% | -0.11% | 0.13% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.12% | 0.14% | -0.25% | 101% | 98% |
| Class F | -0.18% | -0.46% | -0.20% | 101% | 101% |
| Class TGM | -0.47% | -0.49% | -0.52% | 100% | 100% |

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

OPPO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] 	Y. Ma, J. Huo, H. Du, H. Zhang, Z. Zhang, W. Qiao, F. Yang (Xidian), F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO), “EE2-1.14c combination test of EE2-1.14a and EE2-1.14b”, JVET-AD0075, Apr. 2023.

[2] P.-H Lin, J.-L Lin, V. Seregin, M. Karczewicz, “EE2-1.19: IntraTMP with multiple modes”, JVET-AD0194, Apr. 2023.

[3] V. Seregin, J. Chen etc, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-AC2024, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 29th Meeting, by teleconference, 11-20 January 2023.

[4] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.