| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0056-v2 |
| --- | --- |

| Title: | EE1-1.2.2: Content-adaptive LOP filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruiying Yang Maria Santamaria Francesco Cricri Honglei Zhang Jani Lainema Miska M. Hannuksela Antti Hallapuro Nokia Technologies, Tampere, Finland | Email: | {ruiying.yang, maria.santamaria_gomez, francesco.cricri, honglei.1.zhang, jani.lainema, miska.hannuksela, antti.hallapuro}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports the results for the EE1-1.2.2 test which studies an integrated signalling mechanism for the content-adaptive neural network loop-filter in JVET-AE0093, and uses as basis the LOP1 loop-filter. It is reported that a new type of adaptation parameter set (APS), called neural network filter update APS, was designed and implemented. In JVET-AE0093, the inference was conducted on top of NNVC 5.0, using SADL library with half rounding enabled. The attained coding gains were -1.21% (Y), -6.43% (Cb) and -5.52% (Cr). Nevertheless, it was observed that the deactivation of half rounding in SADL library led to a reduction in coding gains by approximately 1%. Therefore, this contribution also studies data-driven quantization as means to improve the inference performance with SADL library and int16 precision. It is reported that, the average coding gains of this approach are -1.19% (Y), -6.40% (Cb) and -5.49% (Cr) with respect to the NNVC 5.0 anchor (NN intra tool ON and LOP1 tool ON). Furthermore, a simulation of signalling the update for each intra frame was performed and it is reported that the average coding gains are -0.34% (Y), -5.52% (Cb) and -4.75% (Cr) with respect to the NNVC 5.0 anchor.

# Introduction

The content-adaptive approach for neural network loop-filter, as detailed in JVET-AE0093 [1], is achieved by means of overfitting one of the models of default LOP1 filter in NNVC 5.0. The generated weight-updates are coded with the MPEG-NNR standard [2, 3]. This EE1 test studies an integrated signalling mechanism to seamlessly carry the MPEG-NNR bitstream and other related data within the video bitstream.

The inference of JVET-AE0093 used the SADL library [4] that, as explicated in JVET-AD0156 [5], was implemented with half rounding enabled to enhance performance in int16 inference. Notably, this quantization approach was not adopted in NNVC 5.0. Therefore, in this EE test, data-driven quantization was applied and subsequently inference was done on top of NNVC 6.0 (note that the NN intra prediction tool and the LOP1 loop-filter were not modified from NNVC 5.0 to NNVC 6.0).

# Overfitting

The content-adaptive loop-filter in JVET-AE0093 [1] is obtained by overfitting multiplier parameters that are applied after each convolutional block.

σW*x+b∙m,
where W is the kernel, * is the convolution operator, x is the input, b is the bias, m is the multiplier and σ is the activation function.

The network architecture is based on JVET-AD0156 [5] (Figure 1) and the difference is that the multiplier layers are added after each convolutional block.

Figure 1 NN architecture from JVET-AD0156 [2].

3x3 conv 3x3x10xM

Leaky ReLu

3x3 conv 3x3xKxL

1x1 conv 1x1xMxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

1x1 conv 1x1xKxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

BS info

Qstep

Yx4+U+V 72x72x10

NxN

1x1 conv 1x1xKxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

Y’x4+U’+V‘ 64x64x6

N’xN’

n Hidden Layers

1x1 conv 1x1xKxR

3x1 Sep conv 3x1xRxR

3x1xRxR

1x1 conv 1x1xRxK

1x3 Sep conv 1x3xRxR

CP decomposition

Layers approximated with CP decomposition

Separable Convolution layers

1x1 conv 1x1xMxR

1x1 conv 1x1xRxM

Table 1 shows the details of NN information during the overfitting stage.

Table 1 Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Overfitting Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU NVIDIA A100-SXM-80GB |
|  | Framework: | TensorFlow 2.8.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 200 epochs for class C&D, 100 epochs for class A,B,F |
|  | Batch size: | 64 |
|  | Loss function: | Weighted-MSE |
|  | Training time (for 1 model): | A1 & A2: 11 hours B: 7 hours C: 2.5 hours D: 1 hour F: 4 hours |
|  | Training data information: | JVET CTC RA mandatory sequences |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 72x72 |
|  | Learning rate: | 1e-3 |
|  | Learning rate update strategy | Constant learning rate |
|  | Optimizer: | ADAM |
|  | Preprocessing: | Convert 144x144 YUV420 signal to 6 72x72 blocks. Normalize to 0~1 |
|  | Mini-batch selection process: |  |
|  | Training data update strategy: |  |
|  | Other information: |  |

The resulting weight updates from the overfitting process are coded using the MPEG-NNR standard [3, 4] and the estimated time of encoding weight updates is about 1 hour for class A, 30 minutes for class B, 7 minutes for class C & F, 2 minutes for class D. At decoder side, the average time for NNR decoding and reconstructing the overfitted model is 5 seconds.

# Quantization

The floating-point overfitted models, obtained after decoding the NNR weight updates, were quantized to int16 using data-driven quantization, which falls under the category of post-training quantization and involves calibration of sequence data that was previously utilized in overfitting. In the quantization process, the naïve approach is applied for both parameters and activations, followed by adjustments to the model quantization parameters (model QPs) which are determined based on the statistical characteristics of the output values obtained from each layer within the neural networks. The estimated time for determining the model QPs is around 8 minutes for class A, 4 minutes for class B, less than 1 minute for class C, D & F. The determined model QPs are signalled to the decoder together with the MPEG-NNR bitstream representing the weight update.

# Neural network update signalling

The MPEG-NNR bitstream and the model QPs are carried within a newly designed NN filter update APS NAL unit, which is described in this section.

## Proposed specification text

### Adaptation parameter set

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| aps_params_type | u(3) |
| aps_adaptation_parameter_set_id | u(5) |
| aps_chroma_present_flag | u(1) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| else if( aps_params_type = = NN_UPDATE_APS ) |  |
| nn_filter_update_data( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

aps_params_type specifies the type of APS parameters carried in the APS as specified in Table 2. The value of aps_params_type shall be in the range of 0 to 2, inclusive, in bitstreams conforming to this version of this Specification. Other values of aps_params_type are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this version of this Specification shall ignore APS NAL units with reserved values of aps_params_type.

Table 2. APS parameters type codes and types of APS parameters

| aps_params_type | Name of aps_params_type | Type of APS parameters |
| --- | --- | --- |
| 0 | ALF_APS | ALF parameters |
| 1 | LMCS_APS | LMCS parameters |
| 2 | SCALING_APS | Scaling list parameters |
| 3 | NN_UPDATE_APS | Neural-network filter update |

All APS NAL units with a particular value of aps_params_type, regardless of the nuh_layer_id values and whether they are prefix or suffix APS NAL units, share the same value space for aps_adaptation_parameter_set_id. APS NAL units with different values of aps_params_type use separate values spaces for aps_adaptation_parameter_set_id.

aps_adaptation_parameter_set_id provides an identifier for the APS for reference by other syntax elements.

When aps_params_type is equal to ALF_APS or SCALING_APS, the value of aps_adaptation_parameter_set_id shall be in the range of 0 to 7, inclusive.

When aps_params_type is equal to LMCS_APS, the value of aps_adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

When aps_params_type is equal to NN_UPDATE_APS, the value of aps_adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

### Neural network filter update data syntax

| nn_filter_update_data( ) { | Descriptor |
| --- | --- |
| nnfu_bitstream_size_minus1 | u(15) |
| for( i = 0; i <= nnfu_bitstream_size_minus1; i++ ) |  |
| nnfu_payload_byte[ i ] | u(8) |
| } |  |
| } |  |

nnfu_bitstream_size_minus1 + 1 specifies the count of bytes in nnfu_payload_byte.

nnfu_payload_byte contains the byte sequence representing the size of the model QPs, the model QPs and the bitstream conforming to ISO/IEC 15938-17. In particular, nnfu_payload_byte[ 0 ] specifies the size of the model QPs. Let NnfuQPsSize be equal to nnfu_payload_byte[ 0 ] and NnfuBitstreamSizeMinus1 be equal to nnfu_bitstream_size_minus1: the bytes from nnfu_payload_byte[ 1 ] to nnfu_payload_byte[ NnfuQPsSize ] contain the model QPs and the bytes from nnfu_payload_byte[ NnfuQPsSize + 1 ] to nnfu_payload_byte[ NnfuBitstreamSizeMinus1 ] shall be a complete bitstream that conforms to ISO/IEC 15938-17 and provides an update relative to the base neural-network filter associated with the same aps_adaptation_parameter_set_id.

# Experiment Results

Table 3 describes the loop-filter information at inference stage.

Table 3 Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL |
|  | Number of GPUs per Task |  |
|  | Number of Parameters (Each Model) | 53724 |
|  | Total Number of Parameters (All Models) | 107448 |
|  | Parameter Precision (Bits) | Int16 |
|  | Memory Parameter (MB) | 0.21 |
|  | Multiply Accumulate (kMAC/pixel) | 16.5 |
|  | Calculation Method | on a block basis |
| Optional |  |  |
|  | Total Conv. Layers | 46 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size | 72x72 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

The anchor is NNVC 5.0, where both the default LOP1 loop-filter and the NN intra prediction tool are enabled. Table 4 shows the results of the EE test, where

- the MPEG-NNR bitstream is carried in the newly designed and implemented NN filter update APS NAL unit;

- data-driven quantization is applied;

- half rounding is disabled in SADL library.

In comparison to Table 5, which shows the results reported in JVET-AE0093 [1], it is observed that the coding gain reduction is relatively marginal.

Table 4. BD-rate of utilizing the APS-based carriage of MPEG-NNR bitstream, data-driven quantization and disabling half rounding in SADL library. Anchor: NNVC 5.0

|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 | -1.62% | -3.27% | -7.57% | -2.24% | -4.29% | -8.95% | 99% | 105% |
| Class A2 | -0.09% | -7.50% | -7.37% | 0.52% | -8.08% | -7.84% | 99% | 105% |
| Class B | -1.68% | -8.50% | -5.00% | -0.21% | -10.05% | -4.82% | 99% | 105% |
| Class C | -1.10% | -5.29% | -3.12% | -0.28% | -5.13% | -3.23% | 99% | 104% |
| Overall | -1.19% | -6.40% | -5.49% | -0.49% | -7.19% | -5.83% | 99% | 105% |
| Class D | -0.85% | -7.80% | -6.97% | 0.42% | -7.13% | -7.20% | 99% | 104% |
| Class F | -1.06% | -7.58% | -5.00% | -0.71% | -7.04% | -5.59% | 100% | 104% |

Table 5. BD-rate of enabling half rounding in SADL library. Anchor: NNVC 5.0. Results from JVET-AD0093

For further comparison, Table 6 shows the BD-rates on class D, with APS-based carriage of MPEG-NNR bitstream and where both data-driven quantization and half rounding are disabled.

Table 6. BD-rate of disabling data-driven quantization and half rounding in SADL library. Anchor: NNVC 5.0

|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class D | -0.59% | -6.66% | -6.19% | 0.90% | -5.27% | -5.37% | 100% | 102% |

Furthermore, a simulation of signalling the neural network update for each intra frame was performed. To this end, the bitrate of the neural network update obtained for the whole sequence was multiplied by the number of intra frames in the sequence. Then, the BD-rates were recomputed by considering such multiplied bitrate overheads. The recomputed BD-rates are shown in Table 7.

Table 7. BD-rate of signaling the neural network update for each intra frame (simulation). Anchor: NNVC 5.0

|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 | -1.22% | -2.93% | -7.22% | -1.77% | -3.85% | -8.54% | 99% | 105% |
| Class A2 | 0.19% | -7.26% | -7.14% | 0.86% | -7.82% | -7.57% | 99% | 105% |
| Class B | -0.87% | -7.79% | -4.27% | 0.91% | -9.20% | -3.89% | 99% | 105% |
| Class C | 0.56% | -3.31% | -1.72% | 2.05% | -2.77% | -1.55% | 98% | 104% |
| Overall | -0.34% | -5.52% | -4.75% | 0.67% | -6.14% | -4.93% | 99% | 104% |
| Class D | 3.20% | -3.97% | -3.00% | 6.51% | -2.24% | -1.86% | 98% | 104% |
| Class F | 2.70% | -4.08% | -1.51% | 4.21% | -2.49% | -1.03% | 100% | 104% |

# Recommendation

Adopt the content-adaptive LOP filter described in this document into the next version of NNVC software, provided that the cross-check is successful.

# References

- R. Yang, M. Santamaria, F. Cricri, H. Zhang, J. Lainema, R. G. Youvalari, M. M. Hannuksela. AHG11: Content-adaptive neural network loop-filter. JVET-AE0093. July 2023.

- ISO/IEC FDIS 15938-17. Information technology – Multimedia content description interface – Part 17: Compression of neural networks for multimedia content description and analysis.

- Test Model of Incremental Compression of Neural Networks for Multimedia Content Description and Analysis (INCTM), N0179. February 2022.

- F. Galpin, T. Dumas, P. Bordes, P. Nikitin, F. Le Leannec, E. Francois. AHG11: Small ad-hoc deep-learning library. JVET-W0181. July 2021.

- J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya, T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy. EE1-1.1: Complexity Reduction on Neural-Network Loop Filter. JVET-AD0156. April 2023.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).