| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0262 |
| --- | --- |

| Title: | Crosscheck of JVET-AH0075 (EE2-2.1: TIMD merge mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Yang Wang Fangheng Building, No.27, 3rd Ring West Rd, Haidian District, 100098, Beijing, China | Tel: Email: | wangyang.cs@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of JVET-AH0075 [1], which proposes a TIMD merge mode inheriting the TIMD modes from the previously TIMD coded blocks. Test 2.1a includes TIMD merge mode with one candidate, while Test 2.1b proposes TIMD merge mode with two candidates. The simulation results match with those provided in JVET-AH0075.

Experimental results

JVET-AH0075 was implemented on top of ECM-12.0. The simulations were conducted under the common test conditions of EE2 [2]. The crosscheck results for Test 2.1a and Test 2.1b are shown in Table 1 and Table 2 respectively.

Table 1. Results of Test #1

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.07% | -0.06% | 0.11% | 103.8% | 101.4% | 100.3% | 100.4% |
| Class A2 | -0.10% | -0.01% | -0.04% | 101.5% | 102.1% | 100.0% | 100.5% |
| Class B | -0.09% | 0.02% | -0.01% | 103.5% | 101.5% | 102.2% | 100.4% |
| Class C | -0.06% | -0.02% | 0.09% | 102.7% | 101.9% | 102.3% | 100.2% |
| Class E | -0.04% | 0.06% | 0.12% | 103.5% | 100.8% | 100.3% | 100.1% |
| Overall | -0.07% | 0.00% | 0.05% | 103.0% | 101.5% | 101.2% | 100.3% |
| Class D | -0.08% | 0.03% | 0.14% | 103.6% | 101.0% | 100.4% | 100.1% |
| Class F | -0.04% | 0.16% | -0.01% | 109.2% | 100.4% | 99.9% | 100.3% |
| Class TGM | 0.00% | 0.02% | -0.14% | 107.4% | 102.0% | 102.8% | 100.5% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.05% | 0.10% | -0.04% | 100.7% | 99.6% | 100.2% | 100.3% |
| Class C | -0.02% | 0.01% | -0.03% | 100.2% | 100.3% | 100.3% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.01% | -0.44% | 0.36% | 100.9% | 100.2% | 100.8% | 100.1% |
| Class F | 0.03% | 0.08% | -0.06% | 103.2% | 100.9% | 100.1% | 100.6% |
| Class TGM | 0.11% | 0.13% | 0.08% | 102.1% | 99.3% | 100.1% | 100.1% |

Table 2. Results of Test #2

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.10% | -0.06% | 0.15% | 102.8% | 102.1% | 100.7% | 100.5% |
| Class A2 | -0.13% | 0.06% | 0.00% | 103.1% | 103.1% | 100.2% | 100.5% |
| Class B | -0.12% | 0.05% | -0.02% | 104.0% | 102.8% | 104.5% | 100.3% |
| Class C | -0.09% | 0.02% | 0.03% | 103.4% | 104.4% | 102.3% | 100.2% |
| Class E | -0.07% | 0.00% | -0.06% | 103.9% | 102.5% | 100.3% | 100.2% |
| Overall | -0.10% | 0.02% | 0.02% | 103.5% | 103.0% | 101.9% | 100.3% |
| Class D | -0.11% | 0.06% | 0.03% | 103.7% | 103.3% | 100.4% | 100.1% |
| Class F | -0.03% | 0.09% | 0.07% | 101.5% | 101.8% | 99.4% | 100.2% |
| Class TGM | 0.00% | 0.06% | -0.10% | 102.2% | 102.3% | 100.3% | 100.4% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.06% | -0.13% | -0.18% | 100.2% | 98.8% | 100.2% | 100.3% |
| Class C | -0.03% | 0.01% | 0.11% | 100.5% | 99.5% | 100.3% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.00% | -0.01% | 0.27% | 101.0% | 99.3% | 100.4% | 100.1% |
| Class F | 0.01% | 0.00% | -0.01% | 100.8% | 100.6% | 100.3% | 100.4% |
| Class TGM | -0.03% | 0.00% | 0.03% | 101.0% | 99.0% | 100.3% | 100.3% |

References

M. Abdoli, R. G. Youvalari, and A. Tissier, “EE2-2.1: TIMD merge mode,” Document of Joint Video Experts Team, JVET-AH0075, Rennes, FR, April 2024.

M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October 2023.