| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ2020-v1 |
| --- | --- |

| Title: | Film grain synthesis technology for video applications, Ed. 2 (Draft 1) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | W. Husak M. Radosavljević D. Grois Y. He P. de Lagrange A. Norkin A. Tourapis | Email: | wjh@dolby.com milos.radosavljevic@interdigital.com dan.grois@anyai.ai yonghe@qti.qualcomm.com philippe.delagrange@interdigital.com anorkin@netflix.com atourapis@apple.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document contains the draft text for a future revision of the technical report on “Film grain synthesis technology for video applications”.

# Clarification of semantics of frequency filtering model

In subclause 7.2.2.2 item b), replace the following:

“Set the value of all elements of the random-value DCT2 array to 0 when the corresponding column and row indices are not within the corresponding high and low horizontal and vertical cut-off frequencies. Also set the DC element to zero.”

with the following:

"Set the values of all elements of the array n to 0 for which any of the following apply:

–	the column index is above the high horizontal cut-off frequency

–	the row index is above the high vertical cut-off frequency

–	the column index is below the low horizontal cut-off frequency and the row index is below the low vertical cut-off frequency.

Also set the DC element of n to zero, i.e., set n[0][0] equal to 0.”