| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 38th Meeting, On-line, 26 March –04 April 2025 | Document: JVET-AL0011-v3 |
| --- | --- |

| Title: | JVET AHG report: Neural network-based video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Shan Liu Andrew Segall Franck Galpin Yun Li Junru Li Ru-Ling Liao Maria Santamaria Tong Shao Mathias Wien Ping Wu | Email: | elena.alshina@huawei.com shanl@global.tencent.com asegall@amazon.com franck.galpin@InterDigital.com yli30@qti.qualcomm.com lijunru@bytedance.com ruling.lrl@alibaba-inc.com maria.santamaria_gomez@nokia.com tshao@dolby.com wien@lfb.rwth-aachen.de pwuzte@gmail.com |
| Source: | AHG11 coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities of AHG11: Neural network-based video coding between the 37th meeting held in Geneva and 38th on-line meeting.

The AHG was established with the following mandates:

- Evaluate and quantify the performance improvement potential of NN-based video coding technologies compared to existing video coding standards such as VVC, including both individual coding tools, architectures and content adaptation with NN parameters overfitting.

- Establish logistics of training data sets, including list, location and md5sums. Generate and distribute anchor encoding, and develop supporting software as needed.

- Study potential improvements of the NNVC CTC document JVET-AJ2016.

- Study the impact of training (including the impact of loss functions) on the performance of candidate technologies and identify suitable material for testing and training.

- Analyse complexity characteristics for technologies under study, including transformers, perform complexity analysis, and develop complexity reductions of candidate technology.

- Discuss and propose improved metrics to perform complexity analysis of NN architectures, in particular also considering the impact of sparse models.

- Finalize and discuss the EE on neural network-based video coding.

- Promote the call for training materials, distribute it, and actively communicate with content owners.

- Coordinate with other groups, including SC 29/AG 5 on the evaluation and assessment of visual quality, and AHG12 on the interaction with ECM coding tools. If possible, prepare encodings with combinations of tools included in the NNVC software for visual quality assessment at the next meeting.

- Coordinate with AHG14 on items related to NNVC software development and study the impact of training set extension.

# Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, for emails exchange. Five emails were exchanged on the reflector related to the AHG mandates.

## Common Test Conditions

### Anchor Encoding

Anchor for the NN-based video coding activity made available though the Git repository used for the AHG activity:

https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/blob/master/Anchor%20performance/NNVC-10-VTM_vs_NNVC-12.xlsm

also distributed by AhG14 in JVET-AK0014.

## New training materials

BVI-AOM training set was used in EE1-2 (NN-Inter) and EE1-3 (retraining NN-filters) categories. Results of training are analyzed in relevant EE1 contributions. So far, some promising performance improvement is observed for HOP NN-filter. It is suggested to conduct detailed discussion, opinion exchange and brainstorming on training using additional materials.

## Dataset access

Training data generation and md5 sums for BVI-AOM been discussed during AhG11/AhG14 teleconference in scope of EE1-3 tests.

## Interaction with ECM

Some version of neural network based Intra included into ECM already. Interface for NNVC in-loop filter testing in ECM has been added and some test results reported in incoming AhG11 contribution.

In random access configuration NN-based in-loop filter provides roughly 1%, 2% and 5% BD-rate gain on top of ECM with only 1%, 2% and 8% CPU encoding run-time increment for VLOP, LOP and HOP versions of filter correspondently.

| JVET-AL0230 | AhG11/AhG12: Performance of the NNVC ILF in ECM | D. Rusanovskyy, K. Panusopone, S. Hong, L. Wang (Nokia) |
| --- | --- | --- |
| JVET-AL0228 | EE2-related: NNLF interface in ECM | T. Poirier, F. Galpin, G. Boisson (InterDigital) |

## EE Coordination

The AHG finalized, conducted, and discussed the EE on NN based video coding. A summary report for the EE is available at this meeting as:

| JVET-AL0023 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, J. Ström, Z. Xie (EE coordinators) |
| --- | --- | --- |

## Teleconferences

The AHG conducted two joint teleconferences with AHG14 and EE1 during the interim period. The teleconferences were held on February 05 and March 05, 2025. In those teleconferences, the following topics were discussed:

- New version of NN-based filter (LOP5) training report and LOP5 inference cross-check

- NNVC12.0 software integration status and anchors performance,

- EE1 description finalization

- Training on BVI-AOM

- Combination of two proposals on LOP  JVET-AK0195 (luma input to chroma processing at the later stage of NN filter) on top of JVET-AK0150 (attention mechanism) was verified by training cross-check and adoption from last meeting confirmed during teleconference.

| JVET-AL0043 | [AHG11] [AHG14] Teleconference on NNVC | E. Alshina, F. Galpin |
| --- | --- | --- |

## Performance Evaluation

The performance and complexity of NN-based tools available in NNVC SW is summarized in the table below. All test data provided by AhG14. Encoding and decoding run time is very dependent on cluster used for simulation. Run time data in this table are all from InterDigital.

In NNVC-12, LOP filter performance improved by 0.6% (in random access test). There is no performance change for other configuration, but there is some speed-up (due to SADL optimization).

| Test vs NNVC (configured as VTM) |  |  | Random Access cfg |  |  |  |  |  |  |  |  |  |  |  |  |  | kMAC/pxl |  |  |  |  |  |  |  |  | Param (Mprm) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc | Dec | Total | Filter | Intra | SR | Total | Filter | Intra | SR |
| NN-Intra & LOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-12.0 (LOP5) | -8.2% | -15.3% | -13.5% | 1.2 | 35 | 21.4 | 16.6 | 4.8 | 0 | 1.5 | 0.25 | 1.3 | 0 |
| NNVC-11.0 (LOP4) | -7.6% | -14.3% | -13.2% | 1.2 | 36 | 21.6 | 16.8 | 4.8 | 0 | 1.5 | 0.21 | 1.3 | 0 |
| NNVC-10.0 (LOP3) | -7.4% | -13.6% | -11.6% | 1.2 | 33 | 21.7 | 16.9 | 4.8 | 0 | 1.5 | 0.21 | 1.3 | 0 |
| NNVC-9.1(LOP3) | -7.3% | -13.1% | -11.3% | 1.2 | 81 | 24.8 | 16.9 | 7.9 | 0 | 1.7 | 0.21 | 1.5 | 0 |
| NNVC-8.0(LOP2) |  | -6.9% |  |  |  | -13.2% |  |  | -12.1% |  |  | 1.2 |  | 73 |  | 25.0 |  |  | 17.1 |  | 7.9 |  | 0 |  | 1.6 |  |  | 0.05 |  | 1.5 |  | 0 |  |
| NNVC-7.1(LOP2) |  | -6.9% |  |  |  | -13.2% |  |  | -12.1% |  |  | 1.3 |  | 86 |  | 25.0 |  |  | 17.1 |  | 7.9 |  | 0 |  | 1.6 |  |  | 0.05 |  | 1.5 |  | 0 |  |
| NNVC-9.1(LOP2CA) |  | -8.2% |  |  |  | -16.5% |  | -15.5% |  |  | 2.5 |  |  | 69 |  | 25.5 |  |  | 17.6 |  | 7.9 |  | 0 |  | 1.6 |  |  | 0.05 |  | 1.5 |  | 0 |  |
| NN-Intra & HOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-10.0 (HOP5) | -14.2% | -19.5% | -19.9% | 2.6 | 1135 | 471 | 466 | 4.8 | 0 | 2.7 | 1.4 | 1.3 | 0 |
| NNVC-9.1(HOP4) | -14.1% | -19.2% | -19.6% | 2.9 | 1447 | 484 | 476 | 7.9 | 0 | 3.0 | 1.44 | 1.5 | 0 |
| NNVC-8.0(HOP3) |  | -13.7% |  |  |  | -13.9% |  |  | -14.5% |  |  | 2.5 |  | 1092 |  | 474 |  |  | 466 |  | 7.9 |  | 0 |  | 2.9 |  |  | 1.40 |  | 1.5 |  | 0 |  |
| NNVC-7.0(HOP2) |  | -13.6% |  |  |  | -12.5% |  | -14.2% |  |  | 4.1 |  |  | 2071 |  | 485 |  |  | 477 |  | 7.9 |  | 0 |  | 3.0 |  |  | 1.50 |  | 1.5 |  | 0 |  |
| NN-Intra & VLOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-11.0 (VLOP3) |  | -5.8% |  |  | -6.6% |  |  | -5.7% |  |  | 1.2 |  |  | 7 |  | 9.9 |  |  | 5.10 |  | 4.8 |  | 0 |  | 1.3 |  |  | 0.07 |  | 1.3 |  | 0 |  |
| NNVC-10.0 (VLOP2) |  | -5.6% |  |  | -7.6% |  |  | -6.4% |  |  | 1.1 |  |  | 15 |  | 10 |  |  | 5.16 |  | 4.8 |  | 0 |  | 1.3 |  |  | 0.06 |  | 1.3 |  | 0 |  |
| NNVC-9.1(VLOP) |  | -5.3% |  |  | -5.4% |  |  | -5.2% |  |  | 1.2 |  |  | 40 |  | 13 |  |  | 5.12 |  | 7.9 |  | 0 |  | 1.5 |  |  | 0.02 |  | 1.5 |  | 0 |  |
| NN-Intra & LOP filter content adaptive (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-10.0 caLOP4 |  | -8.5% |  |  |  | -18.0% |  |  | -17.1% |  |  | 2.3 |  | 36 |  | 21.8 |  |  | 16.9 |  | 4.8 |  | 0 |  | 1.5 |  |  | 0.21 |  | 1.3 |  | 0 |  |
| NN-Intra & LOP filter & adaptive resolution coding (3 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-11.0 NNSR |  | -8.5% |  |  |  | -12.2% |  |  | -10.9% |  |  |  |  |  |  | 26.3 |  |  | 16.8 |  | 4.8 |  | 4.7 |  | 1.4 |  |  | 0.05 |  | 1.3 |  | 0.05 |  |
| NNVC-8.0 RPR |  | -7.5% |  |  |  | -10.9% |  |  | -9.7% |  |  |  |  |  |  | 25.0 |  |  | 17.1 |  | 7.9 |  | 0 |  | 1.6 |  |  | 0.05 |  | 1.5 |  | 0 |  |
| NNVC-8.0 NNSR |  | -7.8% |  |  |  | -11.8% |  |  | -10.5% |  |  |  |  |  |  | 45.3 |  |  | 17.1 |  | 7.9 |  | 20.3 |  | 1.8 |  |  | 0.21 |  | 1.5 |  | 0.1 |  |

More details and analysis for tools and tools combination is expected in AhG14 report.

## Architectural changes

Following proposals from 36th and 37th meetings several companies propose framework for E2E AI coded reference picture insertion. Promising gain reported for this approach (4% and 1% in all intra and random-access configuration correspondingly).

In order to support this feature bit-exact reconstruction of E2E AI coded picture is required. One contribution discusses fundamental aspects of bit-exact reproducibility and potential implementation using different platforms (GPU, NPU, CPU, ASIC).

## NNVC algorithms description

One contribution submitted to this meeting resolves mismatch between LOP5 implementation in NNVC SW and description.

# Input contributions

There are 34 input contributions related to the AHG mandates. The list of input contributions is provided below.

| Reporting (6) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| JVET-AL0023 | EE1: Summary report of exploration experiment on neural network-based video coding |  |  |  | E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, J. Ström, Z. Xie (EE coordinators) |  |  |
| JVET-AL0043 | [AHG11] [AHG14] Teleconference on NNVC |  |  |  | E. Alshina, F. Galpin |  |
| JVET-AL0165 | AHG11: Training NNSR using Reparameterization and Progressive Activation |  |  |  | H. Cho, S. Bahk, H. Y. Kim (KHU), D. Kim, S.-C. Lim (ETRI) |  |
| JVET-AL0230 | AhG11/AhG12: Performance of the NNVC ILF in ECM |  |  |  | D. Rusanovskyy, K. Panusopone, S. Hong, L. Wang (Nokia) |  |
| JVET-AL0228 | EE2-related: NNLF interface in ECM |  |  |  | T. Poirier, F. Galpin, G. Boisson (InterDigital) |  |
| JVET-AL0291 | EE1-related: Recommendations for resolving mismatches between LOP5 description and implementation |  |  |  | Nam Le, Francesco Cricri (Nokia) |  |
| Architectural change and implementation aspects (4) |  |  |  |  |  |  |
| JVET-AL0080 | AHG11: Bit-exact reconstruction for NN video tools |  |  |  | L. Kerofsky, Y. Li, M. Karczewicz (Qualcomm) |  |
| JVET-AL0196 | AHG 11: Neural Network Coded Reference Frame for Intra Coding with Residual Coding and Intra Blocks |  |  |  | F. Brand, T. Solovyev, E. Alshina (Huawei) |  |
| JVET-AL0203 | [AHG11] A Hybrid Framework Integrating End-to-End Learned Image Codec with Conventional Codec |  |  |  | N. Zou, A. Hallapuro, F. Cricri, H. Zhang, A. B. Koyuncu, J. Ahonen, M. M. Hannuksela (Nokia) |  |
| JVET-AL0243 | [AHG11] Multilayer framework for supporting a hybrid codec using End-to-End Learned Image Codec and Conventional Video Codec |  |  |  | F. Urban, Y. Chen, F. Galpin, E. François (InterDigital) |  |
| EE1 contributions (11) |  |  |  |  |  |  |
| JVET-AL0084 | EE1-1.2: LOP5 improvement with parallel 1x3/3x1 Backbone | T. Shao, P. Yin, S. McCarthy (Dolby), J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya (Ittiam) |  |  |  |
| JVET-AL0085 | EE1-3.1: NNVC-LOP5 retraining with additional BVI-AOM dataset | A. Suneja, J. N. Shingala, A. Shyam, S. P. Badya (Ittiam), T. Shao, P. Yin, S. McCarthy (Dolby) |  |  |  |
| JVET-AL0104 | EE1-2.1: Lightweight Multiscale Reference Frame Generation for VVC Inter Coding | P. Li, C. Jung, Q. Qin (Xidian Univ.) |  |  |  |
| JVET-AL0105 | EE1-2.2: RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding | Q. Qin, C. Jung (Xidian Univ.) |  |  |  |
| JVET-AL0144 | EE1-3.1: Retraining LOP4 and LOP5 using extended dataset from BVI-AOM | D. Liu, J. Ström, M. Damghanian, P. Wennersten (Ericsson) |  |  |  |
| JVET-AL0145 | EE1-1.4: Conditional loop-filter | M. Santamaria, F. Cricri (Nokia) |  |  |  |
| JVET-AL0164 | EE1-1.1: Multiscale blocks in LOP5 and VLOP3 filters | R. Yang, M. Santamaria, F. Cricri, H. Zhang, J.Lainema, M.M. Hannuksela (Nokia) |  |  |  |
| JVET-AL0169 | EE1-1.3 Dimension-wise decomposed representation of multiplier for content-adaptive loop filtering | Z. Xu, J. Konieczny, A. Filippov, C. Hollmann, V. Rufitskiy, T. Dong (TCL) |  |  |  |
| JVET-AL0187 | EE1-3.1: NNVC-LOP4 and LOP5 retraining with additional BVI-AOM dataset | T. Dumas, A. Monier, F. Galpin (InterDigital) |  |  |  |
| JVET-AL0189 | EE1-3.2 – NNVC-HOP5 retraining adding BVI-AOM | F. Galpin (InterDigital) |  |  |  |
| JVET-AL0190 | EE1-3.3 – NNVC-VLOP3 retraining adding BVI-AOM | F. Galpin, Z. Ameur (InterDigital), R. Chang, L. Wang, X. Xu, S. Liu (Tencent) |  |  |  |
| New NNVC tools in AhG11 or EE1 related contributions (7) |  |  |  |  |  |  |
| JVET-AL0166 | EE1 related: Improved VLOP with Attention |  | Y. Li, L. Kerofsky, M. Karczewicz (Qualcomm |  |  |  |
| JVET-AL0167 | EE1 related: Further simplification of VLOP with Attention |  | Y. Li, L. Kerofsky, M. Karczewicz (Qualcomm) |  |  |  |
| JVET-AL0184 | EE1-related: Deep Reference Frame Generation for Inter Prediction Enhancement |  | D. Ding, X. Chen, Z. Chen (Wuhan Univ.) |  |  |  |
| JVET-AL0121 | AHG11: Sample-based adaptive blending weight selection for LOP |  | H. Kwon, H. Ko (HYU) |  |  |  |
| JVET-AL0136 | AHG11: Over-Parameterized LOP In-Loop Filter |  | J. Han, C. Jung, Q. Qin (Xidian Univ.) |  |  |  |
| JVET-AL0137 | AHG11: Backbone Block Enhancement of LOP In-Loop Filter with Over-Parameterized Training and Variable Channels |  | J. Han, C. Jung, Q. Qin (Xidian Univ.) |  |  |  |
| JVET-AL0250 | AHG11: Cross-component enhanced NNSR |  | T. Yang, W.-X. He, Y.-Q. Zhu, J.-D. Ye, X.-T. Xie, J.-S. Gong, Q. Liu (HUST), Z.-Y. Lv (vivo) |  |  |  |
| Cross-checks (6 some not yet uploaded at the time report was prepared) |  |  |  |  |  |  |  |
| JVET-AL0173 | Crosscheck of JVET-AL0164 (EE1-1.1: Multiscale blocks in LOP5 and VLOP3 filters) |  |  | Y. Li (Qualcomm) |  |  |
| JVET-AL0185 | Crosscheck of JVET-AL0169 (EE1-1.3 Dimension-wise decomposed representation of multiplier for content-adaptive loop filtering) |  |  | M. Santamaria (Nokia) |  |  |
| JVET-AL0246 | Crosscheck of JVET-AL0084 (EE1-1.2: LOP5 improvement with parallel 1x3/3x1 Backbone) |  |  | Y. Li (Bytedance) |  |  |
| JVET-AL0284 | Crosscheck of JVET-AL0145 (EE1-1.4: Conditional loop-filter) |  |  | J. Strom (Ericsson) |  |  |
| JVET-AL0297 | Crosscheck of JVET-AL0104 (EE1-2.1: Lightweight Multiscale Reference Frame Generation for VVC Inter Coding) |  |  | L. Murn (Nokia) |  |  |
| JVET-AL0298 | Crosscheck of JVET-AL0105 (EE1-2.2: RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding) |  |  | L. Murn (Nokia) |  |  |

# Recommendations

The AHG recommends:

- Review all input contributions.

- Continue investigating neural network-based video coding tools, including coding performance and complexity.

- Continue collecting training materials for neural network-based video coding tool development and investigate training stability.

- Conduct detailed discussion on training, exchange opinions and observations among interested parties (potential BoG)

- Add study of bit-exact reproducibility to the NNVC complexity assessment