| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 38th Meeting, by teleconference, 26 March – 4 April 2025 | Document: JVET-AL0187-v3 |
| --- | --- |

| Title: | EE1-3.1: NNVC-LOP4 and LOP5 retraining with additional BVI-AOM dataset |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Thierry Dumas Antoine Monier Franck Galpin | Email: | franck.galpin@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

This contribution reports the results of EE1-3.1.1, EE1-3.1.2, EE1-3.1.3 and EE1-3.1.4 for LOP4 and LOP5 architectures. EE1-3.1.1 is the reproduction training of LOP4 adopted model with DIV2K, BVI-DVC and TVD datasets. EE1-3.1.2 is the training of LOP4 model with additional BVI-AOM dataset along with existing DIV2K, BVI-DVC and TVD datasets. Only stage 3 training was conducted for both the EE tests.  EE1-3.1.3 is the reproduction training of LOP5 adopted model with DIV2K, BVI-DVC and TVD datasets. EE1-3.1.4 is the training of LOP5 model with additional BVI-AOM dataset along with existing DIV2K, BVI-DVC and TVD datasets. Only stage 3 training was conducted for both the EE tests.

The performance of the EE1-3.1.1 float model (including NNIntra) compared to the NNVC-12.0 anchor (NNIntra + LOP4 fp32) is reported to be
AI {} RA{0.11%	-0.94%	-0.45%}

The performance of the EE1-3.1.2 float model (including NNIntra) compared to the NNVC-12.0 anchor (NNIntra + LOP4 fp32) is reported to be
AI {} RA{}

The performance of the EE1-3.1.3 float model (including NNIntra) compared to the NNVC-12.0 anchor (NNIntra + LOP5 fp32) is reported to be
AI {0.17% -0.03% 0.06%} RA{0.21% -0.12% -0.07%}

The performance of the EE1-3.1.4 float model (including NNIntra) compared to the NNVC-12.0 anchor (NNIntra + LOP5 fp32) is reported to be
AI {-0.03%	0.19%	0.38%} RA{-0.11% -0.01% 0.23%}

# Introduction

It was proposed in JVET-AK0255[1] to extend the NNVC training data set and use BVI-AOM dataset along with the existing NNVC training data set.

The BVI-AOM dataset contains 78 unique 4K (3840x2176) source sequences. These sequences have been down-sampled to 1920x1088, 960x544, and 480x272, similar to BVI-DVC. Each sequence is 64 frames long without any scene cuts. All down-sampled sequences were used for training data generation, while only 8 4K source sequences were used for 4K training data. The sequences were encoded using the same QPs as BVI-DVC for training data generation. The training data was encoded using code in the branch NNVC-6.1_aom_dataset of jvet-ahg-nnvc git repository. The md5sum values were cross verified between proponents after encoding the selected sequences and generating the final training data following decoding. Only stage 3 training was conducted on the LOP5 adopted architecture with the same training parameters as that of adopted LOP5 model.

EE1-3.1.1 and EE1-3.1.3 [2] tests correspond to the trainings of LOP4 and LOP5 models respectively without BVI-AOM data set. EE1-3.1.2 and EE1-3.1.4 [2] tests correspond to the trainings of LOP4 and LOP5 models respectively with BVI-AOM data set.

# Training and inference information

Information related to the training and inference of models can be found in Table 1 and Table 2 respectively. The training time has increased by ~20% for test EE1-3.1.4 compared to EE1-3.1.3.

Table 1: Network information for the model during training.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type |  |
|  | Framework: | Pytorch v2.0.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 60 |
|  | Batch size: | 64 |
|  | Loss function: | L1, L2 |
|  | Training time (for 1 model): | EE1-3.1.1: EE1-3.1.2: EE1-3.1.3: EE1-3.1.4: |
|  | Training data information: | EE1-3.1.1 EE1-3.1.3: DIV2K, TVD, BVI-DVC EE1-3.1.2 EE1-3.1.4: DIV2K, TVD, BVI-DVC, BVI-AOM |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 2e-4 |
|  | Learning rate update strategy | Stage3 milestones: [41,51,55] |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Mini-batch selection process: |  |
|  | Training data update strategy: |  |
|  | Other information: |  |

Table 2: Network information for the model during inference.

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
|  | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL float |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 247422 |
|  | Total Parameter Number | 247422 |
|  | Parameter Precision (Bits) | Floating (32 bits) |
|  | Memory Parameter (MB) | 0.98MB (float) |
|  | MAC/Pixel (Kilo) | 16.6 |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Peak Memory Usage (per Model) |  |
|  | Other information: |  |

# Results

Simulations are performed based on the common test conditions for neural-network-based video coding technology for AI and RA configurations using NNVC-12 [3].

## Results for EE1-3.1.1

Simulation results for EE1-3.1.1 which is the reproduction of LOP4 training using existing LOP training dataset (DIV2K, BVI-DVC and TVD) with NNVC-12.0 NNIntra + LOP4 float as anchor is reported in Table 3 (from previous crosscheck of LOP4)

Table 3: EE1-3.1.1 float vs NNVC-12.0 anchor with LOP4 float.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | 0.10% | -0.94% | -0.97% |
| Class A2 | #VALUE! | -100.00% | -100.00% | 0.11% | -0.86% | -0.18% |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0.12% | -0.89% | -0.35% |
| Class C | 0.06% | -1.03% | -0.56% | 0.11% | -1.06% | -0.40% |
| Class E | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0.11% | -0.94% | -0.45% |
| Class D | 0.06% | -1.10% | -0.76% | 0.09% | -0.95% | -1.07% |

## Results for EE1-3.1.2

Simulation results for EE1-3.1.2 using BVI-AOM as additional training dataset with NNVC-12.0 NNIntra + LOP4 float as anchor is reported in Table 4.

Table 4: EE1-3.1.2 float vs NNVC-12.0 anchor with LOP4 float.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C | 0.22% | 2.36% | 4.25% | 0.11% | 2.05% | 3.81% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.15% | 2.42% | 3.93% | -0.01% | 2.79% | 3.25% |

## Results for EE1-3.1.3

Simulation results for EE1-3.1.3 which is the reproduction of LOP5 training using existing LOP training dataset (DIV2K, BVI-DVC and TVD) with NNVC-12.0 NNIntra + LOP5 float as anchor is reported in Table 5.

Table 5: EE1-3.1.3 float vs NNVC-12.0 anchor with LOP5 float.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0.13% | -0.23% | 0.07% | 0.19% | -0.05% | -0.04% |
| Class A2 | 0.12% | 0.13% | -0.07% | 0.25% | 0.19% | 0.11% |
| Class B | 0.16% | 0.10% | 0.17% | 0.21% | -0.41% | -0.05% |
| Class C | 0.15% | 0.04% | -0.08% | 0.20% | -0.03% | -0.24% |
| Class E | 0.27% | -0.30% | 0.19% |  |  |  |
| Overall | 0.17% | -0.03% | 0.06% | 0.21% | -0.12% | -0.07% |
| Class D | 0.14% | -0.50% | -0.20% | 0.25% | -0.64% | -0.65% |

## Results for EE1-3.1.4

Simulation results for EE1-3.1.4 using BVI-AOM as additional training dataset with NNVC-12.0 NNIntra + LOP5 float as anchor is reported from Table 6 to Table 11.

### Training hyperparameters unchanged

Table 6: EE1-3.1.4 float vs NNVC-12.0 anchor with LOP5 float. For training LOP5 in EE1-3.1.4, all the LOP5 training hyperparameters are left unchanged.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.02% | -0.23% | 0.72% | -0.22% | -0.31% | 0.18% |
| Class A2 | -0.03% | 0.30% | 0.15% | -0.11% | 0.23% | 0.11% |
| Class B | -0.03% | 0.25% | 0.39% | -0.10% | 0.22% | 0.30% |
| Class C | -0.02% | 0.20% | 0.33% | -0.02% | -0.25% | 0.29% |
| Class E | -0.08% | 0.37% | 0.33% |  |  |  |
| Overall | -0.03% | 0.19% | 0.38% | -0.11% | -0.01% | 0.23% |
| Class D | -0.01% | -0.18% | 0.25% | -0.04% | -0.85% | -0.59% |

### Learning rate at 1.5e-4

Table 7: EE1-3.1.4 float vs NNVC-12.0 anchor with LOP5 float. For training LOP5 in EE1-3.1.4, the learning rate is set to 1.5e-4. The other LOP5 training hyperparameters are not modified.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0.10% | -0.15% | 0.81% | -0.10% | -0.21% | 0.25% |
| Class A2 | 0.09% | 0.28% | 0.17% | 0.04% | 0.57% | 0.65% |
| Class B | 0.08% | 0.72% | 0.52% | 0.08% | 0.64% | 0.76% |
| Class C | 0.13% | 0.37% | 0.59% | 0.21% | 0.40% | 0.90% |
| Class E | 0.08% | 0.22% | -0.08% |  |  |  |
| Overall | 0.10% | 0.34% | 0.43% | 0.07% | 0.39% | 0.67% |
| Class D | 0.13% | 0.15% | 0.61% | 0.27% | 0.16% | 0.41% |

### Learning rate at 2.5e-4

Table 8: EE1-3.1.4 float vs NNVC-12.0 anchor with LOP5 float. For training LOP5 in EE1-3.1.4, the learning rate is set to 2.5e-4. The other LOP5 training hyperparameters are not modified.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.02% | 0.18% | 0.49% | -0.35% | -0.45% | -0.01% |
| Class A2 | 0.08% | 0.35% | 0.42% | -0.09% | 0.56% | 0.75% |
| Class B | 0.04% | 0.70% | 0.97% | -0.04% | 0.29% | 0.50% |
| Class C | 0.12% | 0.77% | 0.98% | 0.08% | 0.61% | 1.21% |
| Class E | 0.13% | 0.51% | 0.50% |  |  |  |
| Overall | 0.07% | 0.54% | 0.72% | -0.08% | 0.28% | 0.64% |
| Class D | 0.12% | 0.11% | 1.05% | 0.21% | 0.08% | 1.39% |

### Batch size at 32

Table 9: EE1-3.1.4 float vs NNVC-12.0 anchor with LOP5 float. For training LOP5 in EE1-3.1.4, the batch size is set to 32. The other LOP5 training hyperparameters are not modified.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.08% | -0.44% | 0.36% | -0.38% | -0.98% | -0.67% |
| Class A2 | 0.04% | 0.24% | -0.15% | -0.02% | 0.26% | -0.21% |
| Class B | 0.01% | 0.36% | 0.15% | -0.06% | -0.04% | -0.05% |
| Class C | 0.08% | -0.24% | 0.26% | 0.16% | -0.60% | -0.49% |
| Class E | 0.11% | -0.77% | -0.04% |  |  |  |
| Overall | 0.03% | -0.11% | 0.13% | -0.06% | -0.32% | -0.32% |
| Class D | 0.09% | -0.33% | 0.16% | 0.20% | -0.74% | -0.88% |

### Batch size at 128

Table 10: EE1-3.1.4 float vs NNVC-12.0 anchor with LOP5 float. For training LOP5 in EE1-3.1.4, the batch size is set to 128. The other LOP5 training hyperparameters are not modified.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0.06% | 0.10% | 0.70% | -0.08% | -0.14% | 0.75% |
| Class A2 | 0.09% | 0.31% | 0.02% | 0.16% | 0.25% | 0.14% |
| Class B | 0.10% | 0.54% | 0.15% | 0.14% | 0.40% | -0.19% |
| Class C | 0.16% | 0.47% | 0.29% | 0.28% | 0.53% | -0.30% |
| Class E | 0.14% | 0.31% | 0.52% |  |  |  |
| Overall | 0.11% | 0.37% | 0.31% | 0.14% | 0.30% | 0.03% |
| Class D | 0.16% | 0.52% | 0.44% | 0.29% | -0.42% | -1.00% |

### Number of epochs decreased such that the number of iterations is unchanged

Table 11: EE1-3.1.4 float vs NNVC-12.0 anchor with LOP5 float. For training LOP5 in EE1-3.1.4, mse_epoch = 46, max_epochs = 48, and milestones = [33, 41, 44] to maintain approximatively the same number of iterations when adding BVI-AOM to the set of used datasets.  The other LOP5 training hyperparameters are not modified.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0.05% | -0.25% | 0.20% | -0.21% | -0.80% | -0.31% |
| Class A2 | 0.09% | -0.16% | -0.13% | 0.02% | -0.41% | -0.14% |
| Class B | 0.11% | -0.18% | -0.04% | 0.04% | -0.68% | -0.01% |
| Class C | 0.19% | -0.26% | 0.13% | 0.12% | -0.36% | -0.21% |
| Class E | 0.23% | 0.01% | -0.50% |  |  |  |
| Overall | 0.13% | -0.17% | -0.05% | 0.01% | -0.57% | -0.15% |
| Class D | 0.19% | -0.56% | 0.05% | 0.24% | -1.13% | -0.59% |

# Conclusion

From the simulation, no significative changes are observed when using the augmented dataset.

# References

- F. Zhang, D. Bull, J. Nawala, Y. Jiang, X. Zhu, J. Sole, E. Alshina. “[AHG11] Response to Call for training materials for neural network-based video coding tool development”. JVET-AK0255, 37th JVET meeting, Geneva, CH, 14-22 January 2025.

- E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, J. Ström, Z. Xie. “Exploration Experiments on Neural Network-based Video Coding (EE1)”. JVET-AK2023, 37th JVET meeting, Geneva, CH, 14-22 January 2025.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. “Common Test Conditions and evaluation procedures for neural network-based video coding technology”. JVET-Z2016, 26th JVET meeting, by teleconference, 20-29 April 2022.

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).