| Joint Video Experts Team (JVET) of ITU-T SG 21 WP 3 and ISO/IEC JTC 1/SC 29 38th Meeting, by teleconference, 26 March – 4 April 2025 | Document: JVET-AL0205-r1 |
| --- | --- |

| Title: | EE2-2.1: EIP filters with diagonal shapes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krit Panusopone, Michelle He, Seungwook Hong, Limin Wang, Jani Lainema, Dmytro Rusanovskyy | Tel: Email: | Krit.Panusopone@nokia.com Michelle.He@nokia.com Seungwook.Hong@nokia.com Limin.2.Wang@nokia.com Jani.Lainema@nokia.com Dmytro.Rusanovskyy@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports the test results of EE2-2.1, which presents EIP filters with diagonal shapes on top of ECM-16.1. Simulation results, based on ECM-16.1 are reported as follows:

AI: {-0.02%, 0.01%, 0.02%, 100.2%, 100.1%}.

RA: { -0.01%, 0.02%, 0.01%, 100.2%, 100.1%}.

# Introduction

In ECM-16.1, ECM supports three EIP filter shapes, as shown in Fig. 1. In all three EIP filter shapes, their support areas only cover samples above and to the left of the predicting sample.

Fig. 1. Three EIP filter shapes in ECM-16.1

# Proposed Method

EIP filters with diagonal shapes [1] were proposed to include samples above-right and below-left of the predicting sample in their support areas. The three EIP filter shapes proposed in this test are shown in Fig. 2.

X

X

X

X

X

O

X

X

X

X

X

X

X

X

X

X

X

X

X

X

O

X

X

X

X

X

X

X

X

X

X

X

X

X

X

X

O

input of EIP

output of EIP

X

X

X

X

X

X

X

O

X

X

Fig. 2. Proposed three EIP filter shapes

# Experimental Results

The proposed method has been implemented on top of the ECM-16.1 software. The test was conducted based on the common test conditions specified in the JVET document [2].

Table 1. Simulation results

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-16.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | 0.01% | -0.03% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class A2 | -0.01% | -0.11% | -0.01% | 100.1% | 100.1% | 100.0% | 100.1% |
| Class B | -0.01% | 0.01% | 0.09% | 100.2% | 100.2% | 100.0% | 99.8% |
| Class C | 0.01% | 0.06% | 0.06% | 100.6% | 100.0% | 100.0% | 100.0% |
| Class E | -0.04% | 0.08% | -0.08% | 100.1% | 100.2% | 102.1% | 100.0% |
| Overall | -0.02% | 0.01% | 0.02% | 100.2% | 100.1% | 100.3% | 100.0% |
| Class D | -0.01% | -0.01% | 0.06% | 100.8% | 100.3% | 99.9% | 100.0% |
| Class F | -0.02% | -0.14% | 0.14% | 99.9% | 99.6% | 100.4% | 100.0% |
| Class TGM | 0.00% | 0.02% | 0.01% | 100.0% | 99.6% | 97.1% | 100.1% |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-16.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  | -0.01% |  | 0.22% |  |  | 0.07% |  |  |  | 100.2% |  |  | 100.0% |  |  | 100.1% |  |  |  |  |  | 99.9% |  |  |  |
| Class A2 |  |  |  | -0.01% |  | 0.20% |  |  | 0.21% |  |  |  | 100.2% |  |  | 99.8% |  |  | 100.1% |  |  |  |  |  | 100.3% |  |  |  |
| Class B |  | 0.00% |  |  |  |  |  | -0.16% |  |  |  | -0.15% |  |  |  |  | 100.2% |  |  |  | 100.1% |  |  | 100.0% |  |  |  | 100.2% |  |
| Class C |  | -0.03% |  |  |  |  |  | -0.05% |  |  |  | 0.02% |  |  |  |  | 100.4% |  |  |  | 100.3% |  |  | 100.1% |  |  |  | 99.9% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  | -0.01% |  |  |  | 0.02% |  |  |  | 0.01% |  |  |  | 100.2% |  |  |  |  | 100.1% |  | #NUM! |  |  |  | 100.1% |  |  |  |
| Class D |  | -0.07% |  |  |  |  |  | 0.20% |  |  |  | 0.02% |  |  |  |  | 99.9% |  |  |  | 99.6% |  |  | 100.0% |  |  |  | 100.0% |  |
| Class F |  | 0.00% |  |  |  |  |  | 0.16% |  |  |  | -0.20% |  |  |  |  | 100.0% |  |  |  | 98.8% |  |  | 100.0% |  |  |  | 100.1% |  |
| Class TGM |  | -0.03% |  |  |  |  |  | 0.01% |  |  |  | 0.04% |  |  |  |  | 100.4% |  |  |  | 100.7% |  |  | 100.0% |  |  |  | 100.1% |  |

# Conclusions

The simulation results show -0.02%, 0.01%, 0.02% BD-rate gain on Y/Cb/Cr in AI condition and -0.01%, 0.02%, 0.01% BD-rate gain on Y/Cb/Cr in RA condition, respectively. The proposed method demonstrates improved prediction performance of EIP filter with diagonal shapes. It is recommended to adopt the proposed EIP filter shapes in the ECM-17.0.

# References

[1] K. Panusopone, M. He, S. Hong, L. Wang, J. Lainema, “EE2-related: On EIP filter shapes,” JVET-AK0262, Jan. 2025.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017, July 2024.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).