| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 38th Meeting, by teleconference, 26 March – 4 April 2025 | Document: JVET-AL0215-v3 |
| --- | --- |

| Title: | EE2-4.4: IntraNN NSPT set |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gleb Verba, Muhammed Coban, Vadim Seregin, Marta Karczewicz | Tel: Email: | gverba@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2 Test 4.4, proposed in JVET-AK0214. The contribution proposes to use NSPT kernels from the second set of NSPT kernels for IntraNN-coded blocks and to increase the number of non-zero coefficients for several block shapes. The method was implemented on top of ECM-16.1 and the experimental results following CTC are reported as follows:

Test 4.4a:

AI: -0.01% (Y) 0.03% (U) 0.07% (V) 99.9% (EncT) 100.6% (DecT)

RA: -0.02% (Y) 0.06% (U) 0.03% (V) 100.4% (EncT) 100.2% (DecT)

Test 4.4b:

AI: -0.04% (Y) 0.01% (U) -0.01 (V) 99.8% (EncT) 100.2% (DecT)

RA: -0.03% (Y) -0.03% (U) 0.11% (V) 100.3% (EncT) 99.7% (DecT)

Test 4.4c:

AI: -0.09% (Y) 0.03% (U) -0.03% (V) 100.3% (EncT) 100.6% (DecT)

RA: -0.05% (Y) 0.00% (U) 0.11% (V) 100.0% (EncT) 100.1% (DecT)

Test 4.4d:

AI: -0.13% (Y) -0.01% (U) -0.03% (V) 100.9% (EncT) 100.7% (DecT)

RA: -0.07% (Y) -0.03% (U) 0.00% (V) 100.4% (EncT) 100.0% (DecT)

# Introduction

In ECM-16.1, NSPT kernel set is chosen based on the transform block size, prediction mode, intra mode and MTSS [3].

NSPT primary kernel sets for each mode in ECM-16.1 are summarized in the next table.

| NSPT sets | Set 1 | Set 2 | Set 3 |
| --- | --- | --- | --- |
| Prediction modes | Regular intra, PDP, SGPM, IntraNN | TIMD, DIMD, MIP, EIP | IntraTMP, inter |

The amounts of NSPT output coefficients in ECM-16.1 for each block size WxH are summarized below:

| H \W |  |  |  | 4 |  |  | 8 | 16 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 |  |  |  | 16 |  |  | 20 | 24 | 20 |
| 8 |  |  |  | 20 |  |  | 32 | 40 | 24 |
| 16 |  |  |  | 24 |  |  | 40 | - | - |
| 32 |  |  |  | 20 |  |  | 24 | - | - |

The amounts of NSPT kernels in ECM-16.1 for each block size WxH are summarized below:

| H \W | 4 | 8 | 16 | 32 |
| --- | --- | --- | --- | --- |
| 4 | 245 | 221 | 183 | 113 |
| 8 | 245 | 245 | 194 | 127 |
| 16 | 213 | 213 | - | - |
| 32 | 149 | 149 | - | - |

# The worst-case operation count per TB is for block size 8x32/32x8: 24x256 = 6144 MACs.
The worst-case operation count per sample is for block size 8x16/16x8: 40 MACs/sample. [4]

# Test description

In Test 4.4a, IntraNN-coded blocks are set to reuse the second kernel set, shared with TIMD-, DIMD-, MIP-, and EIP-coded blocks. Proposed NSPT sets are summarized in the table below:

| NSPT sets | Set 1 | Set 2 | Set 3 |
| --- | --- | --- | --- |
| Prediction modes | Regular intra, PDP, SGPM | TIMD, DIMD, MIP, EIP, IntraNN | IntraTMP, inter |

In Test 4.4b, all NSPT kernels are retrained.

In Test 4.4c, NSPT sets are set as in Test 4.4a and all NSPT kernels are retrained.

In Test 4.4d, NSPT sets are set as in Test 4.4a, the amount of output non-zero coefficients for NSPT is increased for block sizes 4x8, 4x16, 8x32 and corresponding transposed block shapes , kernel amount is reduced for several block sizes and all NSPT kernels are retrained. The proposed NSPT output coefficient amounts for each block size WxH are summarized below:

| H \W | 4 | 8 | 16 | 32 |
| --- | --- | --- | --- | --- |
| 4 | 16 | 24 | 32 | 20 |
| 8 | 24 | 32 | 40 | 32 |
| 16 | 32 | 40 | - | - |
| 32 | 20 | 32 | - | - |

To compensate for a memory increase arising from kernel size increase, overall kernel amount is decreased in Test 4.4d, achieved by sharing one retrained kernel between a larger number of modes. The proposed amounts of NSPT kernels in for each block size WxH are summarized below:

| H \W | 4 | 8 | 16 | 32 |
| --- | --- | --- | --- | --- |
| 4 | 243 | 206 | 156 | 93 |
| 8 | 245 | 245 | 152 | 110 |
| 16 | 208 | 181 | - | - |
| 32 | 102 | 146 | - | - |

The worst-case operation count per TB for Test 4.4d is for block size 8x32/32x8: 32x256 = 8192 MACs.
The worst-case operation count per sample for Test 4.4d is for block size 8x16/16x8: 40 MACs/sample, which corresponds to ECM-16.1.

In Tests 4.4b-d, the kernels are retrained specially for the configuration of each specific test, to ensure a better adaptation of the kernels.

# Experimental results

## Test 4.4a

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM16.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | 0.02% | 0.01% | 101.3% | 100.4% | 100.0% | 100.1% |
| Class A2 | 0.01% | 0.04% | 0.02% | 99.7% | 100.3% | 100.0% | 100.1% |
| Class B | -0.02% | 0.00% | 0.14% | 99.2% | 100.3% | 98.2% | 100.0% |
| Class C | 0.01% | 0.03% | 0.06% | 100.7% | 101.5% | 99.6% | 100.0% |
| Class E | -0.05% | 0.08% | 0.11% | 98.7% | 100.6% | 98.0% | 100.1% |
| Overall | -0.01% | 0.03% | 0.07% | 99.9% | 100.6% | 99.1% | 100.1% |
| Class D | -0.01% | 0.00% | 0.00% | 99.6% | 99.7% | 99.7% | 100.0% |
| Class F | -0.01% | -0.15% | -0.28% | 99.3% | 100.2% | 100.7% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM16.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | -0.05% | -0.04% | 100.0% | 100.2% | 99.7% | 99.4% |
| Class A2 | 0.01% | 0.22% | 0.08% | 99.0% | 99.9% | 101.1% | 99.8% |
| Class B | -0.01% | 0.03% | -0.03% | 100.7% | 100.4% | 100.0% | 100.0% |
| Class C | -0.03% | 0.05% | 0.12% | 101.5% | 100.1% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.02% | 0.06% | 0.03% | 100.4% | 100.2% | 100.1% | 99.9% |
| Class D | 0.00% | 0.13% | 0.16% | 101.0% | 100.0% | 100.0% | 100.0% |
| Class F | 0.11% | 0.18% | 0.04% | 98.2% | 99.3% | 100.3% | 100.1% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM16.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.02% | -0.08% | -0.15% | 101.0% | 99.3% | 100.0% | 99.9% |
| Class C | -0.13% | -0.15% | -0.45% | 101.1% | 99.8% | 100.0% | 100.1% |
| Class E | -0.02% | -0.08% | 0.62% | 100.6% | 100.0% | 100.0% | 99.7% |
| Overall | -0.06% | -0.10% | -0.05% | 100.9% | 99.7% | 100.0% | 99.9% |
| Class D | -0.05% | -0.65% | -0.56% | 102.1% | 100.3% | 100.0% | 100.0% |
| Class F | -0.18% | -0.34% | -1.02% | 100.2% | 100.4% | 100.5% | 100.0% |

## Test 4.4b

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM16.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | 0.01% | -0.11% | 99.7% | 100.0% | 100.0% | 100.2% |
| Class A2 | -0.04% | -0.01% | -0.05% | 100.9% | 99.4% | 100.0% | 100.1% |
| Class B | -0.06% | 0.04% | 0.03% | 99.5% | 99.6% | 100.0% | 100.0% |
| Class C | 0.00% | 0.03% | 0.03% | 99.6% | 101.6% | 100.8% | 100.0% |
| Class E | -0.07% | -0.05% | 0.02% | 99.8% | 100.1% | 99.0% | 100.0% |
| Overall | -0.04% | 0.01% | -0.01% | 99.8% | 100.2% | 100.0% | 100.0% |
| Class D | -0.03% | -0.08% | 0.08% | 98.0% | 98.9% | 99.8% | 100.0% |
| Class F | 0.01% | -0.12% | -0.32% | 99.4% | 99.7% | 101.7% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM16.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.09% | 0.08% | 99.7% | 99.6% | 99.7% | 100.1% |
| Class A2 | -0.03% | 0.04% | 0.05% | 99.2% | 99.6% | 101.1% | 100.3% |
| Class B | -0.04% | -0.08% | 0.12% | 100.6% | 99.7% | 100.1% | 99.8% |
| Class C | -0.03% | -0.12% | 0.16% | 101.3% | 99.8% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.03% | -0.03% | 0.11% | 100.3% | 99.7% | 100.2% | 100.0% |
| Class D | -0.11% | 0.11% | -0.01% | 100.3% | 99.6% | 100.0% | 100.0% |
| Class F | -0.01% | 0.26% | 0.00% | 98.2% | 99.0% | 100.8% | 100.2% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM16.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.04% | 0.03% | 0.15% | 99.8% | 99.9% | 100.0% | 100.2% |
| Class C | -0.05% | -0.17% | 0.09% | 100.0% | 99.4% | 100.0% | 100.0% |
| Class E | 0.08% | 0.44% | 1.41% | 101.5% | 100.0% | 100.0% | 100.3% |
| Overall | -0.02% | 0.07% | 0.44% | 100.3% | 99.8% | 100.0% | 100.2% |
| Class D | -0.07% | -0.28% | -1.20% | 100.6% | 100.2% | 100.0% | 100.1% |
| Class F | 0.08% | 0.25% | -0.08% | 100.0% | 99.7% | 99.0% | 100.0% |

## Test 4.4c

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM16.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.07% | 0.20% | -0.01% | 100.8% | 100.3% | 100.0% | 100.2% |
| Class A2 | -0.06% | -0.07% | 0.00% | 100.1% | 101.4% | 100.0% | 100.0% |
| Class B | -0.11% | -0.06% | -0.01% | 100.5% | 100.5% | 100.0% | 100.1% |
| Class C | -0.08% | 0.08% | -0.11% | 99.6% | 100.3% | 100.5% | 100.0% |
| Class E | -0.15% | 0.05% | 0.03% | 100.6% | 100.4% | 99.0% | 100.2% |
| Overall | -0.09% | 0.03% | -0.03% | 100.3% | 100.6% | 99.9% | 100.1% |
| Class D | -0.10% | 0.15% | -0.01% | 100.1% | 100.6% | 100.0% | 100.0% |
| Class F | -0.04% | -0.14% | 0.26% | 100.1% | 100.2% | 102.5% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM16.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.03% | 0.07% | 99.9% | 100.1% | 100.0% | 99.5% |
| Class A2 | 0.00% | 0.10% | 0.07% | 99.3% | 99.9% | 101.1% | 99.8% |
| Class B | -0.05% | -0.01% | 0.21% | 99.9% | 100.2% | 100.1% | 100.1% |
| Class C | -0.08% | -0.03% | 0.05% | 100.7% | 99.9% | 99.9% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.05% | 0.00% | 0.11% | 100.0% | 100.1% | 100.2% | 100.0% |
| Class D | -0.10% | 0.00% | 0.15% | 100.3% | 99.9% | 100.0% | 100.0% |
| Class F | -0.04% | 0.40% | 0.07% | 100.5% | 98.2% | 99.8% | 100.1% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM16.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.02% | 0.27% | -0.39% | 100.5% | 100.4% | 100.0% | 100.2% |
| Class C | -0.03% | -0.21% | 0.26% | 99.0% | 99.7% | 100.0% | 99.8% |
| Class E | -0.07% | 0.50% | -0.41% | 99.5% | 98.7% | 100.0% | 100.1% |
| Overall | -0.03% | 0.17% | -0.18% | 99.7% | 99.8% | 100.0% | 100.0% |
| Class D | -0.07% | -0.87% | -0.41% | 100.6% | 100.4% | 100.0% | 100.0% |
| Class F | -0.19% | 0.53% | 0.01% | 100.7% | 101.4% | 100.6% | 100.1% |

## Test 4.4d

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM16.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.09% | 0.00% | -0.03% | 100.8% | 100.2% | 100.1% | 99.9% |
| Class A2 | -0.08% | 0.00% | 0.06% | 100.3% | 101.5% | 100.0% | 100.0% |
| Class B | -0.15% | 0.00% | -0.04% | 101.0% | 100.9% | 100.1% | 100.2% |
| Class C | -0.15% | 0.03% | 0.04% | 100.8% | 100.4% | 100.0% | 100.1% |
| Class E | -0.17% | -0.12% | -0.19% | 101.9% | 100.3% | 97.0% | 100.2% |
| Overall | -0.13% | -0.01% | -0.03% | 101.0% | 100.7% | 99.5% | 100.1% |
| Class D | -0.17% | -0.06% | 0.07% | 101.5% | 100.2% | 99.9% | 100.0% |
| Class F | -0.10% | -0.10% | -0.08% | 100.3% | 100.1% | 101.9% | 99.9% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM16.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | 0.00% | 0.07% | 99.8% | 99.5% | 100.0% | 99.3% |
| Class A2 | -0.03% | 0.14% | 0.02% | 99.9% | 100.0% | 100.8% | 99.7% |
| Class B | -0.09% | -0.07% | -0.03% | 100.7% | 100.3% | 100.0% | 99.9% |
| Class C | -0.10% | -0.14% | -0.04% | 100.9% | 100.0% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.07% | -0.03% | 0.00% | 100.4% | 100.0% | 100.2% | 99.8% |
| Class D | -0.10% | 0.23% | -0.07% | 100.8% | 99.9% | 100.0% | 100.0% |
| Class F | -0.08% | 0.05% | -0.22% | 99.3% | 99.1% | 100.6% | 100.1% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM16.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.04% | 0.29% | -0.50% | 100.2% | 100.3% | 100.0% | 100.0% |
| Class C | -0.16% | 0.10% | 0.06% | 99.9% | 99.3% | 100.0% | 99.8% |
| Class E | -0.03% | -0.08% | 0.24% | 99.1% | 99.7% | 100.0% | 100.1% |
| Overall | -0.08% | 0.13% | -0.13% | 99.9% | 99.8% | 100.0% | 99.9% |
| Class D | -0.07% | -0.27% | -0.13% | 100.9% | 100.2% | 100.0% | 100.0% |
| Class F | -0.56% | -0.31% | -0.44% | 102.2% | 100.9% | 100.1% | 100.1% |

# References

[1] ECM-15.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] G. Verba, M. Coban, V. Seregin, M. Karczewicz “Non-EE2: IntraNN NSPT set”, JVET- AK0214.

[3] M. Coban, M. Karczewicz, P. Nikitin, P. Garus, B. Ray, V. Seregin, F. Wang, Y. Yu, H. Yu, D. Wang, C. Bonnineau, K. Naser, S. Puri, F. L. Léannec, L. Zhao, K. Zhang, L. Zhang “EE2-3.1: Multiple transform set selection for intra LFNST/NSPT with subsampled DIMD for transform set selection”, JVET-AK0217.

[4] G. Boisson, T. Dumas, F. Galpin, S. Puri, K. Naser, C. Bonnineau “AHG7: Complexity and memory footprint assessment of PDP, Neural Network-based Intra Prediction, MTS and LFNST/NSPT, and ALF in ECM-15”, JVET-AK0047-v3, Jan. 2025

[5] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).