| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 38th Meeting, by teleconference, 26 March – 4 April 2025 | Document: JVET-AL0220-v2 |
| --- | --- |

| Title: | AHG12: Reuse of ALF control information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Hongtao Wang, Marta Karczewicz, Vadim Seregin 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | nanh@qti.qualcomm.com hongtaow@qti.qualcomm.com martak@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In ECM-16.1, for each picture, ALF control information is signalled at both picture and CTB levels. In this proposal, CTB-level information chroma ALF and CCALF can be used by future frames. On top of ECM-16.1 following common test conditions, simulation results of the proposed method are reported below:

|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RA | -0.04% | -0.03% | 0.06% | 100.0% | 99.9% | 100.1% | 100.2% |
| LDB | -0.04% | 0.71% | 0.67% | 99.7% | 100.1% | 100.0% | 100.0% |

# Introduction

In ECM-16.1, ALF and CCALF coefficients are derived for each frame and signalled in APSs, which can be re-used by future encoded pictures. In JVET-AK0176, filter control information at CTB levels is also reused by future pictures, however, the encoder derives filter coefficients and control information for a group of non-reference pictures jointly, which may cause encoder latency.

# Proposed method

In this proposal, CTB-level information of chroma ALF and CCALF of the current picture is stored and can be reused by future coded pictures. For each chroma component, two flags are signalled to indicate whether current picture reuses ALF and CCALF CTB-level information from a previously coded picture. Compared to JVET-AK0176, the encoder in this proposal makes the decision for each picture individually such that no latency is introduced.

# Simulation results

The proposed method is implemented on top of ECM-16.1. The simulations were performed under common test conditions defined in JVET-AI2017. The simulation results are reported in the table below.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-16.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | 0.16% | 0.28% | 100.2% | 100.0% | 100.0% | 100.1% |
| Class A2 | -0.04% | -0.02% | 0.11% | 100.3% | 99.8% | 99.3% | 100.5% |
| Class B | -0.04% | -0.10% | -0.08% | 100.0% | 99.7% | 100.7% | 100.1% |
| Class C | -0.03% | -0.09% | 0.02% | 99.8% | 100.0% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.04% | -0.03% | 0.06% | 100.0% | 99.9% | 100.1% | 100.2% |
| Class D | -0.02% | -0.16% | -0.03% | 98.8% | 100.1% | 100.0% | 100.1% |
| Class F | -0.03% | -0.08% | -0.02% | 99.6% | 99.6% | 99.5% | 100.1% |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-16.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.11% | 0.82% | 0.28% | 99.5% | 99.8% | 100.0% | 99.7% |
| Class C | -0.06% | -0.10% | 0.27% | 99.7% | 100.2% | 100.0% | 100.2% |
| Class E | 0.09% | 1.60% | 1.88% | 99.9% | 100.5% | 100.0% | 100.1% |
| Overall | -0.04% | 0.71% | 0.67% | 99.7% | 100.1% | 100.0% | 100.0% |
| Class D | 0.05% | -0.74% | -0.29% | 99.7% | 100.0% | 100.0% | 99.8% |
| Class F | -0.05% | 0.07% | -0.05% | 100.4% | 100.0% | 100.1% | 100.3% |
| Class TGM |  |  |  |  |  |  |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).