| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 39th Meeting, by Daejeon, 26 June – 4 July 2025 | Document: JVET-AM0105 |
| --- | --- |

| Title: | EE2-related: Candidate replacement in MPM list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hao Tian Yanbo Gao Shuai Li Jianjun Lei Bin Li Fang Xing Pengfei Han | Email: | ybgao@sdu.edu.cn |
| Source: | Shandong University, Hisense |  |  |

- Abstract

In EE2, the MPM list is constructed based on the best RD mode of neighbors. In MPM list, it already has an advantage of bitrate saving. Thus the candidates in MPM list should be the mode of neighbors with the lowest distortion. In this proposal, a candidate replacement in MPM list is presented based on the mode of neighbors with the lowest distortion.

On top of ECM-13.0, results of the proposed method are:

AI:  %, %, %, EncT: % , DecT: %.

Introduction

In ECM, the candidate in MPM list is obtained by using the mode of neighbors with the smallest SAD cost. The SAD cost is computed between the prediction and the reconstruction samples of neighbors (including left (L), above (A), below-left (BL), above-right (AR), and above-left (AL) neighbouring blocks). In ECM 13.0, the candidates are planar, DIMD, and the best mode of neighbors with the smallest SAD cost.

Proposed method

In this proposal, the candidates in the MPM list are replaced as follows. The best mode with the smallest SAD cost is replaced by the best mode with the smallest distortion cost.

Specifically, the planar mode is preserved. If the best RD mode of left is angular mode, the neighbors of this angular mode with offset [-2, -1, 0, 1, 2] are tested based on the criteria of the difference between the reconstruction of left and the prediction of the tested angular modes. Then The best RD mode of left is replaced by the tested angular mode with the smallest difference. The same processing is conducted for the best RD mode of other candidates.

Experimental results

The proposed method has been implemented on top of ECM-13.0. Testing results of the proposed method under AI configuration are summarized in Table 1. The experiments were performed following the common test conditions [1]. More detailed results can be found in the attached spreadsheets.

Table 1. experimental results

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.02% | -0.06% | 0.00% | 111% |  |  |  |
| Class C | -0.01% | 0.02% | 0.03% | 107% |  |  |  |
| Class E | -0.09% | -0.14% | 0.03% | 124% |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.01% | -0.20% | -0.12% | 113% |  |  |  |
| Class F | 0.01% | 0.10% | 0.05% | 123% |  |  |  |

References

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017.

Patent rights declaration(s)

Hisense Visual Technology Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Shandong University do not have current or pending patent rights relating to the technology described in this contribution.