| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 39th Meeting, Daejeon, KR, 26 June – 4 July 2025 | Document: JVET-AM0156-v1 |
| --- | --- |

| Title: | AHG9: On Digitally Signing a SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Gao, Ping Wu, Yaxian Bai, Shaowei Xie, Cheng Huang | Tel: Email: | gao.ying@zte.com.cn ping.wu@zte.com.cn huang.cheng5@zte.com.cn |
| Source: | ZTE Corporation |  |  |

_____________________________

- Abstract

This contribution proposes the following changes related to optimize the functionality of digitally signing a SEI message.

Adding a flag(sei_digital_signature_initialization_present_flag) to specify whether the digital signing initialization related syntax elements are present in the SEI message that requires digital signature verification.

Introduction

During the last meeting, JVET-AL0078 proposed to support digitally signing and verifying some of SEI messages. The SEI message carries initialization information and the digital signature related syntax elements, which are used to verify this SEI message according to the proposed mechanism. It is asserted that digital signing the SEI messages provides benefits of authentication and integrity.

Unlike the digital signature, which uniquely corresponds to each SEI message (meaning that the digital signatures for any two SEI messages are different), the digital signature initialization information, including the secure hash algorithm and key, may be same for different SEI messages. Therefore, it is not necessary to include the digital signature initialization related syntax elements in every SEI message that requires digital signature verification.

Proposal

We propose the following modifications to optimize digital signature mechanism of SEI messages:

Adding a flag(sei_digital_signature_initialization_present_flag) to specify whether the digital signing initialization related syntax elements are present in the SEI message.

Only when sei_digital_signature_initialization_present_flag equals to 1, the digital signing initialization related syntax elements are signaled in the SEI message that requires digital signature verification.

Proposed syntax and semantics

Some related syntax elements and semantics text are added in a generic SEI message as highlighted below.

| sei_message_syntax_structure ( payloadSize ) { | Descriptor |
| --- | --- |
| sei_message_cancel_flag | u(1) |
| if( !sei_message_cancel_flag ) { |  |
| sei_message_persistence_flag | u(1) |
| sei_syntax1 | ue(v) |
| sei_syntax2 | u(v) |
| … |  |
| sei_syntaxN | ue(v) |
| sei_digital_signature_present_flag | u(1) |
| if( sei_digital_signature_present_flag ) { |  |
| sei__digital_signature_initialization_present_flag | u(1) |
| if( sei_digital_signature_initialization_present_flag ) { |  |
| sei_digitial_signature_hash_method_type | u(8) |
| sei_digital_signature_key_source_uri | st(v) |
| } |  |
| sei_digital_signature_length_in_octets_minus1 | u(16) |
| sei_digital_signature | u(v) |
| } |  |
| } |  |

sei_digital_signature_present_flag equal to 1 specifies that the digital signing related syntax elements sei__digital_signature_initialization_present_flag, sei_digitial_signature_hash_method_type, sei_digital_signature_key_source_uri, sei_digital_signature_length_in_octets_minus1, sei_digital_signature are present. Sei_digital_signature_present_flag equal to 0 specifies that the digital signing related syntax elements sei_digitial_signature_hash_method_type, sei_digital_signature_key_source_uri, sei_digital_signature_length_in_octets_minus1, sei_digital_signature are not present.

sei__digital_signature_initialization_present_flag equal to 1 specifies that the digital signing initialization related syntax elements sei_digitial_signature_hash_method_type, sei_digital_signature_key_source_uri are present. sei__digital_signature_initialization_present_flag equal to 0 specifies that the digital signing initialization related syntax elements sei_digitial_signature_hash_method_type, sei_digital_signature_key_source_uri are not present. When the digital signing initialization related syntax elements are not present, the default digital signing initialization information could be used.

Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).