| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 39th Meeting, Daejeon, KR, 26 June – 4 July 2025 | Document: JVET-AM0176-v2 |
| --- | --- |

| Title: | EE2-related: ECM NNLF evaluation using AhG11 trained models |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tangi Poirier Franck Galpin | Tel: Email: | tangi.poirier@interdigital.com franck.galpin@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

This contribution reports the performance of the models trained and cross checked in AhG11 using NNVC software using the interface similar to the one used in NNVC software. The results on top of ECM 17.0 are 	as follow:

VLOP3:

- AI configuration {-0.79%,-1.81%, -1.72%} bdrate gains for Y,U,V with {100.1%, 223.0%} Encoding and decoding time

- RA configuration {-1.16%, -1.51%, -1.25%} bdrate gains for Y,U,V with {101.1%,205.1%} Encoding and decoding time

VLOP3 with TDO:

- RA configuration {-0.94%, -1.40%, -1.16%} bdrate gains for Y,U,V with {101.1%,150.9%} Encoding and decoding time

VLOP3 with TDO alternative lambdas:

- RA configuration {-X.XX%, -X.XX%, -X.XX%} bdrate gains for Y,U,V with {XXX.X%, XXX.X%} Encoding and decoding time

LOP5:

- AI configuration {-1.86%, -5.81%, -5.85%} bdrate gains for Y,U,V with {100.3%, 404.2%} Encoding and decoding time

- RA configuration {-2.54%, -5.13%, -4.70%} bdrate gains for Y,U,V with {101.2%, 337.6%} Encoding and decoding time

LOP5 with TDO:

- RA configuration {-2.35%, -5.05%, -4.70%} bdrate gains for Y,U,V with {101.2%, 241.5%} Encoding and decoding time

HOP5:

- AI configuration {-4.88%, -0.65%, -1.94%} bdrate gains for Y,U,V with {115.7%, 13928.2%} Encoding and decoding time

- RA configuration {-7.01%, -1.49%, -2.65%} bdrate gains for Y,U,V with {114.0%, 9890.9%} Encoding and decoding time

# Introduction

In the current ECM, an extension to test Neural network based loop filter has been adopted in JVET-AL0228. Two interfaces are available: one is similar to the one studied in AhG11 using NNVC software, the other one is reusing an input of the ALF tool.

This contribution proposes to test various models trained and crosschecked during the development of NNVC by AhG11, namely:

- VLOP3 model with and without the optional default Time Distortion Optimization (TDO) (--NnlfTdo=1), alternative lambdas for TDO are also tested

- LOP5 model with and without the optional default TDO

- HOP5 model

# Simulations results

## VLOP3

The model is taken from the NNVC repository in models/nnlf_vlop3_model_int16.sadl

### VLOP3

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-17.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -1.17% | -1.59% | -1.59% | 99.7% | 276.2% | 111.6% | 134.2% |
| Class A2 | -0.84% | -1.90% | -1.26% | 100.0% | 229.4% | 111.5% | 133.5% |
| Class B | -0.68% | -2.04% | -1.84% | 100.0% | 218.7% | 108.0% | 121.0% |
| Class C | -0.33% | -1.41% | -1.36% | 100.6% | 186.6% | 108.5% | 113.7% |
| Class E | -1.18% | -2.07% | -2.57% | 100.1% | 229.5% | 106.7% | 117.0% |
| Overall | -0.79% | -1.81% | -1.72% | 100.1% | 223.0% | 109.0% | 122.7% |
| Class D | -0.42% | -1.25% | -1.30% | 100.9% | 178.4% | 107.7% | 111.3% |
| Class F | -0.39% | -1.14% | -0.69% | 100.8% | 202.8% | 110.3% | 116.9% |
| Class TGM | -0.06% | -0.21% | -0.15% | 100.6% | 176.0% | 113.4% | 120.5% |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-17.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -2.02% | -1.48% | -1.35% | 101.1% | 224.9% | 139.9% | 151.0% |
| Class A2 | -1.47% | -1.55% | -1.00% | 101.6% | 196.7% | 140.4% | 149.9% |
| Class B | -1.00% | -1.69% | -1.45% | 100.8% | 213.0% | 130.0% | 133.9% |
| Class C | -0.50% | -1.29% | -1.11% | 101.0% | 188.3% | 123.7% | 121.6% |
| Class E |  |  |  |  |  |  |  |
| Overall | -1.16% | -1.51% | -1.25% | 101.1% | 205.1% | 132.2% | 136.7% |
| Class D | -0.52% | -1.01% | -1.08% | 102.5% | 178.6% | 116.1% | 115.9% |
| Class F | -0.32% | -0.62% | -0.49% | 101.4% | 179.1% | 123.4% | 127.0% |
| Class TGM | -0.08% | -0.21% | -0.14% | 101.2% | 156.0% | 121.4% | 132.8% |

### VLOP3 + TDO

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-17.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -1.66% | -1.17% | -1.14% | 101.0% | 170.6% | 140.0% | 151.2% |
| Class A2 | -1.23% | -1.49% | -0.99% | 101.8% | 144.2% | 140.5% | 150.1% |
| Class B | -0.84% | -1.70% | -1.50% | 100.7% | 153.6% | 130.0% | 134.2% |
| Class C | -0.31% | -1.14% | -0.86% | 100.9% | 139.2% | 123.7% | 121.5% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.94% | -1.40% | -1.16% | 101.1% | 150.9% | 132.3% | 136.9% |
| Class D | -0.37% | -0.96% | -1.02% | 102.3% | 140.9% | 116.0% | 115.9% |
| Class F | -0.22% | -0.64% | -0.48% | 101.4% | 141.8% | 123.3% | 127.0% |
| Class TGM | -0.09% | -0.20% | -0.15% | 100.9% | 124.8% | 121.5% | 132.4% |

### VLOP3 + alternative TDO

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-17.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class C | -0.22% | -1.19% | -0.98% | 101.5% | 123.2% | 123.7% | 121.5% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.31% | -0.89% | -0.99% | 102.6% | 126.0% | 116.1% | 115.9% |
| Class F | -0.16% | -0.65% | -0.40% | 101.5% | 129.1% | 123.3% | 127.0% |
| Class TGM | -0.10% | -0.22% | -0.17% | 101.4% | 119.8% | 121.4% | 132.7% |

## LOP5

The model is taken from the NNVC repository in models/nnlf_lop5_model_int16.sadl

### LOP5

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-17.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -2.45% | -5.82% | -5.96% | 100.9% | 531.9% | 113.6% | 142.9% |
| Class A2 | -1.67% | -6.20% | -4.25% | 99.6% | 421.7% | 113.5% | 142.1% |
| Class B | -1.54% | -5.78% | -6.17% | 100.0% | 393.0% | 112.4% | 136.6% |
| Class C | -1.26% | -4.89% | -5.21% | 100.5% | 314.0% | 117.7% | 134.7% |
| Class E | -2.81% | -6.66% | -7.67% | 100.7% | 432.1% | 115.6% | 136.6% |
| Overall | -1.86% | -5.81% | -5.85% | 100.3% | 404.2% | 114.4% | 138.1% |
| Class D | -1.49% | -4.70% | -4.72% | 99.9% | 291.5% | 117.8% | 132.7% |
| Class F | -1.12% | -3.53% | -2.61% | 100.5% | 361.0% | 116.6% | 135.5% |
| Class TGM | -0.26% | -0.50% | -0.27% | 99.5% | 315.3% | 119.0% | 136.2% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-17.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -3.43% | -5.01% | -4.78% | 101.2% | 370.9% | 141.1% | 156.4% |
| Class A2 | -3.12% | -5.34% | -3.38% | 101.3% | 327.8% | 141.6% | 154.8% |
| Class B | -2.33% | -5.44% | -5.60% | 101.1% | 348.0% | 132.9% | 145.2% |
| Class C | -1.70% | -4.66% | -4.50% | 101.1% | 309.7% | 130.7% | 139.8% |
| Class E |  |  |  |  |  |  |  |
| Overall | -2.54% | -5.13% | -4.70% | 101.2% | 337.6% | 135.6% | 147.8% |
| Class D | -1.96% | -4.58% | -4.20% | 101.2% | 294.9% | 125.0% | 135.8% |
| Class F | -0.94% | -2.53% | -2.09% | 101.9% | 281.5% | 128.2% | 142.3% |
| Class TGM | -0.35% | -0.55% | -0.42% | 101.8% | 239.7% | 123.5% | 143.9% |

### LOP5 + TDO

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-17.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -3.19% | -4.80% | -4.77% | 101.3% | 280.7% | 141.1% | 156.5% |
| Class A2 | -2.89% | -5.28% | -3.28% | 101.3% | 216.3% | 141.5% | 154.9% |
| Class B | -2.21% | -5.35% | -5.56% | 101.2% | 246.1% | 132.9% | 145.2% |
| Class C | -1.48% | -4.70% | -4.65% | 101.2% | 228.7% | 130.7% | 139.8% |
| Class E |  |  |  |  |  |  |  |
| Overall | -2.35% | -5.05% | -4.70% | 101.2% | 241.5% | 135.6% | 147.8% |
| Class D | -1.78% | -4.59% | -4.11% | 100.9% | 243.6% | 125.0% | 135.9% |
| Class F | -0.90% | -2.48% | -2.10% | 102.0% | 223.2% | 128.1% | 142.2% |
| Class TGM | -0.40% | -0.63% | -0.45% | 101.7% | 173.4% | 123.5% | 144.0% |

## HOP5

The model is taken from the NNVC repository in models/nnlf_hop5_model_int16.sadl

--CbQpOffset=1 --CrQpOffset=1 has been used for HOP5.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-17.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -4.58% | 4.27% | -2.36% | 132.2% | 20575.0% | 116.6% | 156.1% |
| Class A2 | -4.98% | 0.59% | 0.51% | 114.4% | 14976.4% | 116.3% | 154.6% |
| Class B | -4.00% | -0.43% | -0.35% | 112.6% | 13390.0% | 119.0% | 160.3% |
| Class C | -4.82% | -3.65% | -4.06% | 107.1% | 9606.8% | 130.0% | 166.0% |
| Class E | -6.61% | -3.20% | -3.77% | 119.0% | 15365.4% | 124.9% | 166.0% |
| Overall | -4.88% | -0.65% | -1.94% | 115.7% | 13928.2% | 121.5% | 160.8% |
| Class D | -4.92% | -3.22% | -4.44% | 106.4% | 8672.7% | 134.7% | 168.9% |
| Class F | -4.09% | -2.06% | -0.18% | 112.4% | 11985.1% | 127.3% | 163.5% |
| Class TGM | -2.14% | 1.80% | 1.97% | 114.0% | 10549.7% | 129.5% | 159.4% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-17.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -8.50% | 1.61% | -4.64% | 115.2% | 12438.5% | 142.7% | 164.9% |
| Class A2 | -8.37% | -1.85% | -2.85% | 114.6% | 8600.1% | 143.0% | 161.1% |
| Class B | -6.17% | -2.25% | -0.73% | 116.0% | 10567.4% | 137.2% | 161.9% |
| Class C | -5.93% | -2.61% | -3.40% | 110.2% | 8515.7% | 141.3% | 167.3% |
| Class E |  |  |  |  |  |  |  |
| Overall | -7.01% | -1.49% | -2.65% | 114.0% | 9890.9% | 140.6% | 163.8% |
| Class D | -6.57% | -4.29% | -5.21% | 110.5% | 8174.7% | 140.6% | 169.9% |
| Class F | -4.11% | 0.04% | 0.23% | 140.3% | 7761.6% | 134.5% | 164.6% |
| Class TGM | -3.02% | 1.76% | 1.42% | 145.6% | 7364.5% | 126.6% | 159.9% |

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).