| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 39th Meeting, Daejeon, KR, 26 June – 4 July 2025 | Document: JVET-AM0185-v2 |
| --- | --- |

| Title: | AhG11: Decomposed Content-Adaptive VLOP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhuowei Xu Jacek Konieczny Alexey Filippov Christopher Hollmann Vasily Rufitskiy Tianyu Dong Hongdong Qin | Tel: Email: | zhuowei.xu@tcl.com jacek.k@tcl.com alexey.filippov@tcl.com christopher.hollmann@tcl.com vasily.rufitskiy@tcl.com tianyu.dong@tcl.com hongdong.qin@tcl.com |
| Source: | TCL |  |  |

- Abstract

This contribution studies the dimension-wise decomposed representation of multiplier in case of content-adaptive VLOP (CAVLOP). The BD-rate gain under RA configuration are reported as below.

- Compared with NNVC-13 with CAVLOP ON, the averaged gain is reported to be:

{-0.09 %, -0.40%, -0.32%, EncT: XX.X%, DecT: XX.X%}

Introduction

In JVET-AK0175 [1], a dimension-wise decomposed representation of layer multipliers is proposed, for a purpose of content adaptation of Neural Network-based Loop Filter (NNLF).

The proposed decomposed multiplier is achieved by applying a dimension-wise decomposition on the one-dimensional multiplier, as illustrated in Fig 1. Specifically, for a selected multiplier m, it is obtained by overfitting two low-rank matrices as:

|  | m=flatten(Wm1×Wm2⊤) | Eq. (1) |
| --- | --- | --- |

where × is a product operation, and Wm1∈Rd1×r, Wm2∈Rd2×r are dimension-wise decomposed components, d1 and d2 are sizes of decomposition and d1×d2=cm, r is the rank of decomposition and flatten⋅ is a tensor flattening operation to align the result with the original shape of m:

|  | Rd1×d2→ Rcm | Eq. (2) |
| --- | --- | --- |

Fig 1. Decomposed multipliers on LOP3 filter [1].

This tool is demonstrated to be useful in the case of content-adaptive model of LOP (i.e., DCALOP3 in JVET-AK0175 [1], DCALOP4 in JVET-AJ0169 [2]) and gets adopted in NNVC-13.

Proposal

In this proposal, the dimension-wise decomposed multiplier is tested under content-adaptive model of VLOP [3] (as DCAVLOP). Experimental results are provided under NNVC-13.

Overfitting

Table 2 shows the details of NN information during the overfitting stage.

Table 3. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Overfitting Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU NVIDIA A10-24GB |
|  | Framework: | PyTorch 1.12.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 100 epochs |
|  | Batch size: | 64 |
|  | Loss function: | Weighted-MSE |
|  | Training time (for 1 model – 1 segment): | A1 & A2: from 1.6h to 5.6h B: from 0.4h to 2.6h C: ~ 0.3h D: ~ 0.1h F: from 0.2h to 2.5h |
|  | Training data information: | JVET CTC RA mandatory sequences |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144×144 |
|  | Learning rate: | 2e-3 (for MLB=1,2,3) / 1e-3 (for MLB =0) |
|  | Learning rate update strategy | Constant learning rate |
|  | Optimizer: | ADAM |
|  | Preprocessing: | Up-sample 72x72 UV signal to 144x144 blocks Normalize to 0~1 |
|  | Mini-batch selection process: |  |
|  | Training data update strategy: |  |
|  | Other information: |  |

Inference

Table 3 describes the loop-filter information at inference stage.

Table 4. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters | 16955 |
|  | Parameter Precision (Bits) | Int16 |
|  | Memory Parameter (MB) | 0.03 |
|  | MAC/Pixel (Kilo) | 5.48 |
| Optional |  |  |
|  | Total Conv. Layers | 106 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size | 128x128 / 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Experimental results

DCAVLOP is tested over NNVC-13.

Results on VLOP over NNVC-13

Table 3 shows the coding performance of DCAVLOP in the RA configuration, compared with NNVC-13 with CAVLOP [4] ON.

Table 3. Coding performance of DCAVLOP, compared with CAVLOP

|  | BD-rate Over NNVC-13+CAVLOP |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | -0.06% | 0.16% | -0.55% | -0.08% | 0.02% | -0.81% |  |  |  |
| Class A2 | -0.18% | -0.69% | -0.31% | -0.13% | -0.52% | -0.22% |  |  |  |
| Class B | -0.04% | -0.75% | -0.43% | -0.05% | -1.36% | -0.22% |  |  |  |
| Class C | -0.10% | -0.17% | -0.02% | -0.07% | -0.31% | -0.16% |  |  |  |
| Overall | -0.09% | -0.40% | -0.32% | -0.08% | -0.64% | -0.32% |  |  |  |
| Class D | -0.03% | -0.19% | -0.19% | 0.00% | -0.29% | -0.36% |  |  |  |
| Class F | -0.33% | -1.11% | -0.88% | 1.63% | 1.65% | 2.10% |  |  |  |

Recommendation

It is recommended to study the DCAVLOP in next round of EE1.

References

- Z. Xu, J. Konieczny. AhG11: AhG11: Dimension-wise decomposed multiplier for content-adaptive loop-filter. JVET-AK0175. Jan 2025.

- Z. Xu, J. Konieczny, A. Filippov, C. Hollmann, V. Rufitskiy, T. Dong. EE1-1.3 Dimension-wise decomposed representation of multiplier for content-adaptive loop-filter. JVET-AJ0169. Apr 2025.

- E. Alshina, R. Chang, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, Z. Xie. Exploration Experiments on Neural Network-based Video Coding (EE1). JVET-AJ2023. Nov 2025.

- R. Yang, M. Santamaria, F. Cricri, M.M. Hannuksela, D.Bugdayci Sansli, A.Hallapuro, J.Lainema, H. Zhang (Nokia), D.Rusanovskyy, Y.Li, M.Karczewicz (Qualcomm). 	EE1-1.4: Content-adaptive VLOP-filter. JVET-AI0157. Jul 2024.

- Patent rights declaration(s)

TCL may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).