| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 39th Meeting, Daejeon, KR, 26 June – 4 July 2025 | Document: JVET-AM0339-v2 |
| --- | --- |

| Title: | Informal viewing of JVET-AM0078 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Christan Lehmann | Email: | christian.lehmann@hhi.fraunhofer.de |
| Source: |  |  |  |

_____________________________

# Abstract

This document reports the outcome of the informal viewing of JVET-AM0078 [1]. The viewing was made as an A-B test on the two lowest bitrate points of AHG 17 test content classes that use random access configuration. The viewing indicates a clear preference for JVET-AM0045 [2] rather than JVET-AM0078 which also include JVET-AM0045.

# Test setup

The viewing was performed by 7 participants in the viewing room 211. The viewing follows an A-B test procedure where the viewers are not informed about the origin of A and B and just vote which version looks better.

The viewing was split into 2 sessions, one UHD and a HD session, by running the two lowest bitrate points of each sequence in a sequential order. Both sessions where build with JVET-AM0045 as A, and JVET-AM0078 as B.

The test content was compressed video by VTM-23.9 [3] using JVET-AM0045 or JVET-AM0078 following the random access categories of JVET-AL2026 [4].

# Results

The following raw results were obtained (no outlier rejection or other processing was performed):

Session 1 UHD:

Votes A = AM0045:	104

Votes B = AM0078:	55

Session 2 HD:

Votes A = AM0045:	90

Votes B = AM0078:	44

Summarized Votes for AM0045:	194

Summarized Votes for AM0078:	99

# References

[1] K. Andersson, “AHG17: Suggestion to enable DMVR encoder control for VTM anchor “, JVET-AM0078, 39th Meeting, Daejeon, June 2025.

[2] K. Andersson, P. Wennersten, “AHG17: Generated VTM anchor bitstreams using LambdaScaleTowardsNextQP”, JVET-AM0045, 39th Meeting, Daejeon, June 2025.

[3] VTM-23.9, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-23.9

[4] J.-R. Ohm, M. Wien, “Draft Joint Call for Evidence on video compression with capability beyond VVC”, JVET-AL2026, 38th Meeting, by teleconference, April 2025.

# Annex A: Table with votes per volunteer

|  | C2 | A1 | A2 | A3 | D1 | D2 | D3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UHD | V1 | V2 | V3 | V4 | V5 | V6 | V7 | #VOTES A | #VOTES B |
| HRC1 | A | B | A | B | A | A | A | 5 | 2 |
| HRC1 | A | A | B | B | A | A | B | 4 | 3 |
| HRC2 | A | A | A | A | A | A | A | 7 | 0 |
| HRC2 | A | A | A | - | B | A | A | 5 | 1 |
| HRC3 | A | A | B | B | A | A | A | 5 | 2 |
| HRC3 | B | A | B | - | A | B | A | 3 | 3 |
| HRC4 | A | A | A | - | A | A | B | 5 | 1 |
| HRC4 | A | A | A | - | B | B | B | 3 | 3 |
| HRU1 | A | A | A | - | A | A | A | 6 | 0 |
| HRU1 | A | B | B | B | A | A | A | 4 | 3 |
| HRU2 | A | B | A | A | A | A | A | 6 | 1 |
| HRU2 | A | A | B | A | A | A | B | 5 | 2 |
| HRU3 | A | B | B | A | B | B | A | 3 | 4 |
| HRU3 | A | A | A | B | B | B | B | 3 | 4 |
| HRU4 | A | B | A | A | A | A | B | 5 | 2 |
| HRU4 | A | B | B | - | A | A | A | 4 | 2 |
| SRU1 | A | A | A | B | A | A | B | 5 | 2 |
| SRU1 | A | B | B | B | B | B | B | 1 | 6 |
| SRU2 | A | A | A | - | A | A | A | 6 | 0 |
| SRU2 | A | B | B | A | B | B | A | 3 | 4 |
| SRU3 | A | A | A | - | A | A | A | 6 | 0 |
| SRU3 | B | A | B | - | B | B | A | 2 | 4 |
| SRU4 | A | B | A | A | A | A | A | 6 | 1 |
| SRU4 | B | A | A | B | B | B | B | 2 | 5 |
|  |  |  |  |  |  |  | UHD | 104 | 55 |
| HD | V1 | V2 | V3 | V4 | V5 | V6 | V7 | #VOTES A #VOTES B |  |
| SRH1 | B | A | A | A | A | A | B | 5 | 2 |
| SRH1 | A | A | A | - | A | B | B | 4 | 2 |
| SRH2 | A | A | A | B | A | A | A | 6 | 1 |
| SRH2 | B | B | A | - | A | B | A | 3 | 3 |
| SRH3 | A | B | A | - | A | B | A | 4 | 2 |
| SRH3 | B | A | B | B | B | A | A | 3 | 4 |
| SRH4 | A | A | A | A | B | A | A | 6 | 1 |
| SRH4 | A | A | A | - | B | B | B | 3 | 3 |
| URH1 | A | B | A | A | B | A | B | 4 | 3 |
| URH1 | A | A | B | A | B | B | A | 4 | 3 |
| URH2 | A | A | A | A | A | A | A | 7 | 0 |
| URH2 | A | A | B | B | B | A | B | 3 | 4 |
| URH3 | A | A | A | - | A | A | A | 6 | 0 |
| URH3 | A | B | B | B | A | A | A | 4 | 3 |
| URH4 | A | A | A | B | A | A | A | 6 | 1 |
| URH4 | B | A | B | A | A | A | B | 4 | 3 |
| URH5 | A | B | A | A | B | A | B | 4 | 3 |
| URH5 | A | A | B | A | B | B | B | 3 | 4 |
| URH5 | A | B | A | A | A | A | A | 6 | 1 |
| URH6 | A | B | A | - | A | A | A | 5 | 1 |
|  |  |  |  |  |  |  | HD | 90 | 44 |
|  |  |  |  |  |  |  | Total | 194 | 99 |