| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 39th Meeting, Daejeon, KR, 26 June – 4 July 2025 | Document: JVET-AM1008-v1 |
| --- | --- |

| Title: | Conformance testing for HEVC multiview extended and monochrome profiles |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | I. Moccagatta, T. Fu S. Paluri, A. Tourapis | Email: | iole.moccagatta@intel.com futianliang.video@bytedance.com s_paluri@apple.com atourapis@apple.com |  |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document is draft 3 of the additional conformance testing specification for HEVC, for testing the coding features supported in the multiview extended and monochrome profiles (Rec. ITU T H.265 | ISO/IEC 23008-2)

Due to the size of the electronic attachments, the associated conformance test set data files are provided separately, at the following location:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/HEVCMultiview/under_test/

(user id: avguest, password: Avguest201007)

For ftp access, it is suggested to use FileZilla, for which the site manager feature should be used. In FileZilla, ftp encryption should be set to “Require implicit ftp over TLS” and minimum TLS level should be set to 1.0.

Alternatively, the same files are available by http at:

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/HEVCMultiview/under_test/

Draft changes to the specification text

# 6.6.14

# In subclause 6.6.14, add subclauses as follows:

#### 6.6.14.10	Test bitstream MVHEVCS-J

Specification: All the slices are coded as I, P or B slices. The base layer is coded to conform to Main 10 and the enhancement layer is coded to conform to Multiview Extended 10.

The number of layers is 2, the resolution is 1024x768 and the number of frames is 300.

Functional stage: Decoding of two views.

Purpose: To conform to the Multiview Extended 10 profile.

#### 6.6.14.11	Test bitstream MVHEVCS-K

Specification: All the slices are coded as I, P or B slices. The base layer is coded to conform to Main 10 and the enhancement layer is coded to conform to Multiview Extended 10.

The number of layers is 2, the resolution is 1024x768 and the number of frames is 300.

Functional stage: Decoding of two views.

Purpose: To conform to the Multiview Extended 10 profile.

#### 6.6.14.12	Test bitstream MVHEVCS_L

Specification: All the slices are coded as I, P or B slices. The base layer is coded to conform to Main profile and the enhancement layer is coded to conform to Multiview Extended profile.

The number of layers is 2, the resolution is 1024x768 and the number of frames is 300.

Functional stage: Decoding of two views.

Purpose: To conform to the Multiview Extended profile.

#### 6.6.14.12	Test bitstream MVHEVCS_M

Specification: All the slices are coded as I, P or B slices. The base layer is coded to conform to Main profile and the enhancement layer is coded to conform to Multiview Extended profile.

The number of layers is 2, the resolution is 1024x768 and the number of frames is 300.

Functional stage: Decoding of two views.

Purpose: To conform to the Multiview Extended profile.

#### 6.6.14.12	Test bitstream MVHEVCS_N

Specification: All the slices are coded as I, P or B slices. The base layer is coded to conform to Main 10 and the enhancement layer is coded to conform to Multiview Main 10.

The number of layers is 2, the resolution is 1024x768 and the number of frames is 300.

Functional stage: Decoding of two views.

Purpose: To conform to the Multiview Main 10 profile.

#### 6.6.14.12	Test bitstream MVHEVCS_O

Specification: The base layer is coded to conform to Main 10 and the enhancement layer is coded to conform to Multiview Main 10.

The number of layers is 2, the resolution is 1920x1088 and the number of frames is 300.

Functional stage: Decoding of two views.

# Purpose: To conform to the Multiview Main 10 profile.

# 6.7

# In subclause 6.7 (Normative conformance test suites for Rec. ITU-T H.265 | ISO/IEC 23008-2), add subclauses as follows:

    - Bitstreams for Multiview Extended and Multiview Extended 10 profile

Legend:

X – Bitstream that a decoder conforming to the Main tier needs to decode for static and dynamic test

| Table 8 – Bitstreams for Multiview Extended and Multiview Extended 10 profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Base layer profile |  |  |  |  |  |
| Categories | Subcategory | Bitstream | File name | Main | Main 10 | Main Still Picture | Main tier | Level | Frame rate (Frames/sec) |
| Prediction Structure | Inter-view prediction | MVHEVCS-J | MVHEVCS_J_APPLE_2 |  | X |  |  | 4 | 30 |
| (2-view) | Inter-view prediction | MVHEVCS-K | MVHEVCS_K_APPLE_2 |  | X |  |  | 4 | 30 |
|  | Inter-view prediction | MVHEVCS-L | MVHEVCS_L_Bytedance_2 | X |  |  |  | 4 | 30 |
|  | Inter-view prediction | MVHEVCS-M | MVHEVCS_M_Bytedance_2 | X |  |  |  | 4 | 30 |
|  | Inter-view prediction | MVHEVCS-N | MVHEVCS_N_APPLE_1 |  | X |  |  | 4 | 30 |
|  | Inter-view prediction | MVHEVCS-O | MVHEVCS_O_APPLE_1 |  | X |  |  | 4 | 30 |