| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 42nd Meeting, Santa Eulària, ES, 24 April – 1 May 2026 | Document: JVET-0049_v2 |
| --- | --- |

| Title: | [AHG11] On NNVC training sets |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Elena Alshina Franck Galpin Jay N. Shingala Nisha Bhaskar Thierry Dumas Viacheslav Khamidullin | Email: | elena.alshina@huawei.com franck.galpin@interdigital.com jay.shingala@ittiam.com nisha.bhaskar1@huawei.com Thierry.Dumas@InterDigital.com vyacheslav.khamidullin1@huawei.com |
| Source: | Huawei, Interdigital, Ittiam |  |  |

_____________________________

# Abstract

This document clarifies the situation regarding the training sets available for JVET and specifies which sequences were used for each NNVC tools training

# Introduction

A fair comparison of learning-based tools is possible only when the same training set is used. Common practice in EE1 to recommend for adoption only tools for those training was successfully cross-checked. The NNVC activity started a long time ago. In 2024, a call for training materials was issued and several proposals were received. For some training sets, JVET received explicit permission to use materials for standardization purposes. Anticipating CfP, it is reasonable to clarify the situation with training data sets.

# Available training sets

Table 1 below summarizes all training data sets and their locations. The abbreviation ‘RWTH’ refers to the collection of videos previously hosted by RWTH, used or considered in the past JVET and JCTVC activities. None of the training sets (except ‘RWTH’) are hosted by JVET.

For six out of 11 training sets listed in the Table 1 JVET got explicit permission for materials to be used in standardization. Owners of DIV2K explicitly specify their training set is for research purposes only. It is recommended not to use DIV2K and exclude it from the list of training sets.

The situation with UGC and Vimeo 90K is not clear.

Table 1 Training sets, their location and permission to be used for standardization

| Data set | can use? | Location |
| --- | --- | --- |
| BVI-DVC | yes | https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn |
| TVD | yes | https://multimedia.tencent.com/resources/tvd |
| UGC | not clear | https://media.withyoutube.com/ |
| BVI-AOM | yes | http://download.opencontent.netflix.com.s3.amazonaws.com/index.html?prefix=bvi_aom_dataset/ |
| vimeo90K-triplet | not clear | http://toflow.csail.mit.edu/index.html#triplet |
| vimeo90K-septuplet | not clear | http://toflow.csail.mit.edu/index.html#septuplet |
| RWTH | yes | https://vqa.lfb.rwth-aachen.de |
| DIV2K_train | Research purpose only | http://data.vision.ee.ethz.ch/cvl/DIV2K/DIV2K_train_HR.zip |
| DIV2K_valid |  | http://data.vision.ee.ethz.ch/cvl/DIV2K/DIV2K_valid_HR.zip |
| JVET-AH0298 | yes | https://vqa.lfb.rwth-aachen.de/index.php/apps/files/files/550079?dir=/jvet/ctc/gaming/G1. |
| JVET-AH0120 | yes | https://vqa.lfb.rwth-aachen.de/index.php/apps/files/files/551240?dir=/jvet/ahg/trainingset/JVET-AH0120 |

There are three major tools in NNVC: NN-Intra, NN-based filters and DRF.

Training procedures for all of them are described in JVET-AO2019 document. Many useful details for training reproduction (including list of sequences) can be found in training scripts:  https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/NNVC/training/training_scripts/NN_Filtering/LOP?ref_type=heads .

Training of NN-based filter consists of three stages, training from the scratch requires a lot of steps, several thousand of encodings, 3 full trainings and large disk space available. This is why it was agreed among NNVC participants that only stage 3 of training will be performed for proposals targeting to modify NN-based filter. It is known that re-training from scratch provides ~0.3% extra BD-rate gain. Table 2 summarizes information about usage of training sets for NNVC tools training.

It should be noticed that BVI-DVC and BVI-AOM consist of 193 and 239 videos correspondently. Each video comes in four different resolutions. At the beginning, JVET received permission to use only 191 videos from BVI-DVC, so only 191 out of 193 were used in NNVC training. Later, permission was extended to all 193 videos.

In order to keep training time reasonably short not all, but only 20 (out of 191 for BVI-DVC) videos of 4K size were used in NNVC training.

Table 2 Training sets usage for NNVC tools training.

| Data set | NN-filter stages 1-2 | NN-filter stage 3 | NN-Intra | DRF |
| --- | --- | --- | --- | --- |
| BVI-DVC* | yes | yes | yes | no |
| TVD* | yes | yes | yes | no |
| UGC | no | no | yes | no |
| BVI-AOM | no | no | no | no |
| vimeo90K-triplet | no | no | no | yes |
| vimeo90K-septuplet | no | no | no | no |
| RWTH | no | no | yes | no |
| DIV2K_train | yes | yes | yes | no |
| DIV2K_valid |  | yes | yes | no |
| JVET-AH0298 | no | no | no | no |
| JVET-AH0120 | no | no | no | no |

Excel sheet attached to this contribution contacting detailed information about all sequences in all listed above training sets (including md5 check sums).

# Analysis of available training sets

The wider variety of video scenes used to train a NN-based solution; the higher chances developed solution will work for a wide range of applications.  Different training sets contain different numbers of scenes. Table 3 summarizes the number of scenes in each available dataset and the number of scenes actually used in NNVC training.  It appears that just a small portion of available training materials was used for NNVC tools training, which potential limits generality of developed solution.

From this table, it is clear that the number of scenes in video materials available for NNVC training can be drastically increased.

Since DIV2K use conditions are ‘for research only’, it might be good to avoid using this set of videos for training. Instead, JPEG AI training set can be recommended. JPEG AI training sets are 5 times larger than DIV2K and was already used for international standard development.

Since some sequences from ‘RWTH’ are considered for CfP it might be good to avoid using this set of videos for training.

Video sequences proposed in JVET-AH0298 have become a part of gaming test conditions used in AhG15.

Table 3 Number of video sequences

| Data set | Total | Used for NN-filter | Used for NN-Intra | Used for DRF |
| --- | --- | --- | --- | --- |
| BVI-DVC | 193 | 191 | 191 | 0 |
| TVD | 86 | 20 | 86 | 0 |
| UGC | 130 | 0 | 130 | 0 |
| BVI-AOM | 239 | 0 | 0 | 0 |
| vimeo90K-triplet | 73191 | 0 | 0 | 73191 |
| vimeo90K-septuplet | 91701 | 0 | 0 | 0 |
| RWTH | 100 | 0 | 88 | 0 |
| DIV2K_train | 800 | 800 | 800 | 0 |
| DIV2K_valid | 100 | 100 | 100 | 0 |
| JVET-AH0298 | 5 | 0 | 0 | 0 |
| JVET-AH0120 | 9 | 0 | 0 | 0 |
| Total | 166554 | 1111 | 1241 | 73191 |

# Suggested steps

For now training materials are hosted by third parties and there is a risk materials will be lost or modified. For the next generation video coding standard development or even before CfP it would be good to copy all materials JVET has permission to be used in standardization process to https://vqa.lfb.rwth-aachen.de.

In order to simplify re-training only stage 3 of NN-based loop filter it is suggested to make stage 3 materials available on to https://vqa.lfb.rwth-aachen.de. Currently HOP and LOP use different set of training materials for stage 3 (LOP and VLOP use the same). It is suggested to unify and use the only one (from previous study in AhG11 difference would be small). Streams, reconstructions, patches – all materials needed for stage 3 towhich simplify data loading will take about 4 TB.

For AhG11 and EE1 proposals it must be very clearly mentioned which data set / sub-set was used for training and if training of NN-filter was stage 3 only or filter was re-trained from the scratch.

# Conclusion

It is suggested to up-date the NNVC training sets, includes two missed BVI-DVC sequences, 66 unused before TVD sequences, 9 sequences proposed in  JVET-AH0120 and 239 sequences from BVI-AOM data set. DIV2K is available for research purposes only. JVET might consider excluding it from NNVC training set. Training of DRF could benefit if vimeo90K-septuplet will be used additionally to vimeo90K-triplet.