| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 42th Meeting, by SantaEularia, 24 April – 1 May 2026 | Document: JVET-AP0123 |
| --- | --- |

| Title: | EE2-related: Matrix based MPM prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yanbo Gao Hao Tian Shuai Li Jianjun Lei Bin Li Fang Xing Pengfei Han | Email: | ybgao@sdu.edu.cn |
| Source: | Shandong University, Hisense |  |  |

- Abstract

In EE2, the MPM list is constructed to find the best RD mode for the current block based on the best RD mode of neighbors. That is because the best RD mode of the current block is related to the best mode of neighbors. These best mode is related to each other, yet not necessarily exactly the same. Thus a matrix based MPM prediction is presented with , where the relationship between the best mode of the current block and the neighbors is explored. And no additional complexity is introduced.

On top of ECM-19.0, results of the proposed method are:

AI:  -0.01%	-0.07% 	-0.04% , EncT: % , DecT: %.

Introduction

In ECM, the candidate in MPM list is obtained by using the mode of neighbors with the smallest SAD cost. The SAD cost is computed between the prediction and the reconstruction samples of neighbors (including left (L), above (A), below-left (BL), above-right (AR), and above-left (AL) neighbouring blocks). In ECM 19.0, the candidates are planar, DIMD, and the best mode of neighbors with the smallest SAD cost.

Proposed method

In this proposal, the best mode of the current block is predicted by the matrix based method and then inserted in the MPM list.

First, the statistical data, including the best mode of the current block and the corresponding neighbors, is collected based on CTC sequences and BVI-DVC dataset under AI configuration, where the best mode of current block is noted as Y, and that of the neighbors is denoted as X. Their relationship can be represented as Y=AX, where A denotes the relationship between the current block and neighbors. A is optimized offline using the Sum of Absolute Differences (SAD) criterion coupled with the Quasi-Newton method and A serves as a constant term in the ECM. This is why our method does not introduce additional complexity.

Then, during the encoding of the current block in the ECM, the best mode of neighboring blocks is collected as x, and matrix A is subsequently used to predict y via the model y=Ax, which is inserted in MPM list immediately after the DIMD candidate.

Experimental results

The proposed method has been implemented on top of ECM-19.0. Testing results of the proposed method under AI and RA configuration are summarized in Table 1 and 2. The experiments were performed following the common test conditions [1]. More detailed results can be found in the attached spreadsheets.

Table 1. experimental results under AI

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.09% | -0.02% |  |  |  |  |
| Class A2 | -0.01% | 0.01% | 0.05% |  |  |  |  |
| Class B | -0.02% | -0.20% | -0.23% |  |  |  |  |
| Class C | -0.01% | -0.05% | -0.05% |  |  |  |  |
| Class E | 0.00% | -0.03% | 0.06% |  |  |  |  |
| Overall | -0.01% | -0.07% | -0.04% |  |  |  |  |
| Class D | 0.01% | -0.24% | -0.18% |  |  |  |  |
| Class F | -0.04% | 0.19% | 0.21% |  |  |  |  |

Table 2. experimental results under RA

|  | randomaccess Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.04% | 0.33% | 0.04% |  |  |  |  |
| Class E | 0.02% | 0.11% | 0.08% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.04% | -0.11% | -0.14% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |

References

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017.

Patent rights declaration(s)

Hisense Visual Technology Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Shandong University do not have current or pending patent rights relating to the technology described in this contribution.