| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 42nd Meeting, Santa Eulària, ES, 24 April – 1 May 2026 | Document: JVET-AP0222-v2 |
| --- | --- |

| Title: | AHG18: Decoder side NN-based picture generation for lost picture substitution |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kirill Malyshev Maxim Sychev Sergey Ikonin Elena Alshina | Email: | sergey.ikonin@huawei.com elena.alshina@huawei.com |
| Source: | Huawei Technologies |  |  |

_____________________________

# Abstract

This document describes an experiment of applying NNFP functionality for generation of lost pictures unavailable due to network transmission issues in ultra-low latency scenario, and proposes modifications to the AHG18 software to enable this functionality based on existing NNPF SEI mechanism. The proposed implementation is controlled via the NNPF interface and utilizes the additional syntax proposed in JVET-AP0226. The generated pictures can be used both for output to display, and for lost reference frames substitution controlled by specific flag. Being tested according to ULL CTC JVET-AN2039, this picture generation technique allows to reduce video freezes significantly, and noticeable improves subjective quality for short interval freeze substitution. At the same time long distance extrapolation remains challenging. The contribution provides a starting point for further investigation and improvement in various aspects of missed picture generation functionality.

# Introduction

During CfE evaluation of proposal JVET-AN0080 “AHG18: CfE response in additional functionality on ultra-low latency and packet loss resilience” conducted at 40th JVET meeting it has been noted that frequent and long freeze events significantly affects subjective perception. Therefore, technology that can mitigate freeze artifacts and protect continuous decoding process even under really poor network channel would improve visual quality a lot. To address this aspect the experiments utilizing NNPF have been conducted.

# Use cases

Proposed modifications support two main use cases for generated pictures:

- Output generated picture instead of unavailable one (Figures 1 and 2)

- Using generated picture instead of unavailable reference (Figures 3 and 4)

Both use cases are supported for low-delay and random access with short GOP4 configurations evaluated in JVET-AP0200. Frame extrapolation model is used for low-delay configuration, frame interpolation model is used for random access configuration according to figures.

NN

Figure 1 Frame extrapolation for low-delay configuration

T2

T1

T0

NN

NN

Figure 2 Frame interpolation for generation lost frames inside a GOP in random access configuration

NN

Figure 3 Using extrapolated frame as reference for low-delay configuration

T2

T1

T0

NN

Figure 4 Using interpolated frame as reference for random access configuration

# Test results

Three configurations have been tested according to AhG18 ULL CTC JVET-AN2039for QP22 Wi-Fi try 0:

## LDB

|  | Broadcast w/o feedback, transmission priority off, try #0-WiFi, QP 22 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSNR vs Latency | VTM.NNFP over VTM |  |  |  |  |  |  |
| at QP 22 | Y | U | V | Freeze | Delta Freeze | EncT | DecT |
| Class A | -0.8 | -0.1 | -1.0 | -83.6% | 83.6 -> 0% | 147% | 3115% |
| Class B | 1.2 | 0.5 | 0.3 | -68.3% | 68.3 -> 0% | 155% | 2482% |
| Gaming | 1.4 | 1.4 | 0.5 | -47.2% | 47.2 -> 0% | 166% | 886% |
| Overall | 0.7 | 0.6 | 0.0 | -66.7% | 66.7 -> 0% | 156% | 1994% |

## LDB with GDR enabled

|  | Broadcast w/o feedback, transmission priority off, try #0-WiFi, QP 22 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSNR vs Latency | VTM.GDR.NNPF over VTM.GDR |  |  |  |  |  |  |
| at QP 22 | Y | U | V | Freeze | Delta Freeze | EncT | DecT |
| Class A | 0.3 | 0.3 | 0.2 | -51.7% | 51.7 -> 0% | 138% | 1257% |
| Class B | 0.5 | 0.2 | 0.2 | -44.2% | 44.2 -> 0% | 156% | 1121% |
| Gaming | 0.2 | 0.0 | -0.5 | -22.9% | 22.9 -> 0% | 154% | 441% |
| Overall | 0.4 | 0.2 | 0.0 | -40.4% | 40.4 -> 0% | 150% | 897% |

## Random Access GOP4

|  | Broadcast w/o feedback, transmission priority off, try #0-WiFi, QP 22 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSNR vs Latency | VTM.RAGOP4.NNPF over VTM.RAGOP4 |  |  |  |  |  |  |
| at QP 22 | Y | U | V | Freeze | Delta Freeze | EncT | DecT |
| Class A | 2.6 | 1.7 | 1.5 | -38.4% | 40.1 -> 1.7% | 77% | 128% |
| Class B | 2.9 | 1.7 | 2.2 | -27.6% | 27.6 -> 0% | 77% | 120% |
| Gaming | 1.4 | 1.1 | 1.0 | -17.2% | 17.2 -> 0% | 74% | 93% |
| Overall | 2.4 | 1.5 | 1.7 | -27.7% | 28.2 -> 0.5% | 76% | 114% |

## Random Access GOP4 (no reference replacement)

|  | Broadcast w/o feedback, transmission priority off, try #0-WiFi, QP 22 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PSNR vs Latency | VTM.RAGOP4.NNPF_NOREF over VTM.RAGOP4 |  |  |  |  |  |  |
| at QP 22 | Y | U | V | Freeze | Delta Freeze | EncT | DecT |
| Class A | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |  |
| Class B | 1.1 | 0.8 | 1.0 | -15.7% | 27.6 -> 12% | 75% | 86% |
| Gaming | 0.4 | 0.4 | 0.4 | -10.4% | 17.2 -> 6.8% | 73% | 78% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |  |

# Discussion

Results demonstrate meaningful freeze reduction for all tested configurations. Results for LDB is very dependent on content and pattern of losses. GDR configuration gives more stable results with picture extrapolation since it is more robust configuration in itself.

# Random Access shows the most substantial gain because picture interpolation model can generate pictures inside a GOP obtaining temporal information from past and future both. At the same time, there is noticeable improvement for PSNR and freezes both when missed reference replacement is used.

# Software modifications

The functionality of generating missed pictures for output is implemented according to NNPF syntax proposed in JVET-AP0226.

The extended functionality of using generated picture instead of unavailable reference has been additionally implemented and can be enabled by specific flag.

To provide the most fast and convenient evaluation process these models have been integrated as is using PyTorch implementations using floating-point checkpoints from corresponding GitHub [1] [2] .

# Conclusion

The proposed modifications enhance video continuity and quality by mitigating freezes caused by network issues through decoder-side neural network-generated substitute frames. The implementation supports both low-delay (frame extrapolation) and random-access (frame interpolation) configurations, ensuring compatibility with existing workflows while improving resilience against packet loss. It is proposed to include described software modification to AHG18 branch for experimenting on NN-based technologies for missed frame substitution.

# References

- L. Kong et al., "IFRNet: Intermediate Feature Refine Network for Efficient Frame Interpolation," 2022 IEEE/CVF Conference on Computer Vision and Pattern Recognition (CVPR), New Orleans, LA, USA, 2022, pp. 1959-1968, doi: 10.1109/CVPR52688.2022.00201. https://github.com/ltkong218/IFRNet

- S. Hirose et al., "Real-Time Video Prediction With Fast Video Interpolation Model and Prediction Training," 2024 IEEE International Conference on Image Processing (ICIP), Abu Dhabi, United Arab Emirates, 2024, pp. 2015-2021, doi: 10.1109/ICIP51287.2024.10647865. https://github.com/FykAikawa/IFRVP

# Patent rights declaration(s)

Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).