| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 42nd Meeting, Santa Eulària, ES, 24 April – 1 May 2026 | Document: JVET-APxxxx |
| --- | --- |

| Title: | MV-HEVC Conformance Bitstreams for Multiview Extended Profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Alexis M. Tourapis, D. Podborski, J. Kim | Tel: Email: | +1-408-228-7983 atourapis@apple.com |
| Source: | Apple Inc. |  |  |

# Abstract

This document provides a set of conformance bitstreams for Multiview Extended, Multiview Extended 10, and Multiview Monochrome profiles specified in Annex G and Annex H of Rec. ITU-T H.265 | ISO/IEC 23008-2. These bitstreams are intended for inclusion in the HEVC conformance bitstream set (document w16061). Ten bitstreams are provided, covering inter-view prediction, simulcast, auxiliary picture types (depth and unspecified), mixed chroma formats, mixed bit depths, mixed resolutions, and monochrome layers. The bitstream specifications follow the format of w16061 to facilitate direct integration by the editors.

# Introduction

The current set of MV-HEVC conformance bitstreams in w16061 [1] covers only the Multiview Main profile (bitstreams MVHEVCS-A through MVHEVCS-I). No conformance bitstreams exist for the Multiview Extended, Multiview Extended 10, or Multiview Monochrome profiles defined in Annex G of Rec. ITU-T H.265 | ISO/IEC 23008-2 [2].

This document provides ten conformance bitstreams that exercise features of these profiles, including inter-view prediction with 10-bit content, simulcast with mixed bit depths, three-view coding with asymmetric resolutions, auxiliary picture types (depth and unspecified), mixed chroma formats (4:2:0 with 4:0:0), and all-monochrome multilayer coding. The bitstreams were generated using the HM reference software with MV-HEVC multiview extensions [3].

The bitstream specifications in this document follow the format of w16061 Sections 6.6.14 and 6.7 to facilitate direct integration by the editors.

# Test bitstream specifications

The following subsections describe each conformance bitstream following the format of w16061 Section 6.6.14.

## Test bitstream #MVHEVCS_APPLE_A

Specification: All slices are coded as I, P or B slices. The first picture of each view is coded as an IDR picture. A Clean Random Access (CRA) picture occurs at the second intra period. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers of the non-base view is equal to 1. For each picture in the non-base view, inter-view prediction is enabled with both sample and motion prediction. The two views have the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

The number of layers is 2, the chroma format is 4:2:0, the bit depth is 10 bits, the resolution is 1280x720 and the number of frames is 33.

Inter-layer prediction: L1 depends on L0 (sample and motion prediction)

The output layer sets are specified as follows:

- OLS_0 -	layer: 0, PTL idx: 1 (Main 10, Level 4), output_layer_flag: 1

- OLS_1 -	layer: 0, PTL idx: 1 (Main 10, Level 4), output_layer_flag: 1

layer: 1, PTL idx: 2 (Multiview Extended 10, Level 4), output_layer_flag: 1

Functional stage: Decoding of two views with inter-view prediction (sample and motion), inter-prediction, and CRA refresh.

Purpose: To test two-view inter-view prediction with 10-bit content and CRA refresh type.

## Test bitstream #MVHEVCS_APPLE_B

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as an Instantaneous Decoding Refresh (IDR) picture. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers of the non-base view is equal to 0. The two views are coded independently as a simulcast with different bit depths (8 bits for layer 0 and 10 bits for layer 1). The two views have the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

The number of layers is 2, the chroma format is 4:2:0, the bit depth is 8 bits for layer 0 and 10 bits for layer 1, the resolution is 1280x720 and the number of frames is 9.

Inter-layer prediction: None (simulcast with different bit depths)

The output layer sets are specified as follows:

- OLS_0 -	layer: 0, PTL idx: 1 (Main, Level 4), output_layer_flag: 1

- OLS_1 -	layer: 0, PTL idx: 1 (Main, Level 4), output_layer_flag: 1

layer: 1, PTL idx: 2 (Main 10, Level 4), output_layer_flag: 1

Functional stage: Decoding of two views coded independently with different bit depths (simulcast).

Purpose: To test multiview simulcast with mixed bit depths.

## Test bitstream #MVHEVCS_APPLE_C

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as an Instantaneous Decoding Refresh (IDR) picture. Each picture contains only one slice. NumViews is equal to 3. NumDirectRefLayers of view 1 is equal to 1 (depends on view 0). NumDirectRefLayers of view 2 is equal to 0 (coded independently at a different resolution). For each picture in view 1, inter-view prediction is enabled with both sample and motion prediction from view 0. View 2 is coded independently at a lower resolution. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

The number of layers is 3, the chroma format is 4:2:0, the bit depth is 10 bits, the resolution is 1280x720 for layers 0 and 1 and 640x360 for layer 2, and the number of frames is 9.

Inter-layer prediction: L1 depends on L0 (sample and motion prediction); L2 independent (different resolution)

The output layer sets are specified as follows:

- OLS_0 -	layer: 0, PTL idx: 1 (Main 10, Level 4), output_layer_flag: 1

- OLS_1 -	layer: 0, PTL idx: 1 (Main 10, Level 4), output_layer_flag: 1

layer: 1, PTL idx: 2 (Multiview Extended 10, Level 4), output_layer_flag: 1

- OLS_2 -	layer: 0, PTL idx: 1 (Main 10, Level 4), output_layer_flag: 1

layer: 1, PTL idx: 2 (Multiview Extended 10, Level 4), output_layer_flag: 1

layer: 2, PTL idx: 3 (Multiview Extended 10, Level 4), output_layer_flag: 1

Functional stage: Decoding of three views with partial inter-view prediction and asymmetric resolutions.

Purpose: To test three-view coding with mixed resolutions and partial inter-layer prediction.

## Test bitstream #MVHEVCS_APPLE_D

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as an Instantaneous Decoding Refresh (IDR) picture. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers of the non-base view is equal to 1. For each picture in the non-base view, inter-view prediction is enabled with both sample and motion prediction. The two views have the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

The number of layers is 2, the chroma format is 4:2:0, the bit depth is 10 bits, the resolution is 1280x720 and the number of frames is 9.

Inter-layer prediction: L1 depends on L0 (sample and motion prediction)

The output layer sets are specified as follows:

- OLS_0 -	layer: 0, PTL idx: 1 (Main 10, Level 4), output_layer_flag: 1

- OLS_1 -	layer: 0, PTL idx: 1 (Main 10, Level 4), output_layer_flag: 1

layer: 1, PTL idx: 2 (Multiview Extended 10, Level 4), output_layer_flag: 1

Functional stage: Decoding of two views with inter-view prediction (sample and motion) and IDR refresh.

Purpose: To test two-view inter-view prediction with IDR refresh type.

Note: This bitstream uses IDR refresh only (9 frames). MVHEVCS_APPLE_A uses both IDR and CRA refresh (33 frames) for the same configuration.

## Test bitstream #MVHEVCS_APPLE_E

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an Instantaneous Decoding Refresh (IDR) picture. Each picture contains only one slice. The bitstream contains a base layer (layer 0) and an auxiliary depth layer (layer 1, AuxId = 2). The auxiliary layer is coded independently without inter-layer prediction. Both layers have the same spatial resolution and chroma format (4:2:0). All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

The number of layers is 2, the chroma format is 4:2:0, the bit depth is 8 bits, the resolution is 352x240 and the number of frames is 9.

Inter-layer prediction: None (auxiliary picture coded independently)

The output layer sets are specified as follows:

- OLS_0 -	layer: 0, PTL idx: 1 (Main, Level 4), output_layer_flag: 1

- OLS_1 -	layer: 0, PTL idx: 1 (Main, Level 4), output_layer_flag: 1

layer: 1 (auxiliary depth), PTL idx: 2 (Multiview Extended, Level 4), output_layer_flag: 1

Functional stage: Decoding of a base picture with an auxiliary depth picture.

Purpose: To test auxiliary depth picture signalling (AuxId = 2) with 4:2:0 chroma format.

## Test bitstream #MVHEVCS_APPLE_F

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an Instantaneous Decoding Refresh (IDR) picture. Each picture contains only one slice. The bitstream contains a base layer (layer 0) and an auxiliary layer with unspecified type (layer 1, AuxId = 129). The auxiliary layer is coded independently without inter-layer prediction. Both layers have the same spatial resolution and chroma format (4:2:0). All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

The number of layers is 2, the chroma format is 4:2:0, the bit depth is 8 bits, the resolution is 352x240 and the number of frames is 9.

Inter-layer prediction: None (auxiliary picture coded independently)

The output layer sets are specified as follows:

- OLS_0 -	layer: 0, PTL idx: 1 (Main, Level 4), output_layer_flag: 1

- OLS_1 -	layer: 0, PTL idx: 1 (Main, Level 4), output_layer_flag: 1

layer: 1 (auxiliary unspecified), PTL idx: 2 (Multiview Extended, Level 4), output_layer_flag: 1

Functional stage: Decoding of a base picture with an auxiliary picture of unspecified type.

Purpose: To test auxiliary picture signalling with an unspecified AuxId value (AuxId = 129) with 4:2:0 chroma format.

## Test bitstream #MVHEVCS_APPLE_G

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an Instantaneous Decoding Refresh (IDR) picture. Each picture contains only one slice. The bitstream contains a base layer (layer 0, 4:2:0) and a monochrome auxiliary depth layer (layer 1, 4:0:0, AuxId = 2). The auxiliary layer is coded independently without inter-layer prediction. Both layers have the same spatial resolution but different chroma formats. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

The number of layers is 2, the chroma format is 4:2:0 for layer 0 and 4:0:0 for layer 1, the bit depth is 8 bits, the resolution is 352x240 and the number of frames is 9.

Inter-layer prediction: None (different chroma formats, coded independently)

The output layer sets are specified as follows:

- OLS_0 -	layer: 0, PTL idx: 1 (Main, Level 4), output_layer_flag: 1

- OLS_1 -	layer: 0, PTL idx: 1 (Main, Level 4), output_layer_flag: 1

layer: 1 (auxiliary depth), PTL idx: 2 (Multiview Monochrome, Level 4), output_layer_flag: 1

Functional stage: Decoding of a 4:2:0 base picture with a monochrome auxiliary depth picture.

Purpose: To test mixed chroma format coding (4:2:0 base with 4:0:0 auxiliary depth).

## Test bitstream #MVHEVCS_APPLE_H

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an Instantaneous Decoding Refresh (IDR) picture. Each picture contains only one slice. The bitstream contains a base layer (layer 0, 4:2:0) and a monochrome auxiliary layer with unspecified type (layer 1, 4:0:0, AuxId = 129). The auxiliary layer is coded independently without inter-layer prediction. Both layers have the same spatial resolution but different chroma formats. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

The number of layers is 2, the chroma format is 4:2:0 for layer 0 and 4:0:0 for layer 1, the bit depth is 8 bits, the resolution is 352x240 and the number of frames is 9.

Inter-layer prediction: None (different chroma formats, coded independently)

The output layer sets are specified as follows:

- OLS_0 -	layer: 0, PTL idx: 1 (Main, Level 4), output_layer_flag: 1

- OLS_1 -	layer: 0, PTL idx: 1 (Main, Level 4), output_layer_flag: 1

layer: 1 (auxiliary unspecified), PTL idx: 2 (Multiview Monochrome, Level 4), output_layer_flag: 1

Functional stage: Decoding of a 4:2:0 base picture with a monochrome auxiliary picture of unspecified type.

Purpose: To test mixed chroma format coding (4:2:0 base with 4:0:0 auxiliary, unspecified AuxId = 129).

## Test bitstream #MVHEVCS_APPLE_I

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an Instantaneous Decoding Refresh (IDR) picture. Each picture contains only one slice. The bitstream contains a monochrome base layer (layer 0, 4:0:0) and a monochrome auxiliary depth layer (layer 1, 4:0:0, AuxId = 2). The auxiliary layer is coded independently without inter-layer prediction. Both layers have the same spatial resolution and chroma format. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

The number of layers is 2, the chroma format is 4:0:0 (monochrome), the bit depth is 8 bits, the resolution is 352x240 and the number of frames is 9.

Inter-layer prediction: None (auxiliary picture coded independently)

The output layer sets are specified as follows:

- OLS_0 -	layer: 0, PTL idx: 1 (Monochrome, Level 4), output_layer_flag: 1

- OLS_1 -	layer: 0, PTL idx: 1 (Monochrome, Level 4), output_layer_flag: 1

layer: 1 (auxiliary depth), PTL idx: 2 (Multiview Monochrome, Level 4), output_layer_flag: 1

Functional stage: Decoding of two monochrome layers (base and auxiliary depth).

Purpose: To test all-monochrome multilayer coding with auxiliary depth (AuxId = 2).

## Test bitstream #MVHEVCS_APPLE_J

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an Instantaneous Decoding Refresh (IDR) picture. Each picture contains only one slice. The bitstream contains a monochrome base layer (layer 0, 4:0:0) and a monochrome auxiliary layer with unspecified type (layer 1, 4:0:0, AuxId = 129). The auxiliary layer is coded independently without inter-layer prediction. Both layers have the same spatial resolution and chroma format. All NAL units are encapsulated into the byte stream format specified in Rec. ITU-T H.265 | ISO/IEC 23008-2.

The number of layers is 2, the chroma format is 4:0:0 (monochrome), the bit depth is 8 bits, the resolution is 352x240 and the number of frames is 9.

Inter-layer prediction: None (auxiliary picture coded independently)

The output layer sets are specified as follows:

- OLS_0 -	layer: 0, PTL idx: 1 (Monochrome, Level 4), output_layer_flag: 1

- OLS_1 -	layer: 0, PTL idx: 1 (Monochrome, Level 4), output_layer_flag: 1

layer: 1 (auxiliary unspecified), PTL idx: 2 (Multiview Monochrome, Level 4), output_layer_flag: 1

Functional stage: Decoding of two monochrome layers (base and auxiliary with unspecified type).

Purpose: To test all-monochrome multilayer coding with unspecified auxiliary type (AuxId = 129).

# Bitstream summary tables

The following tables summarize the conformance bitstreams grouped by profile, following the format of w16061 Section 6.7.

## Bitstreams for Multiview Extended 10 profile

| Bitstreams for Multiview Extended 10 profile |  |  |  | Base layer profile |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Categories | Subcategory | Bitstream | File name | Main | Main 10 | Main tier | Level | Frame rate (fps) |
| Prediction Structure (2-view) | Inter-view prediction with CRA | MVHEVCS_APPLE_A | MVHEVCS_APPLE_A |  | X | X | 4 and higher | 60 |
|  | Simulcast with mixed bit depth | MVHEVCS_APPLE_B | MVHEVCS_APPLE_B | X |  | X | 4 and higher | 60 |
| Prediction Structure (3-view) | Partial inter-view prediction with asymmetric resolutions | MVHEVCS_APPLE_C | MVHEVCS_APPLE_C |  | X | X | 4 and higher | 60 |
|  | Inter-view prediction with IDR refresh | MVHEVCS_APPLE_D | MVHEVCS_APPLE_D |  | X | X | 4 and higher | 60 |

## Bitstreams for Multiview Extended profile

| Bitstreams for Multiview Extended profile |  |  |  | Base layer profile |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Categories | Subcategory | Bitstream | File name | Main | Main tier | Level | Frame rate (fps) |
| Auxiliary Picture (4:2:0) | Auxiliary depth (AuxId = 2) | MVHEVCS_APPLE_E | MVHEVCS_APPLE_E | X | X | 4 and higher | 30 |
|  | Auxiliary unspecified (AuxId = 129) | MVHEVCS_APPLE_F | MVHEVCS_APPLE_F | X | X | 4 and higher | 30 |

## Bitstreams for Multiview Monochrome profile

| Bitstreams for Multiview Monochrome profile |  |  |  | Base layer profile |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Categories | Subcategory | Bitstream | File name | Main | Monochrome | Main tier | Level | Frame rate (fps) |
| Mixed Chroma (4:2:0 + 4:0:0) | Auxiliary depth (AuxId = 2) | MVHEVCS_APPLE_G | MVHEVCS_APPLE_G | X |  | X | 4 and higher | 30 |
|  | Auxiliary unspecified (AuxId = 129) | MVHEVCS_APPLE_H | MVHEVCS_APPLE_H | X |  | X | 4 and higher | 30 |
| All Monochrome (4:0:0 + 4:0:0) | Auxiliary depth (AuxId = 2) | MVHEVCS_APPLE_I | MVHEVCS_APPLE_I |  | X | X | 4 and higher | 30 |
|  | Auxiliary unspecified (AuxId = 129) | MVHEVCS_APPLE_J | MVHEVCS_APPLE_J |  | X | X | 4 and higher | 30 |

# Bitstream checksums

The following table provides MD5 checksums for the bitstreams and their decoded reconstructions.

| Bitstream | Size (bytes) | MD5(bitstream) | MD5(dec_L0) | MD5(dec_L1) | MD5(dec_L2) |
| --- | --- | --- | --- | --- | --- |
| MVHEVCS_APPLE_A | 149772 | 23ca987bbeea437a9325e0b383647b5a | e42d3e64a1a60466b828d1551dba855c | 72a378eef8ffb01e994e44249721720a |  |
| MVHEVCS_APPLE_B | 80451 | 7022934b7e04e6af166041691f90f1de | 974888f1fa8ed5f36cb0e36c9ab4d2f0 | e385ca9a7d68c84f83791903f2846e5b |  |
| MVHEVCS_APPLE_C | 63004 | c8c3fdf7d723dd24029a402052aec4a6 | 10c75db866beaf5df3d82adef4cdbcf9 | 7bdc6d430f7d2fc01c6641c720427fdf | 0b0bd206d0449febecd9f9804384e1bb |
| MVHEVCS_APPLE_D | 52509 | 0f1e37207d0a8e5fa15d16cd582a940e | 10c75db866beaf5df3d82adef4cdbcf9 | 7bdc6d430f7d2fc01c6641c720427fdf |  |
| MVHEVCS_APPLE_E | 9320 | 2bbac7620c22239ffb62126509d30aca | a89159dc2ec9eac02e0e614b5c723bc5 | b3a22ecbbd49c3d7f3921290bc8be921 |  |
| MVHEVCS_APPLE_F | 9322 | 26f936ba546bcf2b6c9ea80932f3655d | a89159dc2ec9eac02e0e614b5c723bc5 | b3a22ecbbd49c3d7f3921290bc8be921 |  |
| MVHEVCS_APPLE_G | 9123 | 28360baf1ec930c1670e0b131c531394 | a89159dc2ec9eac02e0e614b5c723bc5 | daeeb5ed2a2eb7375876437ca35a3218 |  |
| MVHEVCS_APPLE_H | 9124 | b945d13641cbcd3cc8629462bb782c20 | a89159dc2ec9eac02e0e614b5c723bc5 | daeeb5ed2a2eb7375876437ca35a3218 |  |
| MVHEVCS_APPLE_I | 8980 | ff58e22b85d22670723ae7f1131d35df | d7ce1ce349f02fe710b1af4eaa73daf9 | daeeb5ed2a2eb7375876437ca35a3218 |  |
| MVHEVCS_APPLE_J | 8981 | 80f6c3abdbdaff27bad2a6285ee43778 | d7ce1ce349f02fe710b1af4eaa73daf9 | daeeb5ed2a2eb7375876437ca35a3218 |  |

# Reference software

All bitstreams were generated using the HM reference software with MV-HEVC multiview extensions, available at https://vcgit.hhi.fraunhofer.de/jvet/HM/-/tree/dev-multiview444. The software was built in Release mode on macOS (ARM64) with CMake. Bitstreams were verified by decoding with the same software and checking MD5 checksums of the reconstructed output.

# References

[1]	w16061, "Conformance testing for Rec. ITU-T H.265 | ISO/IEC 23008-2 High efficiency video coding".

[2]	Rec. ITU-T H.265 (V11) (01/2026) | ISO/IEC 23008-2, "High efficiency video coding".

[3]	https://vcgit.hhi.fraunhofer.de/jvet/HM/-/tree/dev-multiview444

# Patent rights declaration(s)

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, may be prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard.