| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 42nd Meeting, Santa Eulària, ES, 24 April – 1 May 2026 | Document: JVET-AP0282-v1 |
| --- | --- |

| Title: | Cross-check of JVET-AP0234 (AHG11: dynamic quantization and hardware optimization for NNIP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | T. Dumas F. Galpin | Email: | thierry.dumas@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports the results of the cross-check of JVET-AP0234. In NNVC-16.1 and ECM-19.1, in the Neural Network-based Intra Prediction (NNIP) implementation, layer activations use 16-bit signed integer, matrices of weights use 16-bit signed integer, and accumulators inside vector-matrix multiplications use 32-bit signed integer. JVET-AP0234 suggests to reduce the activation bit-width to 10-bits and the weight bit-width to 8 bits in order to avoid implementation-dependent behavior due to overflow and reduce hardware area in NNIP implementation.

The following mean BD-rate (Y/Cb/Cr) are reported.

- JVET-AP0234 on top of NNVC-16.1

  - ANCHOR = NNVC-16.1 (Commit 78ec06f1) with “--NnlfOption=0” and “--NnIntraPred=1”, i.e. without LOP7 an with the NN-based intra prediction mode in 16-bit signed integer.

  - TEST = JVET-AP0234 on top of NNVC-16.1 with “--NnlfOption=0” and “--NnIntraPred=1”, i.e. without LOP7 an with the NN-based intra prediction mode in 16-bit signed integer.

    - RA: 0.01%   -0.12%   -0.10%   |   EncT 100% and DecT 96%

    - AI: 0.02%   0.04%   0.01%   |   EncT 100% and DecT 90%.

- JVET-AP0234 on top of ECM-19.1

  - ANCHOR = ECM-19.1 (Commit 20811dbb).

  - TEST = JVET-AP0234 on top of ECM-19.1 with “HARDMWARE_IMP_DEMO” at 0, i.e. not using the hardware implementation showcase.

    - RA: …%   …%   …%   |   EncT …% and DecT …%

    - AI: …%   …%   …%   |   EncT …% and DecT …%

For both JVET-AP0234 on top of NNVC-16.1 and JVET-AP0234 on top of ECM-19.1, the cross-checked BD-rates match those reported by the proponents of JVET-AP0234.

# Introduction

In NNVC-16.1 and ECM-19.1, in the Neural Network-based Intra Prediction (NNIP) implementation, layer activations use 16-bit signed integer, matrices of weights use 16-bit signed integer, and accumulators inside vector-matrix multiplications use 32-bit signed integer. In this design, JVET-AP0234 [1] identifies two major issues. Firstly, the accumulators inside vector-matrix multiplications can overflow, potentially causing indeterministic behavior across different processing units. Secondly, due to the large bit-depth involved in the multiplications, hardware area required to implement NNIP is estimated to be very large.

To address these two issues, JVET-AP0234 proposes

- Dynamically reducing activation precision

A set of activations, corresponding to an input to a neural network layer, is processed in fixed-size chunks of 8 coefficients. For each chunk, the maximum absolute value is detected using a bit-scan operation. Then, a chunk‑specific right shift is determined from this maximum absolute value and applied this chunk so that all the coefficients within this chunk fit within 10-bit range.

- Dynamically reducing weight precision

For a given matrix of weights inside a neural network in NNIP, weights are grouped into fixed‑size chunks of 8 coefficients, matching SIMD widths and hardware execution units. For each chunk, weights are quantized to 8-bit signed integer using adaptive scaling. Thanks to this, all multiplications within a vector-matrix multiplication can be performed using 8-bit weights. In a vector-matrix multiplication, during the accumulation, the shifts applied to activation groups of 8 coefficients are reverted to preserve the relative scaling between the activation samples.

Moreover, inside a neural network in NNIP, for a given series of a vector-matrix multiplication and LeakyReLU, to ease the quantization of the ouput to 10-bit, JVET-AP0234 proposes to fuse the vector-matrix multiplication and LeakyReLU.

# Results

The source code of JVET-AP0234 sent by the proponents of JVET-AP0234 is used to carry out the cross-check of JVET-AP0234. For JVET-AP0234 on top of NNVC-16.1, the cross-check follows the AHG11 CTC [2]. For JVET-AP0234 on top of ECM-19.1, the cross-check follows the AHG12 CTC [3].

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 16.1 with "--NnlfOption=0" and "--NnIntraPred=1" |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | 0.02% | 0.02% | -0.05% | 0.01% | 0.00% | -0.08% | 99% | 88% | 100% |
| Class A2 | 0.03% | 0.08% | 0.04% | 0.03% | 0.14% | 0.04% | 99% | 89% | 100% |
| Class B | 0.02% | 0.06% | 0.14% | 0.04% | 0.07% | 0.21% | 100% | 91% | 100% |
| Class C | 0.02% | 0.10% | -0.09% | -0.02% | 0.21% | -0.19% | 100% | 92% | 100% |
| Class E | -0.02% | -0.08% | -0.04% | -0.06% | -0.13% | -0.04% | 101% | 91% | 100% |
| Overall | 0.02% | 0.04% | 0.01% | 0.00% | 0.07% | 0.00% | 100% | 90% | 100% |
| Class D | 0.00% | -0.13% | 0.07% | -0.02% | -0.27% | -0.14% | 100% | 94% | 100% |
| Class F | -0.01% | 0.09% | 0.02% | 0.03% | 0.01% | 0.25% | 101% | 95% | 100% |

Figure 1: JVET-AP0234 on top of NNVC-16.1 versus NNVC-16.1 in AI. Both ANCHOR and TEST use “--NnlfOption=0” and “--NnIntraPred=1”.

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 16.1 with "--NnlfOption=0" and "--NnIntraPred=1" |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | 0.02% | 0.00% | 0.07% | 0.03% | -0.09% | 0.07% | 100% | 95% | 100% |
| Class A2 | 0.02% | 0.01% | -0.13% | 0.04% | -0.02% | -0.09% | 100% | 97% | 100% |
| Class B | 0.00% | -0.17% | -0.22% | -0.03% | -0.30% | -0.20% | 100% | 96% | 100% |
| Class C | 0.01% | -0.24% | -0.04% | 0.07% | -0.64% | 0.06% | 100% | 96% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% | -0.12% | -0.10% | 0.02% | -0.30% | -0.06% | 100% | 96% | 100% |
| Class D | -0.03% | 0.26% | 0.33% | -0.07% | 0.09% | 0.40% | 100% | 95% | 100% |
| Class F | 0.01% | 0.03% | 0.16% | 0.02% | -0.03% | 0.04% | 100% | 101% | 100% |

Figure 2: JVET-AP0234 on top of NNVC-16.1 versus NNVC-16.1 in RA. Both ANCHOR and TEST use “--NnlfOption=0” and “--NnIntraPred=1”.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-19.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.00% | 0.02% | -0.01% | 100.9% | 101.8% | 100.8% | 102.0% |
| Class E | 0.02% | 0.15% | 0.04% | 101.1% | 101.3% | 100.7% | 101.9% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.01% | -0.11% | -0.05% | 100.2% | 101.7% | 101.2% | 102.2% |
| Class F |  |  |  |  |  |  |  |

Figure 3: JVET-AP0234 on top of ECM-19.1 with “HARDWARE_IMP_DEMO” at 0, c.f. [1], versus ECM-19.1 in AI.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-19.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.02% | 0.40% | 0.10% | 99.7% | 100.5% | 100.7% | 101.7% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.01% | -0.07% | 0.03% | 99.7% | 100.2% | 100.9% | 102.0% |
| Class F | 0.00% | -0.05% | 0.22% | 100.1% | 98.0% | 100.5% | 101.0% |

Figure 4: JVET-AP0234 on top of ECM-19.1 with “HARDWARE_IMP_DEMO” at 0 versus ECM-19.1 in RA.

# Conclusion

For both JVET-AP0234 on top of NNVC-16.1 and JVET-AP0234 on top of ECM-19.1, the cross-checked BD-rates match those reported in [1]. Therefore, the cross-check is viewed as successful.

# References

- A. Akhtar, S. Esenlik, Y. Matsuba, M. Karczewic. “AHG11: dynamic quantization and hardware optimization for NNIP”. JVET-AP0234, 42nd JVET Meeting, Santa Eulària, ES, 24 April – 1 May 2026.

- E. Alshina, R.-L. Liao, S. Liu, and A. Segall. “Common Test Conditions and evaluation procedures for neural network-based video coding technology”. JVET-AF2016, 32nd JVET Meeting, Hannover, DE, 13–20 October 2023.

- M. Karczewicz, Y. Ye. “Common test conditions and evaluation procedures for enhanced compression tool testing”. JVET-AI2017, 35th JVET meeting, Sapporo, JP, 12-19 July 2024.

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).