| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JVET-F0096 |
| --- | --- |

| Title: | EE2-JVET related: Division-free bilateral filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jacob Ström, Kenneth Andersson, Per Wennersten, Martin Pettersson, Jack Enhorn and Rickard Sjöberg Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46720672192 {jacob.strom, kenneth.r.andersson, per.wennersten, martin.m.pettersson, jack.enhorn, rickard.sjoberg} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

During the Hobart meeting, it was requested to have a division-free implementation of the bilateral filter from JVET-F0034 test 2 demonstrated. It was also concluded that the smaller division-lookup-table (division-LUT) of about 1 kB was preferred. This contribution is a response to these requests. To keep the size of the division-LUT small, it is important to reduce the size of both the nominator and the denominator of the division. To reduce the nominator, the filter equation can be rephrased in a mathematically equivalent manner to work with differences instead of absolute values. To reduce the denominator, bilateral filtering is turned off for inter blocks of size 16×16 and larger, which avoids a large center weight coefficient. With these changes, this contribution asserts that it is possible to make a division-free implementation where the division-LUT has a size of 576 bytes. The result by using the division-LUT is identical to the result from using divisions. The reported BD rate numbers are -0.4% / -0.5% / -0.5% / -0.5% for AI / RA / LD / LDP.

# Introduction

This contribution investigates how to remove the division and replace it with a multiplication and a lookup-table (LUT). To keep the size of the LUT as small as possible, the nominator and the denominator of the division should be as small as possible. To reduce the nominator, the filtering equation is rewritten using differences. To reduce the denominator, the large center weight value for inter blocks of size 16×16 and larger is avoided by turning the filter off for these blocks. We will now go through these two steps in detail. After that, we will go through how to do rounding and tie-breaking in a way that matches the regular method.

# Reducing the nominator

The goal of the LUT is to pre-calculate the weights of the bilateral filter

ωi,j,k,l=e-i-k2+j-l22σd2-Ii,j-Ik,l22σr2,         (Eq. 1)

so that the filtered pixel IFi,j can be calculated as

IFi,j=k,lIk,l*ωi,j,k,lk,lωi,j,k,l.                             (Eq. 2)

As is explained in JVET-F0034, due to the fact that the filter only touches the center pixel and its 4-neighbors, this equation can be written as

IF=ICωC+ILωL+IRωR+IAωA+IBωBωC+ωL+ ωR+ ωA+ωB,                (Eq. 3)

where IC is the intensity of the center pixel, and IL, IR, IA and IB are the intensities for the left, right, above and below pixel respectively. Likewise, ωC is the weight for the center pixel, and ωL, ωR, ωA and ωB are the corresponding weights for the neighboring pixels.

The source code [1] in JVET-F0034 was based on Equation 3. In this case, the nominator can become relatively big. As an example, if all the intensities are the same, the non-center weights will have their maximum value of 31. The center weight can be as large as 4079. Also, if IC=IL=IR=IA=IB=1023, then the nominator will be equal to 1023*4079+1023*31+1023*31+1023*31+1023*31 = 4 299 669.

Fortunately, it is possible to rewrite Equation 3 as

IF=IC+ωLΔIL+ωRΔIR+ωAΔIA+ωBΔIBωC+ωL+ ωR+ ωA+ωB,                (Eq. 4)

where ΔIL=IL-IC and ΔIR=IR-IC, etc. To see the details, consult Appendix A. The good thing is that when we have a large difference in intensity ΔI, the bilateral filter will choose a small weight ω. This means that the product ΔI*ω will always be small, since either ΔI is small, or otherwise ω is small. In the implementation of JVET-F0034 test 2, the product ω*ΔI≤1300. Thus the nominator is reduced to 1300*4=5200.

# Reducing the denominator

The denominator in Equation 4 is equal to ωC+ωL+ ωR+ ωA+ωB. The maximum value for the non-center weights ωL, ωR, ωA and ωB is 31 in test 2 of JVET-F0034. As described in Appendix A in JVET-F0034, the center weight can take the following values:

|  | min(TUwidth, TUheight) |  |  |
| --- | --- | --- | --- |
|  | 4 | 8 | 16 |
| intra | 65 | 81 | 196 |
| inter | 113 | 196 | 4079 |

As can be seen in the table, the center weight value for inter when the block size is 16×16 or larger is much bigger than the others. Note here that a large center weight value means weak filtering. Thus, the inter 16×16 filtering is by far the weakest filtering. We have therefore simply disabled bilateral filtering in this case, since it anyway is not affecting the output much.

Removing this case means that the largest ωC is reduced to 196 and the largest possible denominator to 196 + 31*4 = 320.

# Rounding and bit-exact tie-breaking

When calculating the filtered pixel, in order to get correct rounding, one must add half of the denominator to the nominator according to

IF=roundICωC+ILωL+IRωR+IAωA+IBωBωC+ωL+ ωR+ ωA+ωB=floorICωC+ILωL+IRωR+IAωA+IBωB+(ωC+ωL+ ωR+ ωA+ωB)/2ωC+ωL+ ωR+ ωA+ωB.                (Eq. 5)

This is due to the fact that integer division div b will return floor(a/b) rather than round(a/b). In Equation 5, the nominator is always positive, which means that we should always add (ωC+ωL+ ωR+ ωA+ωB)/2 to the nominator. However, when we use the differential form in Equation 4 (repeated here for convenience)

IF=IC+ωLΔIL+ωRΔIR+ωAΔIA+ωBΔIBωC+ωL+ ωR+ ωA+ωB=IC+ab,                (Eq. 6)

the nominator a will sometimes be positive, and sometimes negative. In the negative case, we have to instead subtract half of the denominator, i.e., b/2, to get the rounding correct. If we let s be the sign of the nominator

s=signa=signωLΔIL+ωRΔIR+ωAΔIA+ωBΔIB           (Eq. 7)

we can get the rounded value as

IF=roundIC+ab=IC+s*floors*a+b≫1b.               (Eq. 8)

The above formula will yield a correctly rounded number; since s*a is always positive, it is correct to always add b≫1. However, even though it will always yield a correctly rounded number, it will not always yield the same rounding as if Equation 5 is used. This is due to the fact that a number such as -1.5 can be correctly rounded both to -1 and to -2. To get the same output as in Equation 5, instead of adding (b≫1) to the nominator, we add (b+m≫1), where

m= -1 if a<00 otherwise               (Eq. 9)

This makes the rounding tie-breaking identical with that of Equation 5 and JVET-F0034.

In summary, in JVET-F0034 we calculate

IF=a+b≫1div b               (Eq. 10)

where div means integer division and

a= ICωC+ILωL+IRωR+IAωA+IBωB       and                (Eq. 11)
b = ωC+ωL+ ωR+ ωA+ωB.

Instead in this contribution, we calculate the filtered value as

IF=IC+s*s*a+b+m≫1div b,                (Eq. 12)

where

a= ωLΔIL+ωRΔIR+ωAΔIA+ωBΔIB       and                (Eq. 13)
b = ωC+ωL+ ωR+ ωA+ωB.

which gives exactly the same result as in JVET-F0034 (Equation 9) including bit-exact tie-breaking during rounding. Note also that although Equation 12 includes two multiplications, they are really only conditional negations, which is inexpensive in hardware.

# Division using multiplication

## Implementation

From Equation 12, the next step is now to convert the division to a multiplication. If we call the nominator t and the denominator n, we want to calculate t div n without a division instruction.

As seen in Equations 12 and 13, the maximum value of n is maxωC+ωL+ ωR+ ωA+ωB=196+4*31=320, and the maximum value of t is a+b2=1300*4+3202=5360. Hence, if we can calculate t div n exactly for every combination of t∈[0,5360] and n∈[1,320] we are fine.

This is done using two LUTs. The first LUT is called divOneOverN and stores 214+kdiv n where k is selected so that using k+1 would make the value larger than 214. For some values, 1 is added. The second LUT is called divShift and stores the value of k.

Division t div n is now calculated using t*divOneOverNn≫(14+divShiftn). The multiplication needs 13 bits for the first factor (since t<5360<8191=213-1) and 15 bits for the second factor (since it must be able to hold the value 214, which requires 15 bits).

## Table size

The first table divOneOverN must be able to hold values up to and including 214, which requires 15 bits.

The second table divShift must be able to hold values up to 11, which requires 4 bits.

In total, (15+4)*320/8=760 bytes of data need to be stored.

Note, that since the denominator equals ωC+ωL+ ωR+ ωA+ωB, and since we know that ωC≥65, we only have to tabulate from 65 to 320. This means that it is sufficient to store 15+4*(320-65+1)/8=608 bytes.

Since the divOneOverN only has one value that needs 15 bits, namely the value 2^14, it is possible to instead use 0 to encode this value since divOneOverN does not contain zero. This is simple to do in hardware (b14 = !OR(b0, b1, ..., b13)) and would allow the table to instead become (14+4)*(320-65+1)/8=576 bytes.

To compile the code, use the code base [1] and apply the patch attached to the contribution.

# Objective metrics

Since the filtering has been removed for inter blocks of size 16×16 and larger, the objective results have changed slightly.  The BD rate figures are -0.4%, -0.5%, -0,5%, -0,5% for AI/RA/LDB/LDP.

All intra:

|  | Over HM-16.6-JEM-5.0.1 (parallel, gcc4.5.1) with Precise PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,18% | 0,23% | 0,10% | 106% | 105% |
| Class A2 | -0,35% | 0,04% | 0,10% | 108% | 106% |
| Class B | -0,18% | 0,24% | 0,23% | 103% | 103% |
| Class C | -0,79% | 0,09% | 0,11% | 104% | 102% |
| Class D | -0,53% | 0,23% | 0,21% | 107% | 104% |
| Class E | -0,54% | 0,05% | 0,05% | 106% | 106% |
| Overall | -0,41% | 0,16% | 0,14% | 105% | 104% |
| Class F (optional) | -1,78% | -0,11% | -0,35% | 99% | 100% |

Random access:

|  | Over HM-16.6-JEM-5.0.1 (parallel, gcc4.5.1) with Precise PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,24% | -0,30% | -0,10% | 103% | 96% |
| Class A2 | -0,66% | -0,52% | -0,33% | 102% | 95% |
| Class B | -0,44% | -0,05% | 0,07% | 102% | 97% |
| Class C | -0,55% | -0,05% | -0,29% | 99% | 95% |
| Class D | -0,53% | -0,35% | -0,41% | 106% | 101% |
| Overall (Ref) | -0,48% | -0,24% | -0,20% | 102% | 97% |
| Class F (optional) | -1,21% | -0,25% | -0,08% | 101% | 100% |

Low delay B:

|  | Over HM-16.6-JEM-5.0.1 (parallel, gcc4.5.1) with Precise PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class B | -0,30% | 0,47% | 0,09% | 100% | 97% |
| Class C | -0,53% | 0,07% | -0,14% | 102% | 101% |
| Class D | -0,64% | 0,66% | 0,28% | 102% | 100% |
| Class E | -0,50% | 0,71% | 0,17% | 99% | 97% |
| Overall (Ref) | -0,48% | 0,46% | 0,09% | 101% | 99% |
| Class F (optional) | -1,54% | -0,05% | -0,04% | 100% | 99% |

Low delay P:

|  | Over HM-16.6-JEM-5.0.1 (parallel, gcc4.5.1) with Precise PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class B | -0,42% | 0,56% | 0,94% | 107% | 102% |
| Class C | -0,53% | 0,03% | -0,11% | 109% | 108% |
| Class D | -0,64% | 0,94% | 0,53% | 102% | 98% |
| Class E | -0,56% | 0,35% | 0,93% | 100% | 97% |
| Overall (Ref) | -0,53% | 0,48% | 0,57% | 105% | 101% |
| Class F (optional) | -1,43% | 0,05% | -0,50% | 100% | 98% |

## Test 2

As a reference, we have included the BD rate figures for test 2 of JVET-F0034. They are -0.4%, -0.5%, -0,5%, -0,6% for AI/RA/LDB/LDP. To indicate that these are the old figures, they are written with a gray font.

All intra:

|  | Over HM-16.6-JEM-5.0.1 (parallel, gcc4.5.1) with Precise PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,18% | 0,23% | 0,10% | 105% | 104% |
| Class A2 | -0,35% | 0,04% | 0,10% | 108% | 105% |
| Class B | -0,18% | 0,24% | 0,23% | 107% | 105% |
| Class C | -0,79% | 0,09% | 0,11% | 108% | 106% |
| Class D | -0,53% | 0,23% | 0,21% | 101% | 95% |
| Class E | -0,54% | 0,05% | 0,05% | 106% | 106% |
| Overall | -0,41% | 0,16% | 0,14% | 106% | 103% |
| Class F (optional) | -1,78% | -0,11% | -0,35% | 108% | 108% |

Random access:

|  | Over HM-16.6-JEM-5.0.1 (parallel, gcc4.5.1) with Precise PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,23% | -0,34% | -0,08% | 103% | 97% |
| Class A2 | -0,67% | -0,54% | -0,33% | 102% | 97% |
| Class B | -0,43% | -0,02% | 0,09% | 102% | 99% |
| Class C | -0,59% | 0,06% | -0,24% | 100% | 99% |
| Class D | -0,52% | -0,32% | -0,44% | 104% | 101% |
| Overall (Ref) | -0,48% | -0,22% | -0,19% | 102% | 99% |
| Class F (optional) | -1,22% | -0,25% | -0,08% | 102% | 100% |

Low delay B:

|  | Over HM-16.6-JEM-5.0.1 (parallel, gcc4.5.1) with Precise PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class B | -0,35% | 0,41% | 0,06% | 102% | 101% |
| Class C | -0,55% | 0,34% | 0,02% | 102% | 101% |
| Class D | -0,66% | 0,30% | -0,20% | 100% | 98% |
| Class E | -0,64% | 0,02% | 1,24% | 103% | 101% |
| Overall (Ref) | -0,53% | 0,29% | 0,21% | 102% | 100% |
| Class F (optional) | -1,48% | 0,23% | 0,28% | 101% | 100% |

Low delay P:

|  | Over HM-16.6-JEM-5.0.1 (parallel, gcc4.5.1) with Precise PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class B | -0,44% | 0,51% | 0,44% | 108% | 102% |
| Class C | -0,57% | 0,25% | -0,05% | 106% | 105% |
| Class D | -0,64% | 0,56% | 0,44% | 99% | 97% |
| Class E | -0,70% | 0,37% | 0,68% | 103% | 101% |
| Overall (Ref) | -0,57% | 0,43% | 0,36% | 104% | 101% |
| Class F (optional) | -1,47% | -0,19% | -0,33% | 106% | 104% |

# Summary

In conclusion we have presented a solution where the integer division in JVET F0034 has been replaced by a multiplication and a LUT of size 576 bytes.

# References

[1] Code base: https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/candidates/HM-16.6-JEM-5.0-EE2-E0032

# Appendix A

In this appendix we go through why it is mathematically equivalent to rewrite the filtering formula (Equation 3) in differential form.

Equation 3 gives us

IF=ωCIC+ωLIL+ωRIR+ωAIA+ωBIBωC+ωL+ ωR+ ωA+ωB.                (Eq. A1)

We can now add and subtract ωLIC+ ωRIC+ ωAIC+ ωBIC to the nominator:

IF=ωCIC+(ωLIC+ ωRIC+ ωAIC+ ωBIC)+ωLIL+ωRIR+ωAIA+ωBIB-(ωLIC+ ωRIC+ ωAIC+ ωBIC)ωC+ωL+ ωR+ ωA+ωB

which is equal to

IF=ωC+ωL+ ωR+ ωA+ ωBIC+ωL(IL-IC)+ωR(IR-IC) +ωA(IA-IC)+ωB(IB-IC)ωC+ωL+ ωR+ ωA+ωB

which is furthermore equal to

IF=IC+ωL(IL-IC)+ωR(IR-IC) +ωA(IA-IC)+ωB(IB-IC)ωC+ωL+ ωR+ ωA+ωB        (Eq. A4)

or

IF=IC+ωLΔIL+ωRΔIR+ωAΔIA+ωBΔIBωC+ωL+ ωR+ ωA+ωB.         (Eq. A5)

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).