| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0044-v1 |
| --- | --- |

| Title: | AHG15: Proposed interoperability point syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce, Zhipin Deng, Sam Wong, Lidong Xu | Email: | jill.boyce@intel.com zhipin.deng@intel.com samuel.wong@intel.com lidong.xu@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

A strawman design for signaling constraints in a profile_tier_level( ) syntax structure in the SPS is proposed to enable describing decoder capabilities needed to decode a bitstream. It is based upon the VTM2 design in JVET-K1001-v5. The proposed design draws from the concepts proposed in JVET-K0312, and is intended as a starting point for discussion, rather than as a fully finalized solution.

# Summary of proposal

This contribution provides a strawman design for VTM2 to address the AHG15 mandates, as copied below:

- Study syntax alternatives for interoperability point signalling

- Study selection of constraint flags to be included in the VTM and their impact on syntax, semantics, and decoding process

This proposed design is based upon the VTM2 design in JVET-K1001-v5. It proposes constraint flags in a profile_tier_level( ) syntax structure in the SPS to enable describing decoder capabilities needed to decode a bitstream. An important aspect of the selection criteria used to decide if a constraint flag is recommended to be defined for a tool is how burdensome it would be for decoder implementation and validation to support fallback alternative behavior when the tool was not used, in addition to supporting the tool itself.

In addition to the SPS profile_tier_level( ) constraint flags, enable flags are proposed for many of the tools, which may be located in a sequence or picture parameter set, or slice header. Constraint flags are included in the profile_tier_level( ) syntax structure, and indicate if a decoder capability needed to decode a bitstream. When a constraint flag is set in the profile_tier_level( ) syntax structure, a decoder can safely assume that the tool will not be used in the bitstream. Tool enable flags are used to indicate if a tool is actually in use for a particular coded sequence, picture, or slice, depending on where the flag is located. Even if the tool enable flag is located in the SPS, there could be utility in including such flags in addition to the constraint flags in the profile_tier_level( ) section of the SPS, because the profile_tier_level( ) values may persist longer than the remainder of the SPS, e.g. for the entire bitstream rather than just the individual coded video sequence. The presence of tool enable flags can be conditioned on the value of the associated constraint flag, unless there is a desire to avoid a parsing dependency, e.g. PPS dependent on the SPS.

The table below includes a list of tools, largely taken from AHG13’s JVET-K1005, with a recommendation of constraint and enable flags. The table attempts to give an indication of if there is an impact on the decoder when the tool is not enabled, in terms of both decoding process and entropy coding. Because the full specification of the WD is not yet available, it is difficult to make this determination. Additionally, there is no clear definition of what constitutes a change to decoding process and/or entropy coding, so the values in the table should only be considered to be a starting point for further discussion and more careful study.

Additionally, it is proposed to include syntax elements in the profile_tier_level( ) for max_bitdepth_constraint_idc, u(4) and max_chroma_format_constraint_idc, u(2).

| Tool Name | Abbrev Name | Doc ref | SPS profile_tier_level( ) constraint flag | SPS or picture or slice enable flag | Decoding impact when not enabled |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fallback behavior | Entropy coding change |
| Partition restriction | PR | JVET-K0351 | N | N |  |  |
| Frame boundary partition | FBP | JVET-K0554 | N | N |  |  |
| Split restriction TT when size is larger than 64 | TT64 | JVET-K0230, JVET-K0556 | N | N |  |  |
| Implicit split QT 128x128 to 64x64 in I slice | QT128 | JVET-K0230, JVET-K0556 | N | N |  |  |
| Chroma separate tree | CST | JVET-K0230, JVET-K0556 | Y no_chroma_separate_tree_constraint_flag | Y, SPS chroma_separate_tree_enabled_flag | Y | Y |
| Adaptive loop filter | ALF | JVET-K0371 | Y no_adaptive_loop_filter_constraint_flag | Y, pic or slice adaptive_loop_filter_enabled_flag | N | Y |
| Sample adaptive offset | SAO |  | Y no_sample_adaptive_offest_constraint_flag | Y, pic or slice sample_adaptive_offset_enabled_flag | N | Y |
| Deblocking on 64 pixels sample TU | DB64 | JVET-K0307, JVET-K0237, JVET-K0369, JVET-K0232, JVET-K0315 | N | N |  |  |
| Deblocking on 8x8 grid | DB8x8 |  | N | N |  |  |
| QP upper bound increase** | MaxQP | JVET-K0251 | N | N |  |  |
| Dependent quantization | DQ | JVET-K0072 | Y no_dep_quant_constraint_flag | Y, pic or slice dep_quant_enabled_flag | Y | Y |
| Sign data hiding* | SDH | JVET-K0310 | Y no_sign_data_hiding_constraint_flag | Y, pic or slice sign_data_hiding_enabled_flag | Y | Y |
| Cross-component linear model | CCLM | JVET-K0190 | Y no_cclm_constraint_flag | Y, SPS cclm_enabled_flag | N | Y |
| 67 intra prediction mode +3 MPM intra mode coding | 67IPM | JVET-K0529, JVET-K0368 | Y no_extra_intra_modes_constraint_flag | Y, SPS extra_intra_modes_enabled_flag | N | Y |
| Wide angle intra prediction | WIP | JVET-K0500 | Y no_wide_intra_angles_constraint_flag | Y, SPS wide_intra_angles_enabled_flag | Y | N |
| Position dependent prediction combination | PDPC | JVET-K0063 | N | N |  |  |
| DC average computation | DC | JVET-K0122 | N | N |  |  |
| Adaptive multi-core transform | AMT | JVET-K0171, JVET-K0173, JVET-K0096 | Y no_mts_intra_constraint_flag no_mts_inter_constraint_flag | Y, SPS sps_mts_intra_enabled_flag sps_mts_inter_enabled_flag | N | Y |
| Affine motion model | AFF | JVET-K0184, JVET-K0337 | Y no_affine_motion_constraint_flag | Y , SPS affine_motion_enabled_flag | N | Y |
| High precision MVs |  |  | Y no_high_precision_mvs_constraint_flag | Y, SPS high_precision_mvs_enabled_flag | Y | Y |
| Temporal MV Prediction |  |  | Y no_temporal_mvp_constraint_flag | Y, SPS sps_temporal_mvp_enabled_flag | Y | Y |
| Alternative temporal motion vector prediction | ATMVP | JVET-K0346 | Y no_ sbtmvp_ constraint_flag | Y, SPS sps_sbtmvp_enabled_flag | Y | Y |
| Adaptive motion vector resolution | AMVR | JVET-K0357 | Y no_adaptive_mv_res_ constraint_flag | Y, SPS adaptive_mv_res_enabled_flag | N | Y |

# Proposed Syntax

#### Sequence parameter set RBSP syntax

[Ed. (BB): Preliminary basic SPS, subject to further study and pending further specification development.]

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_tier_level( ) |  |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| qtbtt_dual_tree_intra_flag | ue(v) |
| log2_ctu_size_minus2 | ue(v) |
| log2_min_qt_size_intra_slices_minus2 | ue(v) |
| log2_min_qt_size_inter_slices_minus2 | ue(v) |
| max_mtt_hierarchy_depth_inter_slices | ue(v) |
| max_mtt_hierarchy_depth_intra_slices | ue(v) |
| if ( !no_ chroma_separate_tree_constraint_flag) |  |
| chroma_separate_tree_enabled_flag | u(1) |
| if ( !no_cclm_constraint_flag ) |  |
| sps_cclm_enabled_flag | u(1) |
| if ( !no_extra_intra_modes_constraint_flag ) |  |
| extra_intra_modes_enabled_flag | u(1) |
| if ( !no_wide_intra_angles_constraint_flag ) |  |
| wide_intra_angles_enabled_flag | u(1) |
| if ( ! no_mts_intra_constraint_flag ) |  |
| sps_mts_intra_enabled_flag | u(1) |
| if ( ! no_mts_inter_constraint_flag ) |  |
| sps_mts_inter_enabled_flag | u(1) |
| if( ! no_affine_motion_constraint_flag ) { |  |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) |  |
| sps_affine_type_flag | u(1) |
| } |  |
| if ( !no_high_precision_mvs_constraint_flag ) |  |
| high_precision_mvs_enabled_flag | u(1) |
| if ( !no_temporal_mvp_constraint_flag ) { |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag && !no_sbtmvp_constraint_flag) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| if( sps_sbtmvp_enabled_flag ) |  |
| log2_sbtmvp_default_size_minus2 | u(1) |
| } |  |
| if ( !no_adaptive_mv_res_constraint_flag ) |  |
| sps_amvr_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Profile tier level syntax

| profile_tier_level( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| user_registered_sub_profile_idc | u(8) |
| tier_idc | u(8) |
| level_idc | u(8) |
| intra_only_constraint_flag | u(1) |
| max_bitdepth_constraint_idc | u(4) |
| max_chroma_format_constraint_idc | u(2) |
| frame_only_constraint_flag | u(1) |
| no_chroma_separate_tree_constraint_flag | u(1) |
| no_adaptive_loop_filter_constraint_flag | u(1) |
| no_sample_adaptive_offest_constraint_flag | u(1) |
| no_dep_quant_constraint_flag | u(1) |
| no_sign_data_hiding_constraint_flag | u(1) |
| no_cclm_constraint_flag | u(1) |
| no_extra_intra_modes_constraint_flag | u(1) |
| no_wide_intra_angles_constraint_flag | u(1) |
| no_multi_trans_intra_constraint_flag | u(1) |
| no_multi_trans_inter_constraint_flag | u(1) |
| no_affine_motion_constraint_flag | u(1) |
| no_high_precision_mvs_constraint_flag | u(1) |
| no_temporal_mvp_constraint_flag | u(1) |
| no_sbtmvp_constraint_flag _constraint_flag | u(1) |
| no_adaptive_mv_res_constraint_flag | u(1) |
| reserved_zero_40bits | u(40) |
| profile_extension_present_flag | u(1) |
| if( profile_extension_present_flag ) { |  |
| while( more_rbsp_data( ) ) |  |
| profile_extension_data_flag | u(1) |
| } |  |

### Slice header syntax

[Ed. (BB): Preliminary basic slice header, subject to further study and pending further specification development.]

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| slice_address | u(v) |
| slice_type | ue(v) |
| if ( slice_type != I ) { |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| if( sps_sbtmvp_enabled_flag ) { |  |
| sbtmvp_size_override_flag | u(1) |
| if( sbtmvp_size_override_flag ) |  |
| log2_sbtmvp_active_size_minus2 | u(3) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| } |  |
| if ( !no_dep_quant_constraint_flag ) { |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag && !no_sign_data_hiding_constraint_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| } |  |
| if ( !no_adaptive_loop_filter_constraint_flag ) { |  |
| adaptive_loop_filter_enabled_flag | u(1) |
| if( adaptive_loop_filter _enabled_flag ) |  |
| adaptive_loop_filter_params( ) |  |
| } |  |
| if ( !no_sample_adaptive_offset_constraint_flag ) |  |
| sample_adaptive_offset_enabled_flag | u(1) |
| byte_alignment( ) |  |
| } |  |

# Patent rights declaration(s)

Intel does not have any current or pending patent rights relating to the technology described in this contribution.