| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0548-v1 |
| --- | --- |

| Title: | CE1-related: On maximum/minimum allowed QT/BT/TT sizes for chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chih-Wei Hsu, Ching-Yeh Chen, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, chingyeh.chen, peter.chuang, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

When separate coding tree between luma and chroma is enabled in intra slices, maximum/minimum allowed QT/BT/TT sizes for chroma can be different from those derived from corresponding maximum/minimum QT/BT/TT sizes for luma according to chroma subsampling format. In this contribution, related parameters are named as MaxQtSizeC, MaxBtSizeC, MaxTtSizeC, MinQtSizeC, MinBtSizeC, and MinTtSizeC, where C stands for chroma. Results of four tests compared with VTM2.0.1 are reported as follows.

VTM2.0.1: {MaxQtSizeC, MaxBtSizeC, MaxTtSizeC, MinQtSizeC, MinBtSizeC, MinTtSizeC} in units of chroma samples = {32, 32, 16, 2, 2, 2}

Test1: Related parameters = {32, 32, 32, 2, 2, 2}
AI BD-rates = -0.08% (Y), -0.91% (U), -0.94% (V); EncT=103%; DecT=101%
RA BD-rates = 0.00% (Y), -0.32% (U), -0.31% (V); EncT=100%; DecT=100%
LB BD-rates = 0.01% (Y), -0.09% (U), -0.44% (V); EncT=100%; DecT=99%

Test2: Related parameters = {32, 32, 16, 4, 2, 2}
AI BD-rates = 0.00% (Y), 0.04% (U), 0.04% (V); EncT=100%; DecT=100%
RA BD-rates = 0.00% (Y), -0.01% (U), 0.00% (V); EncT=100%; DecT=100%
LB BD-rates = 0.00% (Y), 0.04% (U), -0.09% (V); EncT=100%; DecT=99%

Test3: Related parameters = {32, 32, 32, 4, 2, 2}
AI BD-rates = -0.08% (Y), -0.89% (U), -0.91% (V); EncT=103%; DecT=100%
RA BD-rates = -0.01% (Y), -0.36% (U), -0.30% (V); EncT=100%; DecT=100%
LB BD-rates = -0.02% (Y), -0.23% (U), -0.50% (V); EncT=100%; DecT=101%

Test4: Related parameters = {32, 32, 32, 4, 4, 4}
AI BD-rates = 0.06% (Y), 0.30% (U), 0.51% (V); EncT=100%; DecT=99%
RA BD-rates = 0.03% (Y), 0.55% (U), 0.78% (V); EncT=100%; DecT=100%
LB BD-rates = -0.05% (Y), 0.08% (U), 0.06% (V); EncT=100%; DecT=97%

It is suggested to Test1 or Test4 for consistent settings between QT, BT, and TT. It is also suggested that Test1 is slightly preferred than Test4 because of coding efficiency.

# Introduction

When separate coding tree between luma and chroma is enabled in intra slices, maximum/minimum allowed quad-tree / binary tree / ternary tree (QT/BT/TT) sizes for chroma can be different from those derived from corresponding maximum/minimum QT/BT/TT sizes for luma according to chroma subsampling format. In this contribution, maximum allowed QT size for chroma, maximum allowed BT size for chroma, maximum allowed TT size for chroma, minimum allowed QT size for chroma, minimum allowed BT size for chroma, and minimum allowed TT size for chroma, are denoted as MaxQtSizeC, MaxBtSizeC, MaxTtSizeC, MinQtSizeC, MinBtSizeC, and MinTtSizeC, respectively.

# Conducted tests

Table 1 summarizes the settings of related parameters for VTM2.0.1 and the four tests in this contribution. All the numbers are in units of chroma samples.

Table 1. Summary of related parameter settings

|  | VTM2.0.1 | Test1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| MaxQtSizeC | 32 | 32 | 32 | 32 | 32 |
| MaxBtSizeC | 32 | 32 | 32 | 32 | 32 |
| MaxTtSizeC | 16 | 32 | 16 | 32 | 32 |
| MinQtSizeC | 2 | 2 | 4 | 4 | 4 |
| MinBtSizeC | 2 | 2 | 2 | 2 | 4 |
| MinTtSizeC | 2 | 2 | 2 | 2 | 4 |

# Experimental results

The tests have been implemented on VTM2.0.1 [1], and the results of Test1, Test2, Test3, and Test4 are shown in Table 2, Table 3, Table 4, and Table 5, respectively. The simulations were performed following the JVET CTC [2].

Table 2. Results of Test1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.99% | -1.20% | 103% | 102% |
| Class A2 | -0.09% | -1.07% | -0.75% | 104% | 100% |
| Class B | -0.06% | -1.02% | -1.21% | 103% | 100% |
| Class C | -0.08% | -0.39% | -0.33% | 102% | 100% |
| Class E | -0.11% | -1.16% | -1.23% | 102% | 100% |
| Overall | -0.08% | -0.91% | -0.94% | 103% | 101% |
| Class D | -0.04% | -0.16% | -0.44% | 102% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.14% | -0.17% | 100% | 101% |
| Class A2 | -0.03% | -0.52% | -0.30% | 100% | 100% |
| Class B | 0.03% | -0.57% | -0.60% | 100% | 100% |
| Class C | 0.00% | 0.00% | -0.06% | 100% | 100% |
| Overall | 0.00% | -0.32% | -0.31% | 100% | 100% |
| Class D | -0.01% | -0.22% | -0.17% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.02% | -0.13% | -0.18% | 100% | 99% |
| Class C | 0.02% | -0.07% | -0.20% | 100% | 99% |
| Class E | 0.00% | -0.04% | -1.19% | 100% | 99% |
| Overall | 0.01% | -0.09% | -0.44% | 100% | 99% |
| Class D | 0.03% | -0.11% | -0.23% | 101% | 99% |

Table 3. Results of Test 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | 0.03% | 100% | 101% |
| Class A2 | 0.01% | 0.01% | 0.00% | 100% | 100% |
| Class B | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Class C | -0.01% | 0.08% | 0.19% | 99% | 100% |
| Class E | 0.00% | 0.06% | -0.05% | 100% | 99% |
| Overall | 0.00% | 0.04% | 0.04% | 100% | 100% |
| Class D | -0.03% | 0.16% | 0.01% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.03% | 0.09% | 100% | 104% |
| Class A2 | 0.02% | 0.07% | -0.01% | 100% | 99% |
| Class B | 0.02% | -0.06% | -0.02% | 100% | 100% |
| Class C | -0.01% | -0.04% | -0.02% | 100% | 99% |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Class D | 0.04% | 0.00% | 0.22% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.01% | -0.09% | -0.20% | 100% | 99% |
| Class C | 0.06% | 0.12% | -0.15% | 100% | 99% |
| Class E | -0.09% | 0.16% | 0.17% | 100% | 100% |
| Overall | 0.00% | 0.04% | -0.09% | 100% | 99% |
| Class D | 0.01% | 0.27% | -0.02% | 100% | 99% |

Table 4. Results of Test 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -1.03% | -1.18% | 103% | 101% |
| Class A2 | -0.10% | -1.10% | -0.75% | 104% | 101% |
| Class B | -0.06% | -1.00% | -1.18% | 102% | 100% |
| Class C | -0.07% | -0.31% | -0.22% | 102% | 100% |
| Class E | -0.12% | -1.12% | -1.25% | 103% | 100% |
| Overall | -0.08% | -0.89% | -0.91% | 103% | 100% |
| Class D | -0.03% | -0.13% | -0.37% | 101% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.13% | -0.18% | 100% | 101% |
| Class A2 | -0.01% | -0.52% | -0.30% | 100% | 99% |
| Class B | 0.01% | -0.54% | -0.54% | 100% | 101% |
| Class C | 0.01% | -0.18% | -0.11% | 100% | 100% |
| Overall | -0.01% | -0.36% | -0.30% | 100% | 100% |
| Class D | 0.01% | -0.24% | -0.15% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.01% | -0.11% | -0.23% | 100% | 101% |
| Class C | 0.03% | -0.12% | -0.19% | 100% | 101% |
| Class E | -0.15% | -0.55% | -1.35% | 100% | 101% |
| Overall | -0.02% | -0.23% | -0.50% | 100% | 101% |
| Class D | -0.02% | 0.49% | -0.68% | 100% | 99% |

Table 5. Results of Test4

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.55% | -1.11% | 101% | 100% |
| Class A2 | 0.12% | 0.15% | 0.29% | 99% | 99% |
| Class B | 0.02% | 0.17% | 0.43% | 101% | 100% |
| Class C | 0.21% | 1.93% | 2.93% | 99% | 97% |
| Class E | -0.09% | -0.63% | -0.75% | 102% | 100% |
| Overall | 0.06% | 0.30% | 0.51% | 100% | 99% |
| Class D | 0.33% | 3.15% | 4.05% | 98% | 95% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.25% | -0.26% | 100% | 105% |
| Class A2 | 0.01% | 0.46% | 0.37% | 100% | 102% |
| Class B | 0.04% | 0.39% | 0.69% | 100% | 100% |
| Class C | 0.09% | 1.43% | 1.97% | 100% | 96% |
| Overall | 0.03% | 0.55% | 0.78% | 100% | 100% |
| Class D | 0.09% | 1.97% | 3.01% | 99% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.00% | -0.17% | -0.14% | 99% | 97% |
| Class C | 0.04% | 0.41% | 0.34% | 100% | 97% |
| Class E | -0.24% | 0.03% | 0.03% | 100% | 99% |
| Overall | -0.05% | 0.08% | 0.06% | 100% | 97% |
| Class D | 0.06% | 0.19% | -0.12% | 100% | 100% |

# Conclusions

It is suggested to unify the chroma size settings between QT, BT, and TT.

# Reference

- VTM-2.0.1: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-2.0.1

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-K1010, July 2018.

# Patent rights declaration(s)

MediaTek Inc. does not have any current or pending patent rights relating to the technology described in this contribution.