| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0483 |
| --- | --- |

| Title: | CE8-related: CPR mode signaling and interaction with inter coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien Vadim Seregin Marta Karczewicz 5775 Morehouse Dr., San Diego CA 92130, USA | Tel: Email: | +1-858-845-1795 wchien@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two schemes to align the interaction between CPR mode with all other inter coding tools. The method is implemented on top of VTM-3.0 with CPR enabled. The first scheme removes   CPR dependency in motion derivation process. It results in negligible BD-rate difference for all test configurations on regular or screen content materials. The second scheme uses CPR mode as a third mode other than intra or inter modes. Motion predictor for CPR mode and inter mode would be derived mutual-exclusively from each other. The simulations show 1.1% in RA configuration and 1.5% in LDB configuration in screen content coding materials.

# Introduction

In 12th JVET meeting, current picture referencing (CPR) mode [1] was adopted into VTM3.0. However, the interaction between CPR mode and newly adopted coding tools, such as pairwise merge candidate, history based motion predictor, intra/inter multi-hypothesis mode, merge mode with motion vector difference (MMVD), and triangular partition were not clearly defined. As a result, the tool interactions were integrated in VTM3.0 in several different ways. First, CPR can be used with pairwise merge candidate and history based motion predictor. A new pairwise CPR merge candidate can be generated by averaging two CPR merge candidates. A new pairwise merge candidate can not be generated by averaging a CPR merge candidate and a non-CPR merge candidate. For history based motion predictor, CPR motion is inserted in to history buffer for future referencing. Second, CPR cannot be used with other inter tools, such as affine motion, intra/inter multi-hypothesis mode, MMVD, and triangular partition. Bitstream conformance checks were added at decoder to disable those tool combinations. Besides bitstream conformance check, motion derivation process in some inter tools were modified which excludes neighboring blocks in CPR mode. Those inter tools are affine merge mode, MMVD and Triangular partition. However, the motion derivation of the regular merge mode and the intra/inter multi-hypothesis mode includes neighboring blocks in CPR mode.

# Proposed methods

Method 1:  reference index signaling and bitstream conformance restrictions

In this method, the blocks in CPR mode are treated as blocks in inter prediction mode. There are no changes needed in derivation process for motion vectors compared to the case when CPR mode is disabled. Bitstream conformance checks are added to the encoder to disallow CPR combining with other coding tools, such as affine motion, intra/inter multi-hypothesis mode, MMVD, and triangular partition.

Method 2:  reference index signaling and bitstream conformance restrictions and disable pairwise merge candidate for CPR mode

In this method, besides the modifications in Method 1, one restriction is further enforced in pairwise merge candidate. The restriction is that a new pairwise merge candidate can only be generated by averaging two non-CPR MV candidates.

Method 3: block level flag signaling and independent CPR mode

In this method, CPR mode is signalled with a block level flag and can be signaled as CPR AMVP mode or CPR skip/merge mode. CPR mode is treated as the third prediction mode other than intra or inter prediction modes. The current picture is no longer included as one of the reference pictures in the reference picture list 0. The derivation process of motion vectors for CPR mode excludes all neighboring blocks in inter mode and vice versa. Bitstream conformance checks are no longer needed at the encoder and redundant mode signaling is removed.

# Simulation Results

The simulation is performed by using VTM3.0 software. Two different anchors are generated, one by using VTM3.0 default configurations and another by using VTM3.0 default configurations with CPR mode. Common test conditions in CE8 [2] are followed. Results are shown in the following two tables:

Table I: coding performance of method 1: Reference index signaling and bitstream conformance restrictions

|  | All Intra Main10 - VTM configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.10% | 0.06% | 124% | 102% | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Class A2 | -0.47% | -0.50% | -0.55% | 130% | 97% | 0.00% | 0.00% | 0.00% | 100% | 96% |
| Class B | -0.30% | -0.39% | -0.36% | 134% | 105% | 0.00% | 0.00% | 0.00% | 98% | 103% |
| Class C | -0.09% | -0.24% | -0.25% | 138% | 101% | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Class E | -0.64% | -0.81% | -0.59% | 140% | 106% | 0.00% | 0.00% | 0.00% | 98% | 102% |
| Overall | -0.27% | -0.40% | -0.33% | 133% | 102% | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | -0.10% | -0.23% | 0.22% | 134% | 100% | 0.00% | 0.00% | 0.00% | 98% | 101% |
| Class F | -12.09% | -12.04% | -12.10% | 150% | 99% | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Class SCC 1080p | -36.92% | -36.11% | -36.36% | 139% | 91% | 0.00% | 0.00% | 0.00% | 98% | 99% |
|  | Random Access Main 10 - VTM configuration |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.05% | 0.25% | 97% | 100% | -0.02% | 0.09% | 0.02% | 99% | 100% |
| Class A2 | 0.01% | 0.08% | 0.20% | 99% | 101% | 0.01% | 0.01% | -0.04% | 101% | 100% |
| Class B | 0.03% | -0.03% | -0.25% | 99% | 101% | 0.01% | -0.03% | -0.04% | 101% | 101% |
| Class C | 0.19% | -0.11% | 0.12% | 103% | 100% | 0.00% | -0.05% | 0.04% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.09% | -0.01% | 0.04% | 99% | 100% | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Class D | 0.14% | -0.27% | 0.17% | 100% | 98% | -0.01% | 0.01% | 0.02% | 99% | 99% |
| Class F | -9.93% | -9.98% | -9.98% | 102% | 96% | -0.03% | -0.02% | 0.01% | 97% | 99% |
| Class SCC 1080p | -22.02% | -21.58% | -22.12% | 101% | 97% | 0.00% | 0.00% | -0.03% | 98% | 99% |
|  | Low delay B Main10 - VTM configuration |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.05% | -0.14% | -0.09% | 106% | 101% | -0.01% | 0.18% | 0.05% | 100% | 98% |
| Class C | 0.15% | 0.43% | 0.60% | 110% | 104% | 0.02% | 0.04% | 0.26% | 101% | 103% |
| Class E | 0.13% | 1.24% | -0.21% | 106% | 100% | -0.15% | 0.18% | -0.02% | 100% | 100% |
| Overall | 0.10% | 0.40% | 0.11% | 107% | 102% | -0.04% | 0.13% | 0.10% | 100% | 100% |
| Class D | 0.08% | -0.04% | 0.50% | 111% | 100% | -0.05% | 0.12% | -0.19% | 100% | 100% |
| Class F | -5.16% | -5.28% | -5.77% | 108% | 99% | -0.02% | 0.12% | -0.30% | 100% | 98% |
| Class SCC 1080p | -13.00% | -13.17% | -13.41% | 107% | 93% | 0.28% | 0.46% | 0.40% | 99% | 98% |

Table II: coding performance of method 2: Reference index signaling and bitstream conformance restrictions

|  | All Intra Main10 - VTM configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.06% | 0.04% | 125% | 104% | 0.00% | 0.04% | -0.02% | 99% | 104% |
| Class A2 | -0.46% | -0.50% | -0.52% | 129% | 97% | 0.01% | 0.00% | 0.02% | 99% | 96% |
| Class B | -0.29% | -0.38% | -0.33% | 132% | 103% | 0.01% | 0.01% | 0.03% | 97% | 102% |
| Class C | -0.08% | -0.21% | -0.23% | 137% | 102% | 0.01% | 0.03% | 0.02% | 98% | 98% |
| Class E | -0.62% | -0.79% | -0.58% | 139% | 103% | 0.01% | 0.01% | 0.02% | 97% | 100% |
| Overall | -0.26% | -0.38% | -0.32% | 132% | 102% | 0.01% | 0.02% | 0.02% | 98% | 100% |
| Class D | -0.09% | -0.13% | 0.20% | 133% | 99% | 0.01% | 0.10% | -0.01% | 98% | 101% |
| Class F | -12.05% | -12.06% | -11.92% | 152% | 102% | 0.05% | -0.02% | 0.20% | 98% | 102% |
| Class SCC 1080p | -36.87% | -36.07% | -36.33% | 139% | 91% | 0.08% | 0.07% | 0.04% | 98% | 101% |
|  | Random Access Main 10 - VTM configuration |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.05% | 0.30% | 96% | 98% | -0.03% | 0.09% | 0.07% | 98% | 98% |
| Class A2 | -0.03% | -0.01% | 0.16% | 97% | 100% | -0.03% | -0.08% | -0.08% | 100% | 99% |
| Class B | 0.00% | -0.08% | -0.13% | 99% | 100% | -0.02% | -0.07% | 0.08% | 100% | 101% |
| Class C | 0.19% | -0.19% | -0.01% | 103% | 100% | 0.00% | -0.13% | -0.09% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.07% | -0.07% | 0.05% | 99% | 100% | -0.02% | -0.06% | 0.00% | 100% | 100% |
| Class D | 0.15% | -0.40% | 0.22% | 103% | 101% | -0.01% | -0.12% | 0.07% | 101% | 102% |
| Class F | -9.85% | -9.89% | -10.03% | 104% | 98% | 0.06% | 0.09% | -0.03% | 100% | 102% |
| Class SCC 1080p | -21.95% | -21.56% | -22.05% | 103% | 98% | 0.09% | 0.03% | 0.06% | 100% | 101% |
|  | Low delay B Main10 - VTM configuration |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.09% | -0.22% | 0.03% | 106% | 102% | 0.02% | 0.10% | 0.16% | 99% | 99% |
| Class C | 0.09% | 0.42% | 0.62% | 109% | 103% | -0.04% | 0.03% | 0.28% | 99% | 102% |
| Class E | 0.12% | 0.36% | -0.61% | 105% | 98% | -0.16% | -0.70% | -0.42% | 99% | 99% |
| Overall | 0.10% | 0.13% | 0.06% | 106% | 101% | -0.04% | -0.13% | 0.06% | 99% | 100% |
| Class D | 0.11% | -0.26% | 0.29% | 111% | 98% | -0.02% | -0.11% | -0.39% | 100% | 98% |
| Class F | -5.17% | -4.80% | -5.42% | 107% | 100% | -0.02% | 0.60% | 0.06% | 99% | 99% |
| Class SCC 1080p | -13.16% | -13.44% | -13.72% | 108% | 96% | 0.08% | 0.13% | 0.02% | 99% | 101% |

Table III: coding performance of method 3: block level flag signaling and independent CPR mode

|  | All Intra Main10 - VTM configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.10% | -0.01% | 131% | 101% | -0.07% | 0.00% | -0.07% | 104% | 101% |
| Class A2 | -0.56% | -0.58% | -0.61% | 133% | 89% | -0.08% | -0.07% | -0.07% | 102% | 88% |
| Class B | -0.40% | -0.38% | -0.45% | 141% | 99% | -0.10% | 0.01% | -0.09% | 104% | 98% |
| Class C | -0.20% | -0.23% | -0.33% | 141% | 97% | -0.11% | 0.01% | -0.09% | 101% | 93% |
| Class E | -0.86% | -0.92% | -0.72% | 149% | 103% | -0.23% | -0.12% | -0.13% | 104% | 99% |
| Overall | -0.39% | -0.42% | -0.42% | 139% | 98% | -0.12% | -0.03% | -0.09% | 103% | 96% |
| Class D | -0.32% | -0.35% | -0.41% | 139% | 97% | -0.22% | -0.11% | -0.63% | 102% | 98% |
| Class F | -12.25% | -12.18% | -12.12% | 157% | 100% | -0.19% | -0.17% | -0.05% | 101% | 100% |
| Class SCC 1080p | -37.14% | -36.30% | -36.56% | 142% | 87% | -0.37% | -0.31% | -0.35% | 100% | 100% |
|  | Random Access Main 10 - VTM configuration |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.46% | -0.55% | 100% | 99% | -0.22% | -0.42% | -0.77% | 102% | 99% |
| Class A2 | -0.08% | -0.38% | -0.16% | 95% | 96% | -0.09% | -0.46% | -0.41% | 97% | 95% |
| Class B | -0.10% | -0.14% | -0.51% | 102% | 95% | -0.12% | -0.14% | -0.30% | 103% | 96% |
| Class C | 0.04% | -0.19% | -0.13% | 105% | 96% | -0.15% | -0.13% | -0.21% | 102% | 95% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.05% | -0.27% | -0.35% | 101% | 96% | -0.14% | -0.26% | -0.39% | 101% | 96% |
| Class D | 0.03% | -0.60% | -0.30% | 104% | 94% | -0.13% | -0.32% | -0.46% | 102% | 96% |
| Class F | -10.09% | -10.07% | -10.10% | 107% | 95% | -0.22% | -0.13% | -0.13% | 103% | 99% |
| Class SCC 1080p | -22.90% | -22.56% | -23.06% | 108% | 94% | -1.08% | -1.19% | -1.18% | 104% | 97% |
|  | Low delay B Main10 - VTM configuration |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3 |  |  |  |  | Over VTM-3 + CPR |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.07% | -0.60% | -0.74% | 111% | 97% | -0.13% | -0.28% | -0.61% | 104% | 94% |
| Class C | -0.05% | -0.51% | -0.60% | 111% | 86% | -0.17% | -0.89% | -0.92% | 102% | 85% |
| Class E | 0.10% | 0.73% | -0.45% | 104% | 92% | -0.17% | -0.32% | -0.27% | 99% | 92% |
| Overall | -0.02% | -0.24% | -0.62% | 109% | 92% | -0.16% | -0.50% | -0.63% | 102% | 91% |
| Class D | -0.05% | -0.54% | -0.14% | 114% | 93% | -0.18% | -0.39% | -0.83% | 102% | 93% |
| Class F | -5.71% | -6.37% | -6.10% | 110% | 98% | -0.64% | -1.07% | -0.68% | 102% | 97% |
| Class SCC 1080p | -14.55% | -15.02% | -15.25% | 112% | 91% | -1.48% | -1.65% | -1.68% | 103% | 95% |

# Conclusions

In this contribution, two different schemes of CPR mode are proposed and tested. Both methods could unify how the inter tools interact with CPR mode. Minor coding difference for normal contents as well as screen content materials are observed in the solution with bitstream conformance checks (Method 2). Over 1% coding gain is observed for screen content materials for Method 3. It is recommended to adopt method 2 or method 3 into the test model.

# References

[1] X. Xu, X. Li, S. Liu, “CE8: CPR mode with local search ranges (Test CE8.3.1 and CE8.3.2)”, JVET-L0293, Macao, CN, Oct. 2018

[2] X. Xu, Y.-C. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-L1028, Macao, CN, Oct. 2018

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).