| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0697-v1 |
| --- | --- |

| Title: | Cross-check of JVET-M0489 (CE7-related: Reduced context models for transform coefficients coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Y. Chen F. Le Léannec 975, av. des Champs Blancs CS 17616 35576 Cesson-Sévigné | Tel: Email: | +33(0)299273198 fabrice.leleannec@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

The cross-checker reports the source code provided by Tencent was carefully analysed and found to be consistent with the description in JVET-M0489. The cross checker could exactly reproduce the results reported in JVET-M0489 as the “proposed technique”. The measured encoder and decoder run time ratios are similar to those reported in JVET- M0489, considering the timing measurements inaccuracy.

# Cross-check of JVET-M0489

The document reports the result of a cross check for JVET-M0489[1]:

- The source code provided by Tencent was carefully analysed. It is confirmed that the implementation matches the description in JVET-M0489.

- Simulations were conducted according to the common test conditions[2] for the VTM-3.0 in the AI, RA and LDB configuration settings.

- The simulations are completed. The test results exactly match the results reported in JVET- M0489 as the “proposed technique”. The run times measured are consistent with the run times provided by the proponent, assuming some timing measurements errors.

Partial detailed results can be found in the attached spreadsheets. The partial results of the cross check are summarized below (others are still running):

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.05% | 0.05% | 88% | 75% |
| Class A2 | 0.04% | 0.08% | 0.03% | 85% | 74% |
| Class B | 0.04% | 0.01% | 0.03% | 86% | 74% |
| Class C | 0.05% | -0.05% | -0.17% | 86% | 74% |
| Class E | 0.05% | 0.02% | 0.07% | 87% | 76% |
| Overall | 0.04% | 0.00% | 0.00% | 86% | 75% |
| Class D | 0.06% | 0.06% | -0.05% | 89% | 80% |
| Class F | 0.04% | 0.04% | 0.08% | 88% | 78% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | 0.20% | -0.04% | 133% | 96% |
| Class C | 0.04% | 0.10% | 0.01% | 87% | 68% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | -0.36% | -0.07% | 89% | 71% |
| Class F | 0.01% | 0.15% | 0.07% | 132% | 85% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.01% | 0.37% | 0.15% | 91% | 74% |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.04% | 0.17% | 0.53% | 91% | 70% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# References

- M. Gao, X. Li, X. Zhao and S. Liu, “CE7-related: Reduced context models for transform coefficients coding”, JVET- M0489, Marrakech, MA, Jan. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, CN, October 2018.