| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0077-v2 |
| --- | --- |

| Title: | Hypothetical Reference Decoder in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui | Tel: Email: | +81 44 754 2049 kazui.kimihiko@fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

- Abstract

It is proposed to incorporate Annex C of the HEVC standard [2] into the VVC draft [1] as the starting point for designing the hypothetical reference decoder (HRD) model for VVC. It is also proposed to add HRD-related parameters to the sequence parameter set (SPS), picture parameter set (PPS) and tile group header, and to define supplemental enhancement information (SEI) messages for signalling HRD parameters as the starting point.

In version 2, section 3 is removed since the author realized that it was already agreed in spirits. Moreover, the proposal on Recovery point SEI is removed since it is not directly related to the HRD model.

Introduction

The specification of the hypothetical reference decoder (HRD) model is not described in the current VVC draft [1]. Since the specification of bitstream and decode conformance plays essential role in video coding standards, it is proposed to start drafting the corresponding part in the next version of the VVC draft.

The author asserts that Annex C of the HEVC standard [2] is a good starting point since there is no significant changes in the requirement from the HEVC standard.

It is also proposed to add HRD-related parameters to the sequence parameter set (SPS), picture parameter set (PPS) and tile group header, and to define supplemental enhancement information (SEI) messages for signalling HRD parameters as the starting point.

Proposed text

Proposed text is in the associated file.

HRD specification

The author proposes to incorporate Annex C of the HEVC standard as the starting point. Highlighted parts with green need to be aligned with the current design.

HRD-related syntax element

The following syntax elements which exist in the HEVC standard are proposed to be included in SPS, PPS and tile group header.

- In SPS

  - conformance_window_flag, conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, conf_win_bottom_offset

  - sps_sub_layer_ordering_info_present_flag

  - sps_max_num_reorder_pics, sps_max_latency_increase_plus1

  - vui_parameters_present_flag, vui_parameters( )

- In PPS

  - output_flag_present_flag

- In tile group header

  - pic_output_flag

HRD-related SEI messages

The following SEI messages are proposed to be added to the next version of the draft.

- Buffering period SEI

- Picture timing SEI

- Recovery point SEI

- Decoding unit info SEI

- Filler payload SEI

HRD-related RBSP

Filler data RBSP, which is defined in the HEVC standard, is missing. The author asserts such RBSP is still useful for bitrate control.

References

[1] “Versatile Video Coding (Draft 4)” JVET-M1001, January 2019

[2] ITU-T H.265 (02/2018)

Patent rights declaration(s)

Fujitsu Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).