| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0161_v2 |
| --- | --- |

| Title: | Non-CE6: Implicit transform selection based on intra mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoqiang Cao Fangdong Chen Li Wang | Tel: Email: | caoxiaoqiang@hikvision.com chenfangdong@hikvision.com wangli7@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

This contribution present two implicit transform selection methods based on intra mode, all the two method are conducted when implicit MTS is switch on. For the first proposed method, it is reported that the BD-rate are -0.22%, -0.05%, -0.02% for AI, RA and LB respectively. For the second proposed method, it is reported that the BD-rate are -0.27%, -0.05%, 0.01% for AI, RA and LB respectively.

This contribution also proposed to unify implicit MTS and ISP based on intra mode:

1. When unifying implicit and ISP with original ISP transform method, it is reported that the BD-rate are:

-0.24%, -0.04%, -0.03% for AI, RA and LB respectively.

2. When unifying implicit and ISP with proposed method 1, it is reported that the BD-rate are:

-0.22%, -0.03%, -0.03% for AI, RA and LB respectively.

3. When unifying implicit and ISP with proposed method 2, it is reported that the BD-rate are:

-0.25%, -0.05%, -0.01% for AI, RA and LB respectively.

Introduction

In current implicit MTS that adopt in VTM-4.0[1]. When the implicit MTS is switched on, the selection of transform is adaptively based on the shape of transform unit. Hence, no signalling is required to indicated the transform pair in the decoder for current transform unit. For example, let tH and tV be the horizontal and the vertical transforms selected respectively for the w×h transform unit, where w is the width and h is the height. Then the transform is selected according to the Table 1:

Table 1 – Implicit MTS based on shape in VTM4.0

tH = (W >= 4 && W <= 16 && W <= H)? DST7: DCT2 tV = (H >= 4 && H <= 16 && H <= W)? DST7: DCT2

In current Intra sub-partition that adopt in VTM4.0[2], the transform choice for ISP mode is implicit. For example, if a CU uses the ISP coding mode, the MTS CU flag will be set to 0 and it will not be sent to the decoder. Therefore, the encoder will not perform RD tests for the different available transforms for each resulting sub-partition. The transform choice for the ISP mode will instead be fixed and selected according the intra mode, the processing order and the block size utilized. Hence, no signalling is required. For example, let tH and tV be the horizontal and the vertical transforms selected respectively for the w×h sub-partition, where w is the width and h is the height. Then the transform is selected as following rules:

- If w=1 or h=1, then there is no horizontal or vertical transform respectively.

- If w=2 or w≥32, tH = DCT2

- If h =2 or h≥32, tV = DCT2

- Otherwise, the transform is selected as in Table 2.

Table 2 – Transform selection depends on intra mode

| Intra mode | tH | tV |
| --- | --- | --- |
| Planar Ang. 31,32,34,36,37 | DST7 | DST7 |
| DC Ang. 33, 35 | DCT2 | DCT2 |
| Ang. 2, 4, 6…28,30 Ang. 39,41,43…63,65 | DST7 | DCT2 |
| Ang. 3,5,7…27,29 Ang. 38,40,42…64,66 | DCT2 | DST7 |

Proposed Method

In this contribution, the proposed method introduced two methods about implicit transform selection based on intra mode. For the first method, when intra mode is Planar, the transform pair is inferred to (DST7, DST7), when intra mode is DC, the transform pair is inferred to (DCT2, DCT2), when intra mode is angular mode from 2 to 66, the transform pair is inferred based on a parity switching mechanism. For the second method, compared to the first method, all odd angular mode can use the same transform pair (DST7, DST7) and there is no need to classify the odd angular mode.

With the same size restriction for non-DCT2 transforms, we also proposed unified implicit MTS and ISP with Table 2, Table 3 and Table 4.

Proposed 1st version of implicit MTS based on intra mode

The transform selection for the whole transform unit will instead be selected according the intra mode and the block size. The transform pair for the whole transform unit is selected according to the following rules:

- If w≥32, tH = DCT2

- If h≥32, tV = DCT2

- Otherwise, the transform is selected as in Table 3.

Table 3 – Proposed transform selection depends on intra mode

| Intra mode | tH | tV |
| --- | --- | --- |
| DC | DCT2 | DCT2 |
| Planar | DST7 | DST7 |
| Ang. [2-34] && even Ang. [35-66] && odd | DST7 | DCT2 |
| Ang. [2-34] && odd Ang. [35-66] && even | DCT2 | DST7 |

Proposed 2st version of implicit MTS based on intra mode

The transform selection for the whole transform unit will instead be selected according the intra mode and the block size. The transform pair for the whole transform unit is selected according to the following rules:

- If w≥32, tH = DCT2

- If h≥32, tV = DCT2

- Otherwise, the transform is selected as in Table 4.

Table 4 – Proposed transform selection depends on intra mode

| Intra mode | tH | tV |
| --- | --- | --- |
| DC | DCT2 | DCT2 |
| Planar Ang. [2-66] && odd | DST7 | DST7 |
| Ang. [2-34] && even | DST7 | DCT2 |
| Ang. [35-66] && even | DCT2 | DST7 |

Experimental Results

To evaluate the coding performance of the proposed method, the VTM-4.0 is served as the experimental platform, the CTC specified in [3], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance. Due to implicit MTS are not include in CTC, thus all the tests are configured as following: MTS=0, Implicit MTS=1

Results for 1st version of proposed method in implicit MTS

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0+ImplicitMTS_ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | 0.19% | 0.15% | 102% | 101% |
| Class A2 | -0.16% | 0.07% | 0.05% | 101% | 101% |
| Class B | -0.25% | 0.19% | 0.19% | 100% | 100% |
| Class C | -0.21% | -0.03% | 0.04% | 103% | 102% |
| Class E | -0.32% | 0.11% | -0.04% | 101% | 100% |
| Overall | -0.22% | 0.11% | 0.09% | 101% | 101% |
| Class D | -0.28% | -0.12% | 0.07% | 102% | 103% |
| Class F (optional) | -0.07% | 0.07% | 0.09% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0+ImplicitMTS_ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.17% | 0.03% | 99% | 100% |
| Class A2 | -0.04% | 0.08% | 0.14% | 99% | 100% |
| Class B | -0.03% | 0.13% | 0.38% | 100% | 99% |
| Class C | -0.06% | 0.29% | -0.02% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | 0.10% | 0.15% | 100% | 100% |
| Class D | -0.15% | 0.04% | -0.22% | 100% | 101% |
| Class F (optional) | -0.03% | -0.01% | -0.18% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0+ImplicitMTS_ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.26% | -0.66% | 100% | 101% |
| Class C | -0.07% | -0.21% | -0.46% | 100% | 101% |
| Class E | 0.06% | -0.73% | -0.08% | 100% | 100% |
| Overall | -0.02% | -0.15% | -0.45% | 100% | 101% |
| Class D | -0.07% | 0.82% | 0.21% | 100% | 101% |
| Class F (optional) | -0.15% | 0.62% | -0.18% | 100% | 99% |

Results for 2st version of proposed method in implicit MTS

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0+ImplicitMTS_ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.07% | -0.17% | 102% | 101% |
| Class A2 | -0.21% | -0.14% | -0.18% | 101% | 102% |
| Class B | -0.29% | -0.04% | -0.03% | 100% | 100% |
| Class C | -0.21% | -0.22% | -0.04% | 102% | 103% |
| Class E | -0.38% | -0.32% | -0.37% | 101% | 100% |
| Overall | -0.27% | -0.15% | -0.14% | 101% | 101% |
| Class D | -0.27% | -0.29% | -0.20% | 102% | 102% |
| Class F (optional) | -0.12% | -0.11% | -0.14% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0+ImplicitMTS_ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.31% | -0.08% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | -0.04% | -0.02% | 0.09% | 100% | 100% |
| Class C | -0.09% | 0.34% | -0.15% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | 0.02% | -0.02% | 100% | 100% |
| Class D | -0.09% | -0.24% | 0.03% | 100% | 100% |
| Class F (optional) | -0.09% | -0.03% | -0.28% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0+ImplicitMTS_ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.09% | -0.52% | 100% | 100% |
| Class C | -0.08% | -0.36% | -0.32% | 100% | 102% |
| Class E | 0.19% | -1.56% | 0.18% | 100% | 101% |
| Overall | 0.01% | -0.55% | -0.28% | 100% | 101% |
| Class D | -0.03% | 0.13% | -0.16% | 101% | 101% |
| Class F (optional) | -0.17% | -0.07% | -0.11% | 100% | 99% |

Results for unified Implicit MTS and ISP with Table2

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0+implicitMTS_ON |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | 0.12% | 0.15% | 102% | 102% |
| Class A2 | -0.18% | 0.12% | 0.03% | 101% | 101% |
| Class B | -0.27% | 0.21% | 0.17% | 100% | 99% |
| Class C | -0.22% | -0.02% | 0.08% | 102% | 102% |
| Class E | -0.33% | 0.03% | -0.04% | 101% | 100% |
| Overall | -0.24% | 0.10% | 0.09% | 101% | 101% |
| Class D | -0.29% | -0.06% | 0.09% | 101% | 100% |
| Class F | -0.09% | -0.08% | -0.01% | 101% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0+implicitMTS_ON |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.14% | -0.11% | 100% | 100% |
| Class A2 | -0.03% | 0.22% | 0.07% | 100% | 100% |
| Class B | -0.03% | 0.18% | 0.16% | 100% | 100% |
| Class C | -0.05% | 0.22% | 0.11% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.14% | 0.07% | 100% | 100% |
| Class D | -0.16% | -0.01% | -0.22% | 101% | 101% |
| Class F | -0.04% | 0.02% | -0.14% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0+implicitMTS_ON |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.21% | -0.37% | 100% | 100% |
| Class C | -0.05% | -0.21% | -0.19% | 100% | 102% |
| Class E | 0.03% | -2.07% | -0.14% | 101% | 102% |
| Overall | -0.03% | -0.68% | -0.25% | 100% | 101% |
| Class D | -0.08% | 0.02% | -0.09% | 100% | 99% |
| Class F | -0.07% | -0.36% | 0.08% | 100% | 100% |

Results for unified Implicit MTS and ISP with Table3

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0+implicitMTS_ON |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | 0.18% | 0.12% | 102% | 101% |
| Class A2 | -0.17% | 0.11% | 0.06% | 101% | 100% |
| Class B | -0.25% | 0.19% | 0.22% | 100% | 99% |
| Class C | -0.22% | -0.01% | 0.02% | 102% | 102% |
| Class E | -0.31% | 0.06% | -0.01% | 101% | 99% |
| Overall | -0.22% | 0.11% | 0.10% | 101% | 100% |
| Class D | -0.28% | -0.02% | 0.15% | 102% | 106% |
| Class F | -0.06% | 0.06% | 0.02% | 101% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0+implicitMTS_ON |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.18% | -0.02% | 100% | 102% |
| Class A2 | -0.04% | 0.33% | 0.04% | 100% | 101% |
| Class B | -0.02% | 0.15% | 0.38% | 100% | 100% |
| Class C | -0.05% | 0.39% | -0.02% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.19% | 0.13% | 100% | 101% |
| Class D | -0.12% | -0.02% | 0.00% | 100% | 102% |
| Class F | 0.00% | 0.07% | -0.10% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0+implicitMTS_ON |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.10% | -0.56% | 100% | 101% |
| Class C | -0.06% | 0.06% | -0.33% | 100% | 102% |
| Class E | -0.04% | -0.70% | 0.54% | 100% | 102% |
| Overall | -0.03% | -0.20% | -0.21% | 100% | 101% |
| Class D | -0.04% | -0.03% | -0.45% | 100% | 100% |
| Class F | -0.14% | 0.25% | 0.90% | 100% | 101% |

Results for unified Implicit MTS and ISP with Table4

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0+implicitMTS_ON |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.12% | -0.14% | 102% | 101% |
| Class A2 | -0.20% | -0.18% | -0.20% | 101% | 101% |
| Class B | -0.27% | -0.12% | -0.09% | 101% | 99% |
| Class C | -0.17% | -0.26% | -0.11% | 103% | 103% |
| Class E | -0.37% | -0.34% | -0.45% | 101% | 100% |
| Overall | -0.25% | -0.20% | -0.18% | 101% | 101% |
| Class D | -0.24% | -0.25% | -0.15% | 102% | 106% |
| Class F | -0.10% | -0.12% | -0.21% | 101% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0+implicitMTS_ON |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.25% | -0.20% | 100% | 101% |
| Class A2 | 0.02% | 0.16% | 0.04% | 100% | 102% |
| Class B | -0.04% | 0.02% | 0.05% | 100% | 100% |
| Class C | -0.09% | 0.16% | -0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | 0.03% | -0.05% | 100% | 101% |
| Class D | -0.11% | -0.11% | -0.21% | 100% | 101% |
| Class F | -0.11% | -0.09% | -0.15% | 100% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0+implicitMTS_ON |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | 0.06% | -0.38% | 101% | 101% |
| Class C | -0.09% | -0.29% | -0.28% | 100% | 101% |
| Class E | 0.14% | -0.59% | 0.38% | 100% | 101% |
| Overall | -0.01% | -0.22% | -0.16% | 100% | 101% |
| Class D | -0.06% | 0.31% | 0.23% | 100% | 100% |
| Class F | -0.15% | 0.47% | -0.08% | 101% | 101% |

Conclusion

This contribution presents two implicit MTS method based on intra mode, compared with implicit MTS based on shape, the first method can achieve -0.22% coding gain in AI, and the second method can achieve -0.27% coding gain in AI. We also proposed to unify the implicit MTS and ISP based on intra mode. It is recommended of the inclusion of the proposed transform versions into VTM.

Reference

[1]	J. Lainema, “CE6: Shape adaptive transform selection,”JVET-M0303, Marrakech, MA, 9–18 Jan. 2019.

[2]	S. De-Luxán-Hernández, V. George, J. Ma, et al. “CE3: Intra Sub-Partitions Coding Mode,”JVET-M0102, Marrakech, MA, 9–18 Jan. 2019.

[3]	F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).