| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0367 |
| --- | --- |

| Title: | Support of chroma 4:4:4 format in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Xiaozhong Xu, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, xiaozhongxu, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes to support chroma 4:4:4 format in VVC by including several SW and spec text fixes. With the proposed fixes, the VTM-4.0 sw is capable to encode and decode video sequence in chroma 4:4:4 format. Comparisons on the coding performance between VTM-4.0 and HM-16.6 for video content with chroma 4:4:4 format are also reported.

# Problem statement and proposal

The current VTM-4.0 sw does not support encoding or decoding video sequence in chroma 4:4:4 format as is. Encoder crash happens with the following configuration.

InputChromaFormat             : 444

To support input video sequence format of chroma 4:4:4, several VTM-4.0 sw fixes are provided, as described in the following sub-sections.

- Cross-component Linear Model

It is proposed to disable the 6-tap downsampling filter when the chroma format of input video is 4:4:4.

- Affine

It is proposed to extend the idea of deriving chroma 4x4 MV from collocated top-left and bottom-right 4x4 luma blocks. When the input video is 4:4:4, the chroma 4x4 MV is derived from its collocated 4x4 luma block.

- DMVR

In function xProcessDMVR of VTM-4.0, it was assumed that chroma format is 4:2:0, in the proposed bug-fix, chroma format 4:4:4 has also been considered.

- Distortion calculation

In the following functions of VTM-4.0: xGetSSE_WTD, xGetSSE2_WTD, xGetSSE4_WTD, xGetSSE8_WTD, xGetSSE16_WTD, xGetSSE16N_WTD, xGetSSE32_WTD, xGetSSE64_WTD, when calculating cShift, the case of input chroma format 4:4:4 is also considered.

const size_t  cShift  = (rcDtParam.compID == COMPONENT_Y || rcDtParam.chromaFormat == CHROMA_444) ? 0 : 1;

# Experiments

Simulations are performed under CTC condition [1] using test materials defined in the CTC used in Screen Content Coding (SCC) [2]. The anchor is HM-16.6 and the test is VTM-4.0 with proposed fixes.

Table 1: Coding performance of VTM-4.0 versus HM-16.6 for chroma 4:4:4 video content.

|  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -45.2% | -40.0% | -39.9% | -44.6% | -37.3% | -39.1% |
| RGB, mixed content, 1440p & 1080p | -38.8% | -34.3% | -33.7% | -34.4% | -28.6% | -28.3% |
| RGB, Animation, 720p | -25.3% | -18.2% | -19.2% | -18.9% | -10.1% | -8.8% |
| RGB, camera captured, 1080p | -24.7% | -22.3% | -18.5% | -15.6% | -11.5% | -7.4% |
| YUV, text & graphics with motion, 1080p & 720p | -49.2% | -44.7% | -44.6% | -50.4% | -46.2% | -46.4% |
| YUV, mixed content, 1440p & 1080p | -43.5% | -35.3% | -35.0% | -39.8% | -30.4% | -30.3% |
| YUV, Animation, 720p | -28.5% | -18.6% | -19.2% | -25.2% | -11.7% | -14.5% |
| YUV, camera captured, 1080p | -25.5% | -17.3% | -15.6% | -19.1% | -8.8% | -2.6% |
| Enc Time[%] | 308% |  |  | 261% |  |  |
| Dec Time[%] | 155% |  |  | 131% |  |  |

# Conclusions

This proposal provides several fixes to VTM software and Spec text for supporting input video format of chroma 4:4:4. It is recommended to adopt the proposed changes into next version of VVC draft and VTM sw.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common test conditions for screen content coding,” JCTVC-U1015, 21st Meeting: Warsaw, PL, 19–26 June 2015.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).