| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0552-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-N0304 (CE1-related: combined bi-directional LIC and post-reconstruction filters mutually exclusive) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-43-299-8526 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

The cross checker reports that the source code provided by Technicolor who proposes JVET-N0304 has been carefully analysed and has been found to be consistent with the description in JVET-N0304. In JVET-N0304, are proposed. The cross checker could reproduce the results reported in JVET-N0304. The difference from measured encoder and decoder run time ratios to those reported in JVET-N0304 is acceptable.

Simulation Results

JVET-N0304 [1] has been implemented on VTM-4.0. The simulation was based on VTM configuration JVET-M1010 [2]. The PC cluster was used for the simulation, but since the same CPUs are allocated to the same parameters of the same sequences, the execution time is almost accurate. In order to measure more accurate decoding times, the decoder was re-run all the bitstreams on single CPU.

The results were consistent with the results of JVET-N0304. The difference from measured encoder and decoder run time ratios to those reported in JVET-N0304 is acceptable.

Table 1 Summary of experimental results (Combination LIC (CE1-5.4b) and bilateral filter)

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.96% | -0.51% | -0.51% | 129% | 101% |  |  |  |  |  |
| Class A2 | -0.72% | -0.47% | -0.41% | 117% | 102% |  |  |  |  |  |
| Class B | -0.93% | -0.47% | -0.36% | 121% | 103% | -1.14% | -0.05% | -0.02% | 128% | 104% |
| Class C | -0.68% | -0.28% | -0.40% | 126% | 103% | -1.03% | 0.17% | -0.18% | 134% | 106% |
| Class E |  |  |  |  |  | -0.34% | 0.57% | 0.34% | 103% | 103% |
| Overall | -0.82% | -0.43% | -0.41% | 123% | 102% | -0.90% | 0.18% | 0.01% | 123% | 104% |
| Class D | -0.51% | -0.04% | -0.05% | 123% | 103% | -0.71% | 0.84% | 0.32% | 129% | 106% |
| Class F | -1.09% | -0.54% | -0.65% | 114% | 102% | -3.22% | -2.70% | -2.48% | 122% | 103% |

Conclusion

In this contribution, the result of JVET-N0304 (combination LIC (CE1-5.4b) and bilateral filter) has been confirmed.

Reference

[1]	P. Bordes, F. Urban and F. Galpin, CE1-related: combined bi-directional LIC and post-reconstruction filters mutually exclusive, Joint Video Experts Team Document, JVET-N0304, Geneva, March 2019.

[2]	F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, JVET common test conditions and software reference configurations for SDR video, Joint Video Experts Team Document, JVET-M1010, Marrakesh, Jan. 2019.

Patent rights declaration(s)

Sharp Corporation does not have any current or pending patent rights relating to the technology described in this contribution.