| cJoint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0710 |
| --- | --- |

| Title: | CE13-2.6/CE13-2.7: Evaluation results of CNN based in-loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Yoshitaka Kidani Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-5066-9075 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution reports the results by the convolutional neural network-based loop filtering which was proposed in JVET-M0872. Experimental results are followed by the test conditions of Core Experiment 13 (CE13) description. Simulation results reportedly show the BD-rates for luma by -1.70%, 2.26%, -0.80% and -1.68% in CE13-2.6a (CE13-2.7a), CE13-2.6b, CE13-2.6c and CE13-2.7b, respectively. The difference is caused by the CNN based filter location and CTU-level switching.

Introduction

A convolutional neural network-based loop filtering was proposed in JVET-M0872 [1]. This contribution reports the experimental results followed by the test conditions of Core Experiment 13 (CE13) description [2]. One of the CE13 purpose is to study the location of the CNN-based filter in the existing loop filter chain. Another purpose is to study the CTU level switching of the CNN-based filter. It is noted that experiments are performed only the first intra period (without second intra picture) for reduce the experimental burden.

Overview of the proposed CNN loop filter

The filter structure is shown in Figure 1. This filter has four layers with 3x3 taps. The input of “sum” block is residual signal from the left signal and reconstructed signal just after deblocking filter. The output of “sum” block is filtered pixels. Actual output is weighted sum of after-filtered and before-filtered pixels based on the distance of edge.

Figure1. The structure of proposed CNNF

Filter coefficients are pre-defined in both the encoder and the decoder so that additional side information is not required. A representation of filter coefficients is fixed point value (integer value). The filter is applied for both intra and inter pictures.

Experimental Results

Coding condition

The proposed methods have been implemented on top of VTM-4.0 [3]. The common test conditions are used for evaluation [4] but the simulation results are conducted in only Random Access 10 condition by the short frame which is defined as first intra period in JVET-M1033 [2].

Test conditions

The test conditions consist total of 6 items as described in Table 1. CE13-2.6 is the evaluation for loop filter chain includes deblocking filter (DBF), SAO, ALF. CE13-2.7 is the evaluation for signaling of CNN. It is noted that CNN filter is adaptively switching on/off in CTU level in CE13-2.6a/b/c. It is also noted that loop filter chain of CE13-2.7a/b is same as CE13-2.6a. That means the results of CE13-2.6a and CE13-2.7a are same.

Table 1 Test conditions according to CE13 description

| CE13-2.6a | Loop filter chain | DBF + NN filter + SAO + ALF, NN filter follows the description in M0872 |
| --- | --- | --- |
| CE13-2.6b |  | NN filter only, NN filter follows the description in M0872 |
| CE13-2.6c |  | NN filter + ALF, NN filter follows the description in M0872 |
| CE13-2.7a | CTU switch on/off | NN filter adaptive on/off in CTU level |
| CE13-2.7b |  | always use NN filter for each CTU |

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Simulation results

CE13-2.6a

Table 2 shows the simulation results of CE13-2.6a.

Table 2 The simulation results of CE13-2.6a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.00% | -2.02% | -2.46% | 149% | 81109% |
| Class A2 | -1.96% | -2.50% | -3.39% | 144% | 80490% |
| Class B | -1.53% | -2.17% | -2.58% | 144% | 61736% |
| Class C | -2.24% | -4.56% | -7.71% | 122% | 44929% |
| Class E |  |  |  |  |  |
| Overall | -1.70% | -2.84% | -4.09% | 139% | 63166% |
| Class D | -3.07% | -3.83% | -8.28% | 124% | 33494% |
| Class F | -0.59% | -3.76% | -3.90% | 169% | 40262% |

    - CE13-2.6b

Table 3 shows the simulation results of CE13-2.6b.

Table 3 The simulation results of CE13-2.6b

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 4.66% | 0.69% | 0.62% | 149% | 81336% |
| Class A2 | 2.97% | 3.06% | 1.60% | 143% | 81310% |
| Class B | 1.91% | 9.73% | 7.59% | 144% | 69092% |
| Class C | 0.38% | -0.51% | -3.30% | 122% | 46360% |
| Class E |  |  |  |  |  |
| Overall | 2.26% | 3.86% | 2.09% | 138% | 66303% |
| Class D | 0.46% | -0.41% | -4.45% | 123% | 29869% |
| Class F | 1.20% | -3.05% | -3.73% | 168% | 48510% |

    - CE13-2.6c

Table 4 shows the simulation results of CE13-2.6c.

Table 4 The simulation results of CE13-2.6c

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.87% | 0.12% | 0.22% | 149% | 81711% |
| Class A2 | -1.12% | -0.52% | -2.11% | 142% | 81263% |
| Class B | -0.83% | -0.47% | -1.20% | 143% | 69595% |
| Class C | -1.76% | -3.64% | -6.80% | 122% | 46645% |
| Class E |  |  |  |  |  |
| Overall | -0.80% | -1.21% | -2.59% | 138% | 66626% |
| Class D | -2.95% | -3.27% | -7.35% | 123% | 32155% |
| Class F | 0.41% | -3.93% | -5.14% | 168% | 48866% |

    - CE13-2.7a

The simulation results are same as in CE13-2.6a.

    - CE13-2.7b

Table 5 shows the simulation results of CE13-2.7b.

Table 5 The simulation results of CE13-2.7b

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.01% | -2.02% | -2.46% | 149% | 81043% |
| Class A2 | -1.97% | -2.49% | -3.45% | 143% | 80977% |
| Class B | -1.45% | -2.06% | -2.50% | 143% | 79659% |
| Class C | -2.24% | -4.51% | -7.64% | 122% | 47698% |
| Class E |  |  |  |  |  |
| Overall | -1.68% | -2.79% | -4.05% | 138% | 69946% |
| Class D | -3.13% | -3.72% | -8.36% | 123% | 41331% |
| Class F | -0.07% | -4.02% | -4.45% | 168% | 91862% |

Conclusion

This contribution reported the experimental results followed by the test conditions of Core Experiment 13 (CE13) description [2]. Simulation results reportedly show the BD-rates for luma by -1.70%, 2.26%, -0.80% and -1.68% in CE13-2.6a (CE13-2.7a), CE13-2.6b, CE13-2.6c and CE13-2.7b, respectively. The difference is caused by the CNN based filter location and CTU-level switching.

References

- K. Kawamura and S. Naito, “AHG9: A Result of Convolutional Neural Network Filter”, JVET-M0872, Marrakech, MA, January. 2019.

- Y. Li, S. Liu, K. Kawamura, “Description of Core Experiment 13 (CE13): Neural Network based Filter for Video Coding”, JVET-M1033, Marrakech, MA, January. 2019.

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 4 (VTM 4)”, JVET-M1002, Marrakech, MA, January. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Marrakech, MA, January. 2019.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).