| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0765-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-N0106 (CE7-related: TU level limits on context-coded bins for coefficient coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 jenny.lai@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This document crosschecks the JVET-N0106 document, which proposes to limit the total number of context-coded bins for coefficient coding at TU-level. The limit is set to number of non-zero coded coefficient groups in a TU times the existing regular coded bin count limit at coefficient group level (2 bins/coefficient). The verification task was done, and the BD-rate results match exactly with those in JVET-N0106.

# Verification

Qualcomm proposed to limit the total number of context-coded bins for coefficient coding at TU-level [1]. The limit is set to number of non-zero coded coefficient groups in a TU times the existing regular coded bin count limit at coefficient group level (2 bins/coefficient).

The test was conducted under the the common test conditions (CTC) defined in [2] using the VTM4.0 software [3]. Table 1 shows the results using VTM4.0 as the anchor and enabling the proposed method in the test. The BD-rates of our experiment match exactly those in JVET-N0106. Our runtime comparison may be not accurate.

Table 1.  The results of JVET-N0106 using VTM4.0 as anchor

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.10% | -0.08% | 102% | 104% |
| Class A2 | -0.06% | 0.01% | -0.11% | 103% | 104% |
| Class B | -0.03% | -0.09% | -0.10% | 102% | 102% |
| Class C | -0.04% | -0.16% | -0.07% | 101% | 100% |
| Class E | -0.04% | 0.03% | -0.16% | 100% | 98% |
| Overall | -0.04% | -0.07% | -0.10% | 102% | 101% |
| Class D | -0.01% | -0.15% | -0.26% | 100% | 99% |
| Class F | -0.11% | -0.13% | -0.15% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.10% | -0.22% | 103% | 102% |
| Class A2 | -0.05% | -0.07% | 0.00% | 102% | 102% |
| Class B | -0.01% | -0.02% | -0.17% | 103% | 97% |
| Class C | -0.02% | -0.21% | -0.18% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.10% | -0.15% | 102% | 100% |
| Class D | -0.01% | -0.16% | -0.33% | 99% | 101% |
| Class F | -0.15% | -0.27% | -0.24% | 99% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.04% | -0.06% | 97% | 97% |
| Class C | -0.06% | 0.17% | -0.16% | 98% | 98% |
| Class E | 0.08% | 1.38% | 0.45% | 101% | 101% |
| Overall | 0.00% | 0.42% | 0.04% | 98% | 98% |
| Class D | -0.05% | 0.40% | -0.12% | 100% | 102% |
| Class F | -0.15% | -0.24% | -0.15% | 99% | 98% |

# References

- M. Coban and M. Karczewicz, “CE7-related: TU level limits on context-coded bins for coefficient coding,” Document of Joint Video Experts Team, JVET-N0106.

- F. Bossen, J. Boyce, K. Sühring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

- VTM4.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).