| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0807-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-N0172 (Non-CE6: Unification of Implicit Transform Core Selection) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Email: | hegilmez@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the crosscheck of JVET-N0172. It is reported that the average BD-rates match those provided by the proponents. The provided software and the description of the tests also match.

# Description

In this proposal, a unification for the implicit transform selection is proposed. Specifically, the transform selection in ISP and implicit MTS is unified. Two methods proposed for unification:

- Method 1 proposes to apply DST-7 if the number of samples to be transformed is less than or equal to 16, otherwise DCT-2 is used.

- Method 2 proposes to apply a ratio adaptive scheme.

The provided software include both methods and match the algorithm description in JVET-N0172.

# Experimental Results

The cross-check experiments are performed on VTM4.0 in AI configuration.

The following AI results for “Method 1” in Tables 1 and 2 match those provided by the proponents and the execution times are within the measurement error range.

Table 1: Results of Method 1 applied for ISP.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | 0.00% | 100% | 100% |
| Class A2 | -0.03% | 0.07% | -0.08% | 100% | 102% |
| Class B | -0.01% | 0.00% | 0.05% | 100% | 98% |
| Class C | 0.04% | -0.11% | -0.04% | 100% | 101% |
| Class E | -0.06% | -0.13% | -0.12% | 100% | 96% |
| Overall | -0.01% | -0.04% | -0.03% | 100% | 99% |
| Class D | 0.01% | -0.03% | -0.19% | 100% | 101% |
| Class F | 0.05% | -0.05% | -0.19% | 100% | 99% |

Table 2: Results of Method 1 applied for Implicit MTS.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 with MTS=0 and ImplicitMTS=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.40% | -0.33% | 101% | 96% |
| Class A2 | -0.12% | -0.33% | -0.42% | 101% | 98% |
| Class B | -0.12% | -0.40% | -0.36% | 101% | 96% |
| Class C | 0.05% | -0.37% | -0.16% | 101% | 95% |
| Class E | -0.18% | -0.43% | -0.53% | 101% | 99% |
| Overall | -0.11% | -0.39% | -0.35% | 101% | 96% |
| Class D | 0.04% | -0.20% | -0.12% | 101% | 94% |
| Class F | -0.02% | -0.30% | -0.10% | 100% | 96% |

The following AI results for “Method 2” in Tables 3 and 4 match those provided by the proponents. The encoding and decoding execution times are inaccurate since different types of processing cores are potentially used in different tests.

Table 3: Results of Method 2 applied for ISP.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | -0.01% | 104% | 115% |
| Class A2 | -0.01% | 0.08% | -0.05% | 103% | 115% |
| Class B | 0.02% | 0.01% | 0.06% | 102% | 113% |
| Class C | 0.06% | -0.05% | 0.16% | 80% | 94% |
| Class E | 0.06% | 0.07% | 0.03% | 81% | 94% |
| Overall | 0.02% | 0.02% | 0.05% | 94% | 106% |
| Class D | 0.05% | -0.02% | 0.01% | 81% | 96% |
| Class F | 0.01% | 0.06% | 0.08% | 86% | 96% |

Table 4: Results of Method 2 applied for Implicit MTS.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 with MTS=0 and ImplicitMTS=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | 0.11% | 0.12% | 98% | 108% |
| Class A2 | 0.05% | -0.03% | -0.07% | 93% | 110% |
| Class B | 0.00% | -0.14% | -0.13% | 99% | 112% |
| Class C | -0.01% | -0.20% | -0.06% | 78% | 92% |
| Class E | -0.10% | -0.14% | -0.12% | 83% | 98% |
| Overall | 0.02% | -0.10% | -0.06% | 90% | 104% |
| Class D | -0.03% | -0.30% | -0.09% | 82% | 92% |
| Class F | -0.06% | -0.20% | -0.27% | 84% | 91% |

# Patent rights declaration

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).