| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0065-v2 |
| --- | --- |

| Title: | CE1-1.2 and CE1-2.1: Simplification of CABAC initialization process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Dong, Amir Said, Hongtao Wang, Vadim Seregin, Marta Karczewicz | Tel: Email: | +1-408-533-9515 jiedong@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports the simulation results in CE1-1.2 and CE1-2.1 [1]. In both tests, the initial probability states represent the probability in the linear domain and can be used for the arithmetic coding engine directly. CE1-1.2 uses 6-bit initialization values, achieving BD-rate reduction -0.02% AI, -0.01% RA, 0.03% LDB, and -0.09% LDP; CE1-2.1 uses 8-bit initialization values, achieving BD-rate reduction -0.02% AI, -0.02% RA, -0.01% LDB, and -0.07% LDP.

# Introduction

The simplification of CABAC initialization process was originally proposed in JVET-N0425, in order to address the inconsistency between the legacy logarithmic domain initialization used in HEVC and the linear domain arithmetic coding engine in the current VVC. JVET-N0425 proposed to perform the initialization process in the linear domain, so that the logarithmic to linear domain mapping as well as the 256-byte table is saved. The detailed operations are showed as below, where slopeIdx and OffsetIdx are both 4-bit integers from 0 to 15 inclusive.

m = slopeIdx × 5 – 45

n = (offsetIdx << 3) + 7

preCtxState = Clip3(1, 127, ((m × (QP – 32)) >> 4) + n)

pStateIdx0 = preCtxState << 3

pStateIdx1 = preCtxState << 7

# Proposed Methods

## CE1-1.2

In CE1-1.2, JVET-N0425 was tested in 6-bit precision. In addition to restricting slopeIdx and offsetIdx to 3 bits, some constants are changed to adjust the values of m and n into the reasonable range as well as for performance and complexity considerations. The detailed operations are showed as below.

m = slopeIdx × 8 – 32

n = (offsetIdx × 18) + 1

preCtxState = Clip3(1, 127, ((m × (QP – 16)) >> 4) + n)

pStateIdx0 = preCtxState << 3

pStateIdx1 = preCtxState << 7

## CE1-2.1

This test is basically JVET-N0425 with a couple of constants refined for complexity reduction. The detailed operations are showed as below.

m = slopeIdx × 4 – 36

n = (offsetIdx << 3) + 7

preCtxState = Clip3(1, 127, ((m × (QP – 24)) >> 4) + n)

pStateIdx0 = preCtxState << 3

pStateIdx1 = preCtxState << 7

# Experimental Results

The proposed CE tests were integrated into VTM5.0 and tested under the conditions specified in CE1 description [1], including CTC and two sets of QPs at high and low QP ranges. The initialization values and window parameters were trained using the tool agreed in CE1.

## Performance under CTC

The average BD-rate reductions under CTC are shown in Table 1 for CE1-1.2 and Table 2 for CE1-2.1.

Table 1 Performance of CE1-1.2 under CTC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 - Regular QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.04% | 89% | 109% |
| Class A2 | 0.00% | 0.01% | -0.04% | 92% | 108% |
| Class B | -0.01% | 0.02% | 0.02% | 89% | 109% |
| Class C | -0.02% | -0.02% | 0.03% | 89% | 108% |
| Class E | -0.05% | -0.06% | -0.04% | 89% | 109% |
| Overall | -0.02% | -0.01% | -0.01% | 90% | 108% |
| Class D | -0.02% | 0.32% | -0.15% | 88% | 105% |
| Class F | -0.01% | 0.12% | -0.08% | 93% | 109% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 - Regular QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.11% | 101% | 110% |
| Class A2 | -0.01% | 0.34% | 0.06% | 99% | 106% |
| Class B | 0.00% | 0.20% | -0.13% | 99% | 106% |
| Class C | -0.04% | 0.16% | -0.05% | 102% | 110% |
| Overall | -0.01% | 0.18% | -0.02% | 100% | 108% |
| Class D | 0.02% | 0.29% | -0.01% | 99% | 114% |
| Class F | 0.01% | 0.05% | 0.08% | 101% | 112% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 - Regular QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.01% | 0.34% | -0.12% | 100% | 107% |
| Class C | -0.01% | 0.37% | -0.25% | 100% | 103% |
| Class E | 0.16% | -1.00% | 0.72% | 100% | 109% |
| Overall | 0.03% | 0.01% | 0.05% | 100% | 106% |
| Class D | -0.03% | 0.50% | -1.76% | 100% | 111% |
| Class F | -0.07% | -0.34% | -0.30% | 104% | 112% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-5.0 - Regular QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.05% | 0.02% | -0.31% | 99% | 105% |
| Class C | -0.07% | 0.02% | -0.15% | 99% | 103% |
| Class E | -0.16% | -0.65% | 0.35% | 103% | 110% |
| Overall | -0.09% | -0.15% | -0.09% | 100% | 106% |
| Class D | 0.05% | 0.21% | -1.66% | 100% | 104% |
| Class F | -0.12% | -0.01% | -0.11% | 104% | 109% |

Table 2 Performance of CE1-2.1 under CTC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 - Regular QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.09% | -0.08% | 101% | 97% |
| Class A2 | -0.01% | -0.01% | -0.03% | 100% | 95% |
| Class B | -0.02% | -0.07% | -0.02% | 94% | 103% |
| Class C | -0.03% | -0.04% | 0.02% | 101% | 92% |
| Class E | -0.06% | 0.01% | 0.07% | 90% | 103% |
| Overall | -0.02% | -0.04% | -0.01% | 97% | 98% |
| Class D | -0.06% | 0.12% | -0.06% | 100% | 91% |
| Class F | -0.04% | 0.04% | -0.14% | 93% | 95% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 - Regular QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.18% | 0.17% | 99% | 98% |
| Class A2 | 0.01% | 0.18% | 0.05% | 99% | 99% |
| Class B | 0.00% | 0.27% | -0.07% | 100% | 102% |
| Class C | -0.06% | 0.06% | 0.01% | 101% | 102% |
| Overall | -0.02% | 0.18% | 0.02% | 100% | 101% |
| Class D | -0.05% | 0.31% | -0.08% | 101% | 102% |
| Class F | -0.02% | 0.13% | 0.05% | 100% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 - Regular QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.01% | 0.52% | -0.10% | 99% | 98% |
| Class C | -0.10% | 0.13% | -0.09% | 99% | 96% |
| Class E | 0.12% | 0.16% | 0.29% | 92% | 95% |
| Overall | -0.01% | 0.30% | 0.00% | 97% | 97% |
| Class D | -0.09% | -0.60% | -0.29% | 103% | 103% |
| Class F | -0.15% | -0.19% | -0.41% | 103% | 99% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-5.0 - Regular QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.04% | 0.26% | -0.19% | 90% | 100% |
| Class C | -0.10% | -0.16% | 0.19% | 100% | 96% |
| Class E | -0.09% | 0.44% | 0.20% | 96% | 97% |
| Overall | -0.07% | 0.16% | 0.03% | 95% | 98% |
| Class D | -0.04% | -0.83% | -0.53% | 102% | 97% |
| Class F | -0.12% | 0.47% | -0.25% | 97% | 97% |

## Performance at High QP Range

CE1-1.2 and CE1-2.1 were tested at high QP range (42, 47, 52, 57). Note that the results in RA are mandatory and the results in AI, LDB, and LDP are supplementary.

Table 3 Performance of CE1-1.2 at high QP range

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 - high QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.16% | -0.21% | 102% | 102% |
| Class A2 | 0.02% | -0.01% | -0.38% | 101% | 102% |
| Class B | -0.05% | 0.46% | 0.09% | 101% | 104% |
| Class C | -0.06% | 0.84% | 0.44% | 102% | 100% |
| Class E | -0.27% | 0.55% | 0.29% | 98% | 99% |
| Overall | -0.06% | 0.38% | 0.07% | 101% | 102% |
| Class D | -0.10% | 1.33% | 0.89% | 99% | 107% |
| Class F | 0.91% | -0.41% | -0.48% | 100% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 - high QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.54% | 1.46% | 0.65% | 101% | 100% |
| Class A2 | 0.36% | 1.06% | 0.29% | 100% | 105% |
| Class B | 0.13% | 0.65% | 0.37% | 97% | 104% |
| Class C | -0.02% | 0.54% | 0.56% | 98% | 107% |
| Overall | 0.22% | 0.87% | 0.46% | 99% | 104% |
| Class D | 0.32% | 0.78% | 1.80% | 94% | 110% |
| Class F | -0.04% | 0.26% | 0.45% | 98% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 - high QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.09% | 0.20% | 0.28% | 100% | 105% |
| Class C | -0.34% | -1.04% | 1.31% | 100% | 110% |
| Class E | -1.35% | -0.86% | -2.16% | 95% | 103% |
| Overall | -0.49% | -0.48% | 0.02% | 99% | 106% |
| Class D | 0.46% | -0.70% | 0.32% | 95% | 104% |
| Class F | -0.89% | 0.16% | 0.07% | 101% | 106% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-5.0 - high QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.23% | 0.21% | -0.04% | 97% | 100% |
| Class C | -0.49% | -1.67% | 1.31% | 98% | 109% |
| Class E | -0.77% | -0.24% | -1.19% | 98% | 104% |
| Overall | -0.45% | -0.53% | 0.12% | 98% | 104% |
| Class D | 0.12% | 0.26% | 1.18% | 94% | 109% |
| Class F | -0.69% | -0.82% | 0.27% | 100% | 105% |

Table 4 Performance of CE1-2.1 at high QP range

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 - high QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.13% | -0.34% | 99% | 100% |
| Class A2 | -0.04% | -0.05% | -0.24% | 99% | 100% |
| Class B | -0.16% | 0.09% | -0.09% | 99% | 104% |
| Class C | -0.21% | 0.54% | 0.18% | 104% | 97% |
| Class E | -0.30% | 0.41% | -0.57% | 94% | 100% |
| Overall | -0.15% | 0.18% | -0.18% | 99% | 100% |
| Class D | -0.27% | 0.43% | 0.21% | 99% | 99% |
| Class F | 0.94% | -1.07% | -1.11% | 98% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 - high QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.26% | 2.71% | 1.37% | 98% | 95% |
| Class A2 | 1.00% | 1.91% | 0.94% | 96% | 97% |
| Class B | 0.80% | 1.34% | 1.17% | 94% | 99% |
| Class C | 0.07% | 0.45% | 0.13% | 95% | 99% |
| Overall | 0.74% | 1.49% | 0.89% | 96% | 98% |
| Class D | 0.35% | 0.93% | 0.45% | 94% | 101% |
| Class F | 0.10% | 0.40% | 0.30% | 97% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 - high QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.48% | 0.98% | -0.11% | 99% | 106% |
| Class C | -0.15% | 0.60% | -0.74% | 97% | 103% |
| Class E | -0.71% | -0.81% | -0.50% | 94% | 100% |
| Overall | -0.03% | 0.40% | -0.42% | 97% | 103% |
| Class D | 0.52% | -1.47% | 2.10% | 93% | 109% |
| Class F | -0.58% | -0.52% | 1.19% | 98% | 102% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-5.0 - high QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.83% | 1.34% | 0.34% | 96% | 99% |
| Class C | -0.06% | -1.38% | 0.29% | 98% | 101% |
| Class E | -0.90% | -0.58% | 0.32% | 97% | 100% |
| Overall | 0.10% | -0.05% | 0.32% | 97% | 100% |
| Class D | -0.04% | -0.51% | 1.77% | 94% | 104% |
| Class F | -0.35% | -0.30% | 0.71% | 96% | 99% |

## Performance at Low QP Range

CE1-1.2 and CE1-2.1 were tested at low QP range (2, 7, 12, 17). Only RA results are provided, which are mandatory.

Table 5 Performance of CE1-1.2 at low QP range

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 - low QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.08% | 0.07% | 97% | 110% |
| Class A2 | -0.01% | 0.02% | 0.02% | 100% | 102% |
| Class B | -0.01% | 0.05% | 0.04% | 101% | 105% |
| Class C | -0.03% | 0.06% | 0.01% | 100% | 106% |
| Overall | -0.01% | 0.05% | 0.04% | 100% | 105% |
| Class D | -0.03% | 0.13% | -0.11% | 102% | 114% |
| Class F | 0.15% | 0.04% | -0.02% | 100% | 111% |

Table 6 Performance of CE1-2.1 at low QP range

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 - low QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.03% | 100% | 100% |
| Class A2 | -0.02% | 0.01% | 0.01% | 101% | 101% |
| Class B | -0.01% | 0.03% | 0.02% | 101% | 100% |
| Class C | -0.04% | 0.01% | 0.00% | 100% | 100% |
| Overall | -0.02% | 0.02% | 0.02% | 100% | 100% |
| Class D | -0.08% | -0.10% | -0.13% | 101% | 105% |
| Class F | 0.23% | 0.00% | 0.01% | 99% | 104% |

# Reference

- F. Bossen, J. Dong, H. Kirchhoffer, “Description of Core Experiment 1 (CE1): CABAC Initialization,” JVET-N1021, Geneva, Switzerland, Mar. 2019.

# Appendix

The code below shows the SIMD-coded initialization loop. A set of 400 contexts is assumed (which is close to the current number in VVC).

void init(int qp, uint32_t *probs, uint8_t *states, uint8_t *rates)

{

qp -= 16;

for (int i = 0; i < 400; i += 8)

{

__m128i x = _mm_loadl_epi64((__m128i *)&states[i]);

__m128i y = _mm_loadl_epi64((__m128i *)&rates[i]);

__m128i zero = _mm_setzero_si128();

__m128i one = _mm_set1_epi16(1);

__m128i max_val = _mm_set1_epi16(127);

__m128i y0 = _mm_unpacklo_epi8(y, zero);

__m128i xl = _mm_and_si128(x, _mm_set1_epi16(0x0f0f));

xl = _mm_unpacklo_epi8(xl, zero);

__m128i xh = _mm_and_si128(x, _mm_set1_epi16(0xf0f0));

xh = _mm_unpacklo_epi8(xh, zero);

xh = _mm_srli_epi16(xh, 4);

xh = _mm_sub_epi16(xh, _mm_set1_epi16(4));

xh = _mm_mullo_epi16(xh, _mm_set1_epi16(qp));

xh = _mm_srai_epi16(xh, 1);

xl = _mm_mullo_epi16(xl, _mm_set1_epi16(18));

x = _mm_add_epi16(xh, xl);

x = _mm_add_epi16(x, _mm_set1_epi16(1));

x = _mm_min_epi16(max_val, _mm_max_epi16(one, x));

__m128i hi = _mm_add_epi16(_mm_slli_epi16(x, 9), _mm_srli_epi16(x, 6));

__m128i lo = _mm_add_epi16(_mm_slli_epi16(x, 10), y0);

__m128i y00 = _mm_unpacklo_epi16(lo, hi);

__m128i y01 = _mm_unpackhi_epi16(lo, hi);

_mm_store_si128((__m128i *)&probs[i], y00);

_mm_store_si128((__m128i *)&probs[i + 4], y01);

}

# }

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).