| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0983 |
| --- | --- |

| Title: | Crosscheck of JVET-O0532 (Non-CE5: Simplification on clipping value calculation for adaptive loop filter) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyohei Unno 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-70-4560-2752 ky-unno@kddi-research.jp |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

It is reported that cross-check results of JVET-O0532. The cross checker reports that the source code provided by Qualcomm who proposes JVET-O0532 has been analysed and has been found to be consistent with the description in JVET-O0532. In JVET-O0532, it is tested that alternative definitions of the clipping thresholds for the Non-Linear ALF. There are 3 sub-tests in JVET-O0532. The method 1 changes the definition of the clipping thresholds from power of 2 to bit-shifts. In the method 2 and 3, fixed values are used instead of the values calculated in the method 1 and the current VVC design. The cross checker could reproduce the results reported in JVET-O0532. The difference from measured encoder and decoder run time ratios to those reported in JVET-O0532 is acceptable.

Simulation Results

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Test results

JVET-O0532 [1] has been implemented on top of VTM-5.0. The simulation was based on VTM configuration JVET-N1010 [2].

The results were consistent with the results of JVET-O0532. The difference from measured encoder and decoder run time ratios to those reported in JVET-O0532 is acceptable.

Test 1

Table 1 Experimental results of Test 1 for AI condition.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.05% | 99% | 100% |
| Class A2 | 0.00% | -0.09% | -0.01% | 99% | 100% |
| Class B | 0.01% | 0.01% | -0.01% | 99% | 100% |
| Class C | 0.01% | -0.01% | -0.09% | 98% | 100% |
| Class E | 0.03% | -0.05% | -0.04% | 100% | 99% |
| Overall | 0.01% | -0.02% | -0.02% | 99% | 100% |
| Class D | 0.01% | -0.04% | -0.08% | 99% | 100% |
| Class F | 0.06% | 0.00% | -0.05% | 99% | 99% |

Table 2 Experimental results of Test 1 for RA condition.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | 0.13% | 100% | 100% |
| Class A2 | 0.00% | 0.08% | 0.15% | 100% | 100% |
| Class B | 0.01% | 0.07% | -0.10% | 100% | 100% |
| Class C | 0.01% | -0.02% | -0.18% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | -0.03% | 100% | 100% |
| Class D | 0.03% | 0.16% | 0.21% | 100% | 101% |
| Class F | 0.03% | 0.03% | -0.01% | 100% | 101% |

Table 3 Experimental results of Test 1 for LDB condition.

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.08% | -0.13% | 100% | 99% |
| Class C | 0.00% | 0.51% | 0.27% | 100% | 99% |
| Class E | 0.04% | 0.60% | -0.24% | 100% | 100% |
| Overall | 0.00% | 0.35% | -0.02% | 100% | 99% |
| Class D | 0.02% | 0.30% | 0.36% | 100% | 100% |
| Class F | 0.03% | 0.43% | -0.26% | 100% | 100% |

Test 2

Table 4 Experimental results of Test 2 for AI condition.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.05% | 100% | 100% |
| Class A2 | -0.01% | -0.09% | -0.01% | 100% | 99% |
| Class B | 0.01% | 0.01% | -0.01% | 100% | 101% |
| Class C | 0.02% | -0.02% | -0.10% | 100% | 100% |
| Class E | 0.03% | -0.05% | -0.04% | 100% | 100% |
| Overall | 0.01% | -0.02% | -0.03% | 100% | 100% |
| Class D | 0.02% | -0.05% | -0.09% | 100% | 102% |
| Class F | 0.05% | -0.01% | -0.05% | 99% | 99% |

Table 5 Experimental results of Test 2 for RA condition.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.12% | 0.02% | 100% | 100% |
| Class A2 | 0.01% | -0.04% | 0.05% | 100% | 100% |
| Class B | 0.02% | 0.04% | -0.14% | 100% | 100% |
| Class C | -0.02% | -0.02% | -0.18% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.03% | -0.08% | 100% | 100% |
| Class D | 0.03% | -0.03% | 0.05% | 100% | 100% |
| Class F | 0.05% | 0.11% | -0.03% | 100% | 101% |

Table 6 Experimental results of Test 2 for LDB condition.

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.08% | 0.07% | 100% | 99% |
| Class C | 0.02% | 0.27% | 0.19% | 100% | 99% |
| Class E | 0.07% | -0.20% | -0.68% | 100% | 100% |
| Overall | 0.03% | 0.01% | -0.08% | 100% | 99% |
| Class D | 0.08% | 0.11% | 1.12% | 100% | 100% |
| Class F | -0.01% | 0.99% | 0.21% | 100% | 100% |

Test 3

Table 7 Experimental results of Test 3 for AI condition.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.22% | 0.16% | 100% | 101% |
| Class A2 | 0.00% | 0.05% | 0.04% | 100% | 100% |
| Class B | 0.00% | 0.12% | 0.15% | 100% | 100% |
| Class C | -0.01% | 0.12% | 0.11% | 99% | 100% |
| Class E | 0.01% | -0.10% | 0.03% | 99% | 100% |
| Overall | 0.00% | 0.09% | 0.10% | 99% | 100% |
| Class D | -0.01% | 0.19% | 0.08% | 99% | 100% |
| Class F | -0.01% | 0.21% | 0.38% | 99% | 100% |

Table 8 Experimental results of Test 3 for RA condition.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.38% | 0.42% | 99% | 100% |
| Class A2 | 0.06% | 1.07% | 0.52% | 99% | 99% |
| Class B | 0.03% | 0.41% | 0.20% | 99% | 100% |
| Class C | 0.02% | 0.21% | 0.13% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.48% | 0.29% | 99% | 100% |
| Class D | 0.00% | 0.19% | 0.10% | 99% | 98% |
| Class F | 0.03% | 0.30% | 0.28% | 99% | 99% |

Table 9 Experimental results of Test 3 for LDB condition.

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 1.21% | 1.39% | 99% | 100% |
| Class C | 0.07% | 0.84% | 0.59% | 100% | 101% |
| Class E | -0.05% | 0.34% | 0.35% | 99% | 99% |
| Overall | 0.02% | 0.87% | 0.86% | 99% | 100% |
| Class D | 0.01% | 0.75% | 0.88% | 99% | 100% |
| Class F | 0.00% | 0.40% | 0.05% | 99% | 99% |

Conclusion

In this contribution, the result of JVET-O0532 has been confirmed.

References

- N. Hu, V. Seregin and M. Karczewicz, “Non-CE5: Simplification on clipping value calculation for adaptive loop filter”, JVET-O0532, Gothenburg, SE, July 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, Mar. 2019.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).