| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1064-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0550 (CE2-related: Further simplification of chroma residual scaling factors derivation) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 shih-ta.hsiang@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This document reports the results of crosschecking JVET-O0550 proposed by Huawei. JVET-O0550 proposes three simplified methods for derivation of chroma residual scaling factors to reduce the latency for the chroma residual scaling. The verification task was mostly done. Our partial BD bitrate results of Method 1 match exactly with the corresponding results in JVET-O0550. However, mismatched experimental results between encoding and decoding are observed for Methods 2 and 3.

# Verification

Huawei provided the source code of their proposed method described in JVET-O0550 [1].

JVET-O0550 explored three aspects in simplification of derivation of chroma residual scaling factors to reduce the latency for the chroma residual scaling. The first aspect related to using luma DC prediction value instead of using an average value of luma samples for chroma scaling factor derivation. The second aspect about using shifted reference line with offset depended on directional intra prediction angle. Third aspect related to pipeline latency reduction in Inter processing by using reference samples corresponded to displacement with integer motion vector of current block.

Three verification tests were carried out for each of tested aspects using the provided software. The tests are conducted under the common test conditions (CTC) defined in [3]. Tables 1, 2, and 3 show the results of using VTM5.0 as the anchor and enabling the proposed methods using aspects 1, 2 and 3, respectively.  Our partial results of Test 1 match exactly the corresponding results in JVET-O0550. However, mismatched results between encoding and decoding are observed for Test 2 and Test 3.

Table 1. The results of Test 1 using Luma DC prediction

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.13% | 0.02% | 100% | 98% |
| Class A2 | 0.02% | -0.13% | -0.07% | 99% | 99% |
| Class B | 0.02% | -0.08% | -0.15% | 100% | 97% |
| Class C | 0.00% | 0.05% | -0.14% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.06% | -0.10% | 99% | 98% |
| Class D | 0.01% | -0.10% | 0.03% | 99% | 100% |
| Class F | 0.00% | 0.01% | 0.01% | 99% | 96% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.35% | 0.45% | 0.48% | 52% | 77% |
| Class C | -0.02% | 0.09% | -0.23% | 100% | 100% |
| Class E | 0.05% | 0.44% | -0.22% | 99% | 97% |
| Overall | 0.15% | 0.33% | 0.07% | 76% | 89% |
| Class D | 0.02% | 0.40% | 0.61% | 98% | 98% |
| Class F | -0.02% | -0.05% | -0.03% | 97% | 97% |

Table 2. The results of Test 2 using Directional Intra modes

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.13% | -0.02% | 100% | 109% |
| Class A2 | 0.02% | 0.00% | -0.08% | 100% | 110% |
| Class B | 0.02% | -0.06% | -0.11% | 100% | 99% |
| Class C | -0.02% | 0.13% | -0.10% | 99% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | -0.08% | 100% | 102% |
| Class D | 0.00% | 0.08% | 0.12% | 99% | 101% |
| Class F | 0.00% | 0.04% | 0.00% | 100% | 94% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.08% | -0.05% | 99% | 100% |
| Class C | 0.01% | 0.08% | -0.01% | 100% | 99% |
| Class E | 0.02% | 0.43% | -0.15% | 98% | 97% |
| Overall | 0.00% | 0.17% | -0.06% | 99% | 99% |
| Class D | 0.01% | -0.02% | 0.57% | 97% | 98% |
| Class F | 0.03% | -0.05% | -0.04% | 97% | 97% |

Table 3. The results of Test 3 using reference from Inter modes

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.72% | -2.39% | -1.79% | 96% | #NUM! |
| Class A2 | 0.64% | 0.66% | 0.74% | 98% | #NUM! |
| Class B | 216.82% | 122.20% | 131.14% | 53% | #NUM! |
| Class C | 0.30% | 0.52% | 0.73% | 97% | #NUM! |
| Class E |  |  |  |  |  |
| Overall | 72.14% | 40.53% | 43.70% | 79% | #NUM! |
| Class D | 0.59% | 0.76% | 0.64% | 98% | #NUM! |
| Class F | 0.01% | 0.04% | -0.02% | 99% | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.21% | -0.91% | 0.07% | #NUM! | 94% |
| Class C | 0.05% | 0.15% | -0.08% | 99% | 99% |
| Class E | 0.06% | 0.43% | -0.13% | 99% | 99% |
| Overall | -0.06% | -0.22% | -0.03% | #NUM! | 97% |
| Class D | 0.02% | -0.19% | 0.45% | 97% | 98% |
| Class F | -0.03% | 0.13% | 0.05% | 97% | 97% |

# References

[1] M. Sychev and E. Alshina, “CE2-related: Further simplification of chroma residual scaling factors derivation,” Document of Joint Video Experts Team, JVET-O0550, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

[3] VTM5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).