        - Specification of INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra prediction mode

Inputs to this process are:

- the intra prediction mode predModeIntra,

- a sample location ( xTbC, yTbC ) of the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block,

- chroma neighbouring samples p[ x ][ y ], with x = −1, y = 0..2 * nTbH − 1 and x = 0.. 2 * nTbW − 1, y = − 1.

Output of this process are predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The current luma location ( xTbY, yTbY ) is derived as follows:

( xTbY, yTbY )  =  ( xTbC << ( SubWidthC − 1 ), yTbC << ( SubHeightC − 1 ) )	(8-165)

-

-

-

-

-

The variable bCTUboundary is derived as follows:

bCTUboundary = ( yTbC & ( 1 << ( CtbLog2SizeY − 1 ) − 1 )  = =  0 )  ?  TRUE  :  FALSE. 	(8-171)

When predModeIntra is equal to INTRA_LT_CCLM	-, the following applies:

- The variables numSampT and numSampL are set equal to nTbW and nTbH, respectively.

- The variables startPosT and startPosL are set equal to (nTbW >> 2) and (nTbH >> 2), respectively.

- The variables pickStepT and pickStepL are set equal to (nTbW  >> 1) and (nTbH  >> 1), respectively.

- The variables cntT and cntL are set equal to 2.

Otherwise, if predModeIntra is equal to INTRA_L_CCLM, the following applies:

numSampL is set equal to (nTbH + Min(nTbW, nTbH)).

startPosL is set equal to (nTbH + Min(nTbW, nTbH)) >>  3.

pickStepL is set equal to (nTbH + Min(nTbW, nTbH)) >> 2.

cntL is set equal to 4.

Otherwise, if predModeIntra is equal to INTRA_T_CCLM, the following applies:

numSampT is set equal to (nTbW + Min(nTbW, nTbH)).

startPosT is set equal to (nTbW + Min(nTbW, nTbH)) >>  3.

pickStepT is set equal to (nTbW + Min(nTbW, nTbH)) >> 2.

cntT is set equal to 4.

The variable picPosN[pos] with pos = 0.. cntN – 1 and N being replaced by L and T are derived as follows:

pickPosN[ pos ] = (startPosN + pos * pickStepN)

The prediction samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

-

  - The collocated luma samples pY[ x ][ y ] with x = 0..nTbW * SubWidthC − 1, y= 0..nTbH * SubHeightC − 1 are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY + x, yTbY + y ).

  - For the generation of the neighbouring luma samples pY[ x ][ y ]

  - with x = −3.. −1, y = −3.. SubHeightC * numSampL − 1 and x = 0.. SubWidthC * numSampT − 1, y = −3.. −1, the following applies:

    - The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the sample location (xCurr, yCurr) set equal to

    - ( xTbC, yTbC ), the neighbouring chroma location ( xTbC + x, yTbC + y), checkPredModeY set equal to FALSE, the colour component index 1 as inputs, and the

    - availability of the neighbouring chroma samples availC[ x ][ y ] with x = −1, y = −1..2 * nTbH − 1 and x = 0..2 * nTbW − 1, y = −1 as output.

    - The availablity of the neighbouring luma samples availY[ x ][ y ] with x = −1, y = −1.. 2 * SubHeightC * nTbH − 1 and x = 0.. 2 * SubWidthC * nTbW − 1, y = −1 are derived as follows:

      - availY[ −1 ][ −1 ] is set equal to availC[ −1 ][ −1 ].

      - availY[ x ][ y ] with x = −1, y = 0.. 2 * SubHeightC * nTbH − 1 is derived as follows:

availY[ x ][ y * SubHeightC ] = availC[ x ][ y ]

If SubHeightC is equal to 2, the following applies:

availY[ x ][ y * SubHeightC + 1 ] = availC[ x ][ y ]

      - availY[ x ][ y ] with x = 0.. 2 * SubWidthC * nTbW − 1, y = −1 is derived as follows:

availY[ x * SubWidthC ][ y ] = availC[ x ][ y ]

If SubWidthC is equal to 2, the following applies:

availY[ x * SubWidthC + 1][ y ] = availC[ x ][ y ]

    - For the reference line index refIdx with 0..2, when at least one sample availY[ x ][ y ] with x = −1, y = −1.. SubHeightC * numSampL − 1 and x = 0.. SubWidthC * numSampT − 1, y = −1 is equal to FALSE, the reference sample substitution process as specified in clause 8.4.5.2.8 is invoked with the intra prediction reference line index refIdx, the reference sample width (SubWidth * numSampT), the reference sample height (SubHeightC * numSampL), the reference samples availY[ x ][ y ] with x = −1, y = −1.. SubHeightC * numSampL − 1 and x = 0.. SubWidthC * numSampT − 1, y = −1, and the colour component index 0 as inputs, and the neighbouring luma samples pY[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx.. (SubHeightC * numSampL) and x = −refIdx.. (SubWidthC  * numSampT), y = −1 − refIdx as output.

  - The down-sampled collocated luma samples pDsY[ x ][ y ] with x = 0..nTbW − 1,  y = 0..nTbH − 1 are derived as follows:

    - If both SubWidthC and SubHeightC are equal to 1, the following applies:

      - pDsY[ x ][ y ] with x = 1..nTbW − 1, y = 1..nTbH − 1 is derived as follows:

pDstY[ x ][ y ] = pY[ x ][ y ]				(8-174)

    - Otherwise, the following applies:

      - The one-dimensional filter coefficients array F1 and F2, and the 2-dimensional filter coefficients arrays F3 and F4 are specified as follows.

-

F2[ 0 ] = 1, F2[ 1 ] = 2, F2[ 2 ] = 1	(8-176)

F3[ i ][ j ] = F4[ i ][ j ] = 0, with i = 0..2, j = 0..2	(8-177)

        - If both SubWidthC and SubHeightC are equal to 2, the following applies:

-

F3[ 0 ][ 1 ] = 1, F3[ 1 ][ 1 ] = 4, F2[ 2 ][ 1 ] = 1, F3[ 1 ][ 0 ] = 1, F3[ 1 ][ 2 ] = 1	(8-179)

F4[ 0 ][ 1 ]  = 1, F4[ 1 ][ 1 ] = 2, F4[ 2 ][ 1 ] = 1	(8-180)

F4[ 0 ][ 2 ] = 1,  F4[ 1 ][ 2 ] = 2, F4[ 2 ][ 2 ] = 1	(8-181)

        - Otherwise, the following applies:

-

F3[ 1 ][ 1 ] = 8					(8-183)

F4[ 0 ][ 1 ] = 2, F4[ 1 ][ 1 ] = 4, F4[ 2 ][ 1 ] = 2,	(8-184)

      - If sps_cclm_colocated_chroma_flag is equal to 1, the following applies:

        - pDsY[ x ][ y ] with x = 1..nTbW − 1, y = 1..nTbH − 1 is derived as follows:

pDsY[ x ][ y ] = ( F3[ 1 ][ 0 ] * pY[ SubWidthC * x ][ SubHeightC * y − 1 ] +
		F3[ 0 ][ 1 ] * pY[ SubWidthC * x − 1 ][ SubHeightC * y ] +
		F3[ 1 ][ 1 ] * pY[ SubWidthC * x ][ SubHeightC * y ] +	(8-185)
		F3[ 2 ][ 1 ] * pY[ SubWidthC * x + 1 ][ SubHeightC * y ] +
		F3[ 1 ][ 2 ] * pY[ SubWidthC * x ][ SubHeightC * y + 1 ] + 4 ) >> 3

-

-

-

-

-

-

-

-

      - Otherwise (sps_cclm_colocated_chroma_flag is equal to 0), the following applies:

        - pDsY[ x ][ y ] with x = 1..nTbW − 1, y = 0..nTbH − 1 is derived as follows:

pDsY[ x ][ y ] = ( F4[ 0 ][ 1 ] * pY[ SubWidthC * x − 1 ][ SubHeightC * y ] +
		F4[ 0 ][ 2 ] * pY[ SubWidthC * x − 1 ][ SubHeightC * y + 1 ] +
		F4[ 1 ][ 1 ] * pY[ SubWidthC * x ][ SubHeightC * y ] +	(8-194)
		F4[ 1 ][ 2 ] * pY[ SubWidthC * x ][ SubHeightC * y + 1] +
		F4[ 2 ][ 1 ] * pY[ SubWidthC * x + 1 ][ SubHeightC * y ] +
		F4[ 2 ][ 2 ] * pY[ SubWidthC * x + 1][ SubHeightC * y + 1 ] + 4 ) >> 3

-

-

  - When cntL is greater than 0, the selected neighbouring left chroma samples pSelC[ idx ] are set equal to p[ −1 ][ pickPosL[ idx ] ] with idx = 0..cntL − 1, and the selected down-sampled neighbouring left luma samples pSelDsY[ idx ] with idx = 0..cntL − 1 are derived as follows:

    - The variable y is set equal to pickPosL[ idx ].

    - If both SubWidthC and SubHeightC are equal to 1, the following applies:

pSelDsY[ idx ] = pY[ − 1][ y ]			(8-197)

    - Otherwise the following applies:

      - If sps_cclm_colocated_chroma_flag is equal to 1, the following applies:

pSelDsY[ idx ] = ( F3[ 1 ][ 0 ] * pY[ − SubWidthC ][ SubHeightC * y − 1 ] +
		F3[ 0 ][ 1 ] * pY[ −1 − SubWidthC ][ SubHeightC * y ] +
		F3[ 1 ][ 1 ] * pY[ −SubWidthC ][ SubHeightC * y ]  + 	(8-198)
		F3[ 2 ][ 1 ] * pY[ 1 − SubWidthC ][ SubHeightC * y ] +
		F3[ 1 ][ 2 ] * pY[ −SubWidthC ][ SubHeightC * y + 1 ] + 4 ) >> 3

-

      - Otherwise (sps_cclm_colocated_chroma_flag is equal to 0), the following applies:

pSelDsY[ idx ] = ( F4[ 0 ][ 1 ] * pY[ −1 − SubWidthC ][ SubHeightC * y ] +
		F4[ 0 ][ 2 ] * pY[ −1 − SubWidthC ][ SubHeightC * y + 1 ] +
		F4[ 1 ][ 1 ] * pY[ −SubWidthC ][ SubHeightC * y ] +	(8-200)
		F4[ 1 ][ 2 ] * pY[ −SubWidthC ][ SubHeightC * y + 1] +
		F4[ 2 ][ 1 ] * pY[ 1 − SubWidthC ][ SubHeightC * y ] +
		F4[ 2 ][ 2 ] * pY[ 1 − SubWidthC][ SubHeightC * y + 1 ] + 4 ) >> 3

  - When cntT is greater than 0, the selected neighbouring top chroma samples pSelC[ idx ] are set equal to p[ pickPosT[ idx − cntL ] ][ -1 ] with  idx = cntL..cntL + cntT − 1, and the down-sampled neighbouring top luma samples pSelDsY[ idx ] with idx = 0..cntL + cntT − 1 are specified as follows:

    - The variable x is set equal to pickPosT[ idx − cntL ].

    - If both SubWidthC and SubHeightC are equal to 1, the following applies:

pSelDsY[ idx ] = pY[ x ][ − 1]			(8-201)

    - Otherwise, the following applies:

          - If bCTUboundary is equal to

-

    - TRUE, the following applies:

pSelDsY[ idx ] = ( F2[ 0 ] * pY[ SubWidthC * x − 1 ][ −1 ] +
		F2[ 1 ] * pY[ SubWidthC * x ][ −1 ]  +	(8-203)
		F2[ 2 ] * pY[ SubWidthC * x + 1 ][ −1 ] + 2 ) >> 2

    - Otherwise (bCTUboundary is equal to FALSE), the following applies:

      - If sps_cclm_colocated_chroma_flag is equal to 1, the following applies:

pSelDsY[ idx ] = ( F3[ 1 ][ 0 ] * pY[ −1 ][ − 1 − SubHeightC ] +
		F3[ 0 ][ 1 ] * pY[ −1 ][ −SubHeightC ] +
		F3[ 1 ][ 1 ] * pY[ 0 ][ −SubHeightC ] +	(8-204)
		F3[ 2 ][ 1 ] * pY[ 1 ][ −SubHeightC ] +
		F3[ 1 ][ 2 ] * pY[ −1 ][ 1 − SubHeightC ] + 4 ) >> 3

-

-

-

      - Otherwise (sps_cclm_colocated_chroma_flag is equal to 0), the following applies:

pSelDsY[ idx ] = ( F4[ 0 ][ 1 ] * pY[ SubWidthC x − 1 ][ −2 ] +
		F4[ 0 ][ 2 ] * pY[ SubWidthC * x − 1 ][ −1 ] +
		F4[ 1 ][ 1 ] * pY[ SubWidthC * x ][ −2 ] +	(8-208)
		F4[ 1 ][ 2 ] * pY[ SubWidthC * x ][ −1] +
		F4[ 2 ][ 1 ] * pY[ SubWidthC * x + 1 ][ −2 ] +
		F4[ 2 ][ 2 ] * pY[ SubWidthC * x + 1 ][ −1 ] + 4 ) >> 3

-

-

-

-

-

  - The variables minY, maxY, minC and maxC are derived as follows:

    - The arrays minGrpIdx and maxGrpIdx are derived as follows:

minGrpIdx[ 0 ] = 0		(8-214)

minGrpIdx[ 1 ] = 2		(8-215)

maxGrpIdx[ 0 ] = 1		(8-216)

maxGrpIdx[ 1 ] = 3		(8-217)

    - When pSelDsY[ minGrpIdx[ 0 ] ] is greater than pSelDsY[ minGrpIdx[ 1 ] ], minGrpIdx[ 0 ] and minGrpIdx[ 1 ] are swapped as follows:

( minGrpIdx[ 0 ], minGrpIdx[ 1 ] ) = Swap( minGrpIdx[ 0 ], minGrpIdx[ 1 ] )	(8-218)

    - When pSelDsY[ maxGrpIdx[ 0 ] ] is greater than pSelDsY[ maxGrpIdx[ 1 ] ], maxGrpIdx[ 0 ] and maxGrpIdx[ 1 ] are swapped as follows:

( maxGrpIdx[ 0 ], maxGrpIdx[ 1 ] ) = Swap( maxGrpIdx[ 0 ], maxGrpIdx[ 1 ] )	(8-219)

    - When pSelDsY[ minGrpIdx[ 0 ] ] is greater than pSelDsY[ maxGrpIdx[ 1 ] ], arrays minGrpIdx and maxGrpIdx are swapped as follows:

( minGrpIdx, maxGrpIdx ) = Swap( minGrpIdx, maxGrpIdx )	(8-220)

    - When pSelDsY[ minGrpIdx[ 1 ] ] is greater than pSelDsY[ maxGrpIdx[ 0 ] ], minGrpIdx[ 1 ] and maxGrpIdx[ 0 ] are swapped as follows:

( minGrpIdx[ 1 ], maxGrpIdx[ 0 ] ) = Swap( minGrpIdx[ 1 ], maxGrpIdx[ 0 ] )	(8-221)

    - The variables maxY, maxC, minY and minC are derived as follows:

maxY = ( pSelDsY[ maxGrpIdx[ 0 ] ] + pSelDsY[ maxGrpIdx[ 1 ] ] + 1 )  >>  1	(8-222)

maxC = ( pSelC[ maxGrpIdx[ 0 ] ] + pSelC[ maxGrpIdx[ 1 ] ] + 1 )  >>  1	(8-223)

minY = ( pSelDsY[ minGrpIdx[ 0 ] ] + pSelDsY[ minGrpIdx[ 1 ] ] + 1 )  >>  1	(8-224)

minC = ( pSelC[ minGrpIdx[ 0 ] ] + pSelC[ minGrpIdx[ 1 ] ] + 1 )  >>  1	(8-225)

  - The variables a, b, and k are derived as follows:

-

-

-

diff = maxY − minY		(8-229)

    - If diff is not equal to 0, the following applies:

diffC = maxC − minC	(8-230)

x = Floor( Log2( diff ) )	(8-231)

normDiff = ( ( diff << 4 ) >> x ) & 15	(8-232)

x += ( normDiff  !=  0 )  ?  1  :  0	(8-233)

y = Floor( Log2( Abs ( diffC ) ) ) + 1	(8-234)

a = ( diffC * ( divSigTable[ normDiff ] | 8 ) + 2y − 1 ) >> y	(8-235)

k = ( ( 3 + x − y ) < 1 )  ?  1  :  3 + x − y	(8-236)

a = ( ( 3 + x − y ) < 1 )  ?  Sign( a ) * 15  :  a	(8-237)

b = minC − ( ( a * minY ) >> k )	(8-238)

where divSigTable[ ] is specified as follows:

divSigTable[ ] = { 0, 7, 6, 5, 5, 4, 4, 3, 3, 2, 2, 1, 1, 1, 1, 0 }	(8-239)

    - Otherwise (diff is equal to 0), the following applies:

k = 0	(8-240)

a = 0	(8-241)

b = minC	(8-242)

  - The prediction samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0.. nTbH − 1 are derived as follows:

predSamples[ x ][ y ] = Clip1C( ( ( pDsY[ x ][ y ] * a ) >> k ) + b )	(8-243)