| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0485 |
| --- | --- |

| Title: | AHG18: Performance of CE7-1.2d and CE7-2.2 on lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Alican Nalci, Hongtao Wang, Muhammed Coban, Marta Karczewicz | Tel: Email: | +1-858-8452163 yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the transform skip modification proposed in CE7-1.2d-alternative and the transform skip binarization proposed in CE7-2.2a-alternative for lossless coding. Two test scenarios are tested: In the first scenario, the anchor base uses only regular transform residaul coding (TR) for lossless coding with BDPCM support in luma and the test base uses only transform skip (TS) residual coding with BDPCM support in luma and chroma. In the 2nd scenario, the anchor base is AHG18 lossless branch and the test base is the method 1 in JVET-P0082, which enables transform (TR) residual coding, transform skip (TS) residual coding,and BDPCM for lossless coding. The results show:

- 1st test scenario (TR only v.s. TS only)

  - CE7-1.2d-alternative:

Overall:    -1.64% AI, 0.40% RA, xxx% LB

Class F:    -2.92% AI, -1.39% RA, -2.37% LB

Class TGM: -7.03% AI, -9.45% RA, xxx% LB

- 2nd test scenario (AHG18 v.s. TS + TR)

  - CE7-1.2d-alternative:

Overall:    -2.72% AI, -1.28% RA, -1.51% LB

  - CE7-2.2a-alternative:

Overall:    -3.30% AI, -1.69% RA, -1.92% LB

# Proposed Method

## Method 1

In CE7-1.2d [1], a method of Rice Golomb coding is applied to encode the transform skip residuals after the context coded bin limit is reached. If there’s no context bin that can be used for any of the component of a coefficient, the abs(level) is coded using Rice-Golomb coding and followed by a bypass coded sign flag. The rice parameter is derived as follows for this case:

auiGoRicePars[std::min(sum, 31)]

Otherwise, if one or more bins for a coefficient in the 1st pass is context coded but there’s no context bin for any component in for the coefficient in the 2nd pass. abs(level) – 1 is coded using Rice-Golomb coding. The rice parameter is derived as follows for this case:

auiGoRicePars[std::min(sum + 4, 31)]

If one or more bins for a coeffcient in the 2nd path is context coded, the remainder (if gt9 flag is 1) is coded using Rice-Golomb coding, where the rice parameter is derived as follows:

auiGoRicePars[ std::min(std::max(sum -8, 0), 31) ]

## Method 2

In CE7-2.2a [2], a new binarization is proposed for transform skip residual coding, where the binarization is adjusted depending on the dynamic range of the coefficient. The binarization is also tested on top of the Rice-Golomb coding after bypass switch proposed in CE7-1.2d-alternative (CE7-2.2a-alternative).

Simulation Results

## Test scenario 1

In the first scenario, the anchor base uses only regular transform residaul coding (TR) for lossless coding with BDPCM support in luma and the test base uses only transform skip (TS) residual coding with BDPCM support in luma and chroma.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | TR ONLY | TS ONLY+CE7-1.2d* |  | TR ONLY | TS ONLY+CE7-1.2d* |  | TR ONLY | TS ONLY+CE7-1.2d* |  |
| Class A1 | 2.2 | 2.2 | -0.01% | 2.3 | 2.3 | 1.45% |  |  |  |
| Class A2 | 1.8 | 1.8 | 0.54% | 1.9 | 1.8 | 2.03% |  |  |  |
| Class B | 2.1 | 2.1 | -0.21% | 2.3 | 2.2 | 2.64% | 2.3 | 2.3 | -0.02% |
| Class C | 2.1 | 2.1 | -0.27% | 2.6 | 2.5 | 4.11% | 2.6 | 2.6 | -0.04% |
| Class D | 2.0 | 2.0 | -1.45% | 2.8 | 2.7 | 3.50% | 2.8 | 2.8 | -0.03% |
| Class E | 3.0 | 3.0 | -1.04% |  |  |  | 3.4 | 3.4 | 0.01% |
| Class F | 5.7 | 5.9 | -2.92% | 40.2 | 42.2 | -1.39% | 61.2 | 64.4 | -2.37% |
| TGM | 12.5 | 13.5 | -7.03% | 113.4 | 127.7 | -9.45% | 142.8 | 155.1 | -35.94% |
| All (A1, A2, B, C, E) | 2.2 | 2.2 | -0.20% | 2.3 | 2.2 | 2.67% | 2.7 | 2.7 | -0.02% |
| Overall | 4.1 | 4.3 | -1.64% | 25.3 | 27.8 | 0.40% | 35.8 | 31.9 | -4.90% |
| Enc Time[%] | 109% |  |  | 111% |  |  | #NUM! |  |  |
| Dec Time[%] | 103% |  |  | 106% |  |  | #NUM! |  |  |

## Test scenario 2

In the simulation, the anchor is AHG18 VTM-6.0 Lossless branch. The following settings are used to run both the anchor and test: TransquantBypassEnable : 1, CUTransquantBypassFlagForce : 1, CostMode : lossless, InternalBitDepth : 0. QP value was set as 0. Additionally, BDPCM is set to be 1 for the test. Table 1 and 2 shows the results of method 1 and 2.

Table 1: Results of method 1 (CE7-1.2d-alternative on lossless coding)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal |  | AHG18 LL | Proposal |  | AHG18 LL | Proposal |  |
| Class A1 | 2.2 | 2.3 | -1.03% | 2.3 | 2.3 | -0.42% |  |  |  |
| Class A2 | 1.7 | 1.8 | -1.74% | 1.9 | 1.9 | -0.42% |  |  |  |
| Class B | 2.1 | 2.1 | -1.57% | 2.3 | 2.3 | -0.29% | 2.3 | 2.3 | -0.32% |
| Class C | 2.1 | 2.1 | -1.81% | 2.6 | 2.6 | -0.38% | 2.6 | 2.6 | -0.40% |
| Class D | 1.9 | 2.0 | -3.21% | 2.8 | 2.8 | -0.62% | 2.7 | 2.8 | -0.53% |
| Class E | 2.9 | 3.0 | -2.89% |  |  |  | 3.3 | 3.4 | -1.01% |
| Class F | 5.5 | 5.8 | -4.51% | 39.4 | 41.5 | -3.10% | 60.7 | 64.6 | -3.36% |
| TGM | 12.3 | 12.9 | -4.66% | 112.6 | 116.7 | -3.52% | 142.4 | 148.1 | -3.61% |
| Overall (A1, A2, B, C, E) | 2.2 | 2.2 | -1.78% | 2.3 | 2.3 | -0.37% | 2.6 | 2.7 | -0.52% |
| Overall | 4.0 | 4.2 | -2.72% | 25.1 | 26.0 | -1.28% | 35.6 | 37.2 | -1.51% |
| Enc Time[%] | 108% |  |  | 109% |  |  | 113% |  |  |
| Dec Time[%] | 94% |  |  | 100% |  |  | 101% |  |  |

Table 2: Results of method 1 (CE7-2.2a-alternative on lossless coding)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal |  | AHG18 LL | Proposal |  | AHG18 LL | Proposal |  |
| Class A1 | 2.2 | 2.3 | -1.00% | 2.3 | 2.3 | -0.36% |  |  |  |
| Class A2 | 1.7 | 1.8 | -1.73% | 1.9 | 1.9 | -0.37% |  |  |  |
| Class B | 2.1 | 2.1 | -1.62% | 2.3 | 2.3 | -0.29% | 2.3 | 2.3 | -0.31% |
| Class C | 2.1 | 2.1 | -1.73% | 2.6 | 2.6 | -0.34% | 2.6 | 2.6 | -0.36% |
| Class D | 1.9 | 2.0 | -3.10% | 2.8 | 2.8 | -0.56% | 2.7 | 2.8 | -0.47% |
| Class E | 2.9 | 3.0 | -2.73% |  |  |  | 3.3 | 3.4 | -0.99% |
| Class F | 5.5 | 5.8 | -5.01% | 39.4 | 42.0 | -3.46% | 60.7 | 65.0 | -3.55% |
| TGM | 12.3 | 13.5 | -8.80% | 112.6 | 120.7 | -6.14% | 142.4 | 153.2 | -6.01% |
| Overall (A1, A2, B, C, E) | 2.2 | 2.2 | -1.75% | 2.3 | 2.3 | -0.33% | 2.6 | 2.7 | -0.50% |
| Overall | 4.0 | 4.3 | -3.30% | 25.1 | 26.7 | -1.69% | 35.6 | 38.2 | -1.92% |
| Enc Time[%] | 110% |  |  | 111% |  |  | 115% |  |  |
| Dec Time[%] | 96% |  |  | 100% |  |  | 101% |  |  |

# Reference

- M. Karczewicz, H. Wang, Y.-H. Chao, M. Coban, “CE7: Modifications to transform-skip residual coding (CE7-1.2)”, JVET-P0079, 15th meeting, Gothenburg, SE, July 2019.

- M. Karczewicz, H. Wang, Y.-H. Chao, M. Coban, “CE7-2.2: QP dependent binarization in TS residual coding”, JVET-P0076, 15th meeting, Gothenburg, SE, July 2019.

- A. Nalci, H. Wang, H. E. Egilmez, M. Coban, M. Karczewicz, “AHG18: BDPCM for Lossless”, JVET-P0082, 15th meeting, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)