| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0757-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-P0525 Test2 (AHG18: Residual coding selection signaling for lossless VVC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck of a Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 Takeshi.Tsukuba@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution reports the cross-check results of the JVET-P0525 Test2(AHG18: Residual coding selection signaling for lossless VVC), which signals 1 bit syntax by reusing the transform_skip_flag in TB level to indicate the selection between residual coding and transform coefficient coding scheme for non-BDPCM modes to improve the coding efficiency for lossless coding. The source code provided by the proponent was carefully analyzed and found to be consistent with the description in JVET-P0525 Test2. The cross-checker’s results matched with the results reported in JVET-P0525.

Note: Some results are not finished and will be updated in a revision of the document.

Simulation Results

Anchor is AHG18 VTM-6.0-lossless branch. The following setting are used to run both the anchor and tested: TransquantBypassEnable: 1, CUTransquantBypassFlagForce: 1, CostMode: lossless, InternalBitDepth: 0, QP: 22. Additionally, for Tested, ChromaTS:1 configuration was used for all sequences.

Table 2-1: Crosscheck Results of JVET-P0525 Test2

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 | Tested |  | AHG18 | Tested |  | AHG18 | Tested |  |
| Class A1 | 2.2 | 2.2 | -0.18% |  |  |  |  |  |  |
| Class A2 | 1.7 | 1.7 | 0.00% |  |  |  |  |  |  |
| Class B | 2.1 | 2.1 | -0.05% | 2.3 | 2.3 | -0.03% |  |  |  |
| Class C | 2.1 | 2.1 | -0.14% | 2.6 | 2.6 | -0.09% | 2.6 | 2.6 | -0.09% |
| Class D | 1.9 | 2.0 | -0.18% | 2.8 | 2.8 | -0.07% | 2.7 | 2.7 | -0.10% |
| Class E | 2.9 | 2.9 | -0.33% |  |  |  |  |  |  |
| Class F | 5.5 | 5.5 | -1.09% | 39.4 | 40.7 | -1.84% | 78.8 | 83.2 | -2.96% |
| TGM | 12.3 | 12.8 | -3.61% | 112.6 | 115.5 | -2.62% | 114.7 | 116.5 | -2.15% |
| Overall | 2.2 | 2.2 | -0.13% | 1.5 | 1.5 | -0.06% | 0.9 | 0.9 | -0.09% |
| Enc Time[%] | 115% |  |  | 114% |  |  | 114% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  | 96% |  |  |

Reference

[1] T.-C. Ma, Y.-W. Chen, X. Xiu, H.-J. Jhu, X. Wang, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S. Lei, “AHG18: Residual coding selection signaling for lossless VVC,” JVET-P0525

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).