| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0973 |
| --- | --- |

| Title: | Development of a VVC Live Software Decoder |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Adam Wieckowski, Gabriel Hege, Christian Bartnik, Christian Lehmann, Christian Stoffers, Jens Brandenburg, Tobias Hinz, Benjamin Bross, H. Schwarz Detlev Marpe, Thomas Schierl Thomas Wiegand | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The VVC standard reached the committee draft stage at the 15th JVET Meeting in Gothenburg. As the standard is slowly maturing, Fraunhofer HHI started the development of a Live Software Decoder for the upcoming VVC standard. The project is set to be ready shortly after the standard finalization providing an optimized VVC Software Live Decoder for applications requiring live decoding. Currently, the HHI software decoder is compatible with VTM version 4.2, with work in progress to get up-to date with the current development. It can provide live 60fps HD and 30fps UHD decoding on modern mobile x86 platforms.

# Introduction

The VTM reference software is the basis for coding experiments performed by the JVET community in the development process for VVC, the future video codec. While very flexible and successful in this process, it not optimized for practical use. As a reference software it also lacks any multi-threading support on the decoder side. Fraunhofer HHI is developing a dedicated software live decoder for VVC without a burden of having to support the encoder and the decoder in a single software basis. With additional optimizations and a support for multi-threading this already allows software live decoding of VVC bitstreams as specified by the development draft [1].

Fraunhofer HHI first presented the live decoder at the IBC 2019 in Amsterdam showcasing the viability of the standard for practical use. Future version of the decoder will be presented at upcoming trade shows including NAB 2020 and IBC 2020. If request, a presentation can be performed at the JVET Meeting.

# Performance

The following section presents a few performance metrics demonstrating tool optimization potential already exploited by developers as well as the current fps performance of the decoder.

## x86-optimizations

The results were generated on a modern laptop using the anchor bitstreams (averaged over all bitstreams within a category). The unoptimized tests were performed using the HHI-decoder. Deblocking filter also includes also includes parameter derivation. SAO also includes memory operation required by the multi-threaded design.

## Multi-threaded performance

The following tests were performed on the CTC sequences encoded for QP values between 29 and 41, excluding very strong outliers w.r.t. bitrate. The results present the decoding fps performance depending on the number of used threads as well as the bitrate. The results are presented for two different platforms – a modern x86-laptop as well as a modern x86-desktop.

For the decoding, the multi-threading is so far implemented within the frame scope, thus not introducing additional latency. Parsing of multiple frames is allowed in parallel to reduce the impact of this biggest bottle-neck.

### HD results

### UHD results

# Outlook

Future work will concentrate on the incorporation of future versions of the VVC development standard until finalization as well as the optimization of new and existing codec aspects.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- B. Bross, J. Chen, S. Liu, Versatile Video Coding (Draft 4), JVET, doc. JVET-M1001, January 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET common test conditions and software reference configurations for SDR video, JVET, doc. JVET-N1010, March 2019.