| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 Oct. 2019 | Document: JVET-P1026-v1 |
| --- | --- |

| Title: | Non-CE6: Combination of JVET-P0196 and JVET-P0392 on applying LFNST for ISP Blocks and simplifying the transform signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Santiago De-Luxán-Hernández, Valeri George, Gayathri Venugopal, Jens Brandenburg, Benjamin Bross, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Einsteinufer 37 10587 Berlin, Germany Moonmo Koo, Mehdi Salehifar, Jaehyun Lim, Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA Jani Lainema | Email: | santiago.de.luxan@hhi.fraunhofer.de firstname.lastname@hhi.fraunhofer.de moonmo.koo@lge.com mehdi.salehifar@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com first.lastname@nokia.com |
| Source: | Fraunhofer HHI, LG Electronics Inc., Nokia |  |  |

_____________________________

# Abstract

This document combines proposals JVET-P0196 and JVET-P0392. The following results are reported: 0.19% gain, 101% EncT and 100% DecT for AI and 0.18% gain, 101%, 100% DecT in the RA case.

# Introduction

This proposal combines two aspects present in JVET-P0196 and JVET-P0392:

- MTS signalling is moved from the transform unit syntax to after LFNST signalling.

- The restriction that disallows the usage of LFNST if the CU is using ISP is removed.

In the proposed syntax, both LFNST and MTS are signalled in the same place (at the end of the CU) and both are signalled conditionally to the last significant position syntax element. This aspect is compatible with the removal of the ISP-LFNST restriction, so that now the LFNST can be applied to all intra-predicted blocks with the appropriate size.

More details of these aspects can be found in JVET-P0196 and JVET-P0392.

# Experimental results

According to the common test conditions, the proposed method is evaluated for intra-only (AI) and random-access (RA) configuration with the VTM-6.0 software. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and a GCC 7.2.1 compiler. SCC TGM sequences have also been simulated.

Table I: Results for the AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.52% | -0.40% | 103% | 100% |
| Class A2 | -0.10% | -0.33% | -0.34% | 102% | 101% |
| Class B | -0.16% | -0.51% | -0.62% | 101% | 99% |
| Class C | -0.30% | -0.40% | -0.33% | 99% | 101% |
| Class E | -0.23% | -0.57% | -0.60% | 101% | 100% |
| Overall | -0.19% | -0.47% | -0.47% | 101% | 100% |
| Class D | -0.21% | -0.30% | -0.27% | 101% | 104% |
| Class F | -0.21% | -0.29% | -0.30% | 101% | 101% |

Table II: Results for the RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.34% | -0.14% | 102% | 99% |
| Class A2 | -0.09% | -0.19% | -0.25% | 101% | 99% |
| Class B | -0.18% | -0.12% | -0.09% | 101% | 101% |
| Class C | -0.25% | -0.12% | -0.31% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.18% | -0.18% | -0.19% | 101% | 100% |
| Class D | -0.20% | -0.46% | -0.20% | 100% | 98% |
| Class F | -0.17% | -0.21% | 0.10% | 100% | 97% |

# Draft changes

Changes are highlighted in yellow

coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) {

…

LfnstDcOnly = 1

LfnstZeroOutSigCoeffFlag = 1

MtsZeroOutSigCoeffFlag = 1

transform_tree( x0, y0, cbWidth, cbHeight, treeType )

lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : cbWidth : ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT) ? cbWidth / NumIntraSubPartitions : cbWidth

lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : cbHeight : ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight

if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && ( !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && tu_mts_idx[ x0 ][ y0 ] = = 0 && Max( cbWidth, cbHeight ) <= MaxTbSizeY) {

if( ( IntraSubPartitionsSplitType ! = ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 )

lfnst_idx[ x0 ][ y0 ]

}

if( treeType != DUAL_TREE_CHROMA && lfnst_idx[ x0 ][ y0 ] = = 0 && transform_skip_flag[ x0 ][ y0 ] = = 0 && Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT && ( !cu_sbt_flag ) && MtsZeroOutSigCoeffFlag = = 1 && tu_cbf_luma[ x0 ][ y0 ] ) {

if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) )

mts_idx[ x0 ][ y0 ]

}

…

transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) {

…

if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) {

if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize )

transform_skip_flag[ x0 ][ y0 ]

if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) && ( !transform_skip_flag[ x0 ][ y0 ] ) )

tu_mts_idx[ x0 ][ y0 ]

}

…

residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) {

…

if( ( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbWidth > 4 )

log2ZoTbWidth = 4

else

log2ZoTbWidth = Min( log2TbWidth, 5 )

MaxCcbs = 2 * ( 1 << log2TbWidth ) * ( 1<< log2TbHeight )

if( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbHeight > 4 )

log2ZoTbHeight = 4

else

log2ZoTbHeight = Min( log2TbHeight, 5 )

…

if( ( lastSubBlock > 0 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ¦ ¦ ( lastScanPos > 7 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && log2TbWidth = = log2TbHeight ) )

LfnstZeroOutSigCoeffFlag = 0

if( ( LastSignificantCoeffX > 15 ¦ ¦ LastSignificantCoeffY > 15 ) && cIdx = = 0 )

MtsZeroOutSigCoeffFlag = 0

…

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived as follows:

- If cIdx is greater than 0, trTypeHor and trTypeVer are set equal to 0.

- When any of the following conditions is true, trTypeHor and TrTypeVer are set equal to 0:

  - cIdx is greater than 0

  - IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and lfnst_idx[ xTbY ][ yTbY ] is not equal to 0

# Conclusion

The reported results show a gain 0.19% in AI and 0.18% in RA. It can be observed that these results are almost the sum of the individual gains reported in JVET-P0196 and JVET-P0392, which confirms that the proposed aspects can be combined without problems.

# Patent rights declaration(s)

Fraunhofer HHI, LG Electronics and Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).