| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0255 |
| --- | --- |

| Title: | AHG9: Coded Picture NAL unit |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande Andrew Segall | Tel: Email: | +46 721 88 21 69 samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution contains a proposal for a new NAL unit type to be added to VVC. In the most recent draft of VVC, each access units consists of at least two NAL units: Picture Header and Coded Slice(s). It is asserted that it would be desirable with support for a mode of operation in VVC in which an access unit can be fully represented with a single NAL unit. It is asserted that such support would be particularly beneficial for low bitrate applications and applications in which there is as significant overhead associated with each NAL unit. A NAL unit type called Coded Picture is proposed, and it is proposed to include the following four elements:

- A syntax element pic_type.

- A picture header syntax structure.

- A slice header syntax structure.

- A slice data syntax structure.

The proposed NAL unit type has been implemented in VTM 7.1 and the results show an average luma BD-rate gain of 0.35% for LDB case. Results are also presented for when the suggested new NAL unit type is combined with JVET-Q0258 and proposal 1-4 of JVET-Q0259 showing an average luma BD-rate gain of 0.63% for LDB.

# Introduction

In development of the VVC standard it has been identified that support for in-picture error concealment is not a high priority design goal and it has been decided to gather parameters that applies to a picture in a picture header instead of repeating it in every slice header. Each picture contains exactly one picture header and it will generally not be possible to parse, decode or interpret a coded slice unless the associated picture header is available. Consequently, there will be limited benefit to splitting up pictures in slices, especially since parallel encoding (such as tiles and wavefronts) are supported in a single slice. One exception is Virtual Reality applications where slices can be used to extract and decode only parts of the video (e.g. using subpictures). However, for most general video applications it can be assumed that the most efficient encoding will be achieved by encoding pictures in a single slice and it can be assumed that this will be a very common configuration. It would therefore be highly desirable to support this configuration as efficiently as possible.

## Low bitrate video applications

Even though modern video codecs generally enable support for higher resolutions, higher bitdepths and higher bitrates, it should be noted that new codecs are also used to provide a more acceptable and enjoyable video experience at the low end of the bitrate range. One example is the “HLS Authoring Specification for Apple Devices” [1] where a bitrate ladder starting at 145 kbps is recommended for both H.264/AVC and HECV/H.265. Another example is the low-quality profile recently introduced by Vimeo to increase reach and reduce buffer times. The profile uses AVC/H.264 at 300 kbps [2].

Since VVC provides clearly improved compression performance compared to HEVC and massively improved compression performance over H.264/AVC it reasonable to assume that VVC can be used at bitrates as low as 150 kbps (or even lower).

When encoding a 30 fps video at 150 kbps, each picture uses on average 625 bytes. Consequently, a single byte of extra overhead increases the bitrate by 0.16%.

# Overhead of using multiple NAL units

In all encapsulation formats, file formats and transport protocols there is a certain amount of signalling overhead associated with each NAL unit.

- In Annex B byte streams there is a startcode overhead of typically 3 or 4 bytes.

- In the file format for Carriage of NAL unit structured video in the ISO Base Media File Format, a sample length field of 4 bytes is typically used [3].

- The RTP payload format offers the ability to aggregate multiple NAL units in the same RTP packet wherein each NAL unit is preceded by a 2 byte length field. However, the aggregation of NAL units can only be used if all aggregated NAL units completely fit into a single RTP packet. If the aggregation does not fit completely in a single RTP packet, the NAL units have to be sent in separate RTP packets which is typically associated with an overhead of at least 40 bytes (20 bytes IP-header, 8 bytes UDP-header, and 12 bytes RTP header) [4].

In addition to this overhead, there is the overhead of the NAL unit header, which in VVC is 2 bytes.

# Proposal

It is proposed to allow a mode of operation in VVC where a single NAL unit per picture is used. This could for example be accomplished by making the picture header optional and adding back some syntax elements to the slice header that have been moved to the picture header. Another option, which was proposed in [6] would be to allow the picture header to be included as a syntax structure in the slice header.

However, both of these options have an impact on the current structure of picture header and slice header. It is asserted that the current structure is useful in the case where multiple pictures are used per picture and it is therefore desirable with a solution that have minimal impact on the current structure.

We propose to keep the Picture Header NAL unit and the different Coded Slice NAL unit types and introduce a new NAL unit type for Coded Picture. The Coded Picture NAL unit is proposed to include the following four elements:

- A syntax element pic_type.

- A picture header syntax structure.

- A slice header syntax structure.

- A slice data syntax structure.

## Proposed draft text

A new NAL unit type is proposed:

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | TRAIL_NUT | Coded slice of a trailing picture slice_layer_rbsp( ) | VCL |
| 1 | STSA_NUT | Coded slice of an STSA picture slice_layer_rbsp( ) | VCL |
| 2 | RADL_NUT | Coded slice of a RADL picture slice_layer_rbsp( ) | VCL |
| 3 | RASL_NUT | Coded slice of a RASL picture slice_layer_rbsp( ) | VCL |
| 4..5 | RSV_VCL_4.. RSV_VCL_5 | Reserved non-IRAP VCL NAL unit types | VCL |
| 6 | CODED_PIC | Coded picture picture_layer_rbsp( ) | VCL |
| 7 8 | IDR_W_RADL IDR_N_LP | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 9 | CRA_NUT | Coded slice of a CRA picture silce_layer_rbsp( ) | VCL |
| 10 | GDR_NUT | Coded slice of a GDR picture slice_layer_rbsp( ) | VCL |
| 11 12 | RSV_IRAP_11 RSV_IRAP_12 | Reserved IRAP VCL NAL unit types | VCL |
| 13 | DPS_NUT | Decoding parameter set decoding_parameter_set_rbsp( ) | non-VCL |
| 14 | VPS_NUT | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 15 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 16 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 17 18 | PREFIX_APS_NUT SUFFIX_APS_NUT | Adaptation parameter set adaptation_parameter_set_rbsp( ) | non-VCL |
| 19 | PH_NUT | Picture header picture_header_rbsp( ) | non-VCL |
| 20 | AUD_NUT | AU delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 21 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 22 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 23 24 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 25 | FD_NUT | Filler data filler_data_rbsp( ) | non-VCL |
| 26 27 | RSV_NVCL_26 RSV_NVCL_27 | Reserved non-VCL NAL unit types | non-VCL |
| 28..31 | UNSPEC_28.. UNSPEC_31 | Unspecified non-VCL NAL unit types | non-VCL |

The new picture_layer_rbsp() includes a pic_nal_type which can be used to indicate if the picture is an IDR picture, CRA picture, GDR picture etc. It also includes a picture header structure, a slice header strucutre and a slice data structure.

| picture_layer_rbsp( ) { | Descriptor |
| --- | --- |
| pic_nal_type | u(4) |
| picture_header_rbsp( ) |  |
| slice_header( ) |  |
| slice_data( ) |  |
| } |  |

The pic_nal_type is used in the slice header, and in other places to provide the functionality of nal_type_idc.

pic_nal_type indicates the corresponding Nal Unit type of the picture.

The value of NalType is derived as follows:

NalType = ( nal_unit_type = = CODED_PIC ) ? pic_nal_type : nal_unit_type

## Optimizations

In addition to the basic functionality provided by the proposal in section 3.1, the following changes are proposed in order to provide a more efficient solution:

- Remove the byte alignement between picture header and slice header in the coded picture strucutre

- Conditional signalling of gdr_pic_flag based on NalType

- Conditional signalling of no_output_of_prior_pics_flag based on NalType

| picture_header_structurerbsp( ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| if( NalType = = PH_NUT ) |  |
| gdr_pic_flag | u(1) |
| if( NalType = = PH_NUT ¦ ¦ NalType = = IDR_W_RADL ¦ ¦ NalType = = IDR_N_LP ¦ ¦ NalType = = CRA_NUT ¦ ¦ NalType = = GDR_NUT ) |  |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| … |  |
| if( picture_header_extension_present_flag ) { |  |
| ph_extension_length | ue(v) |
| for( i = 0; i < ph_extension_length; i++) |  |
| ph_extension_data_byte[ i ] | u(8) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| picture_header_structure( ) |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| picture_layer_rbsp( ) { | Descriptor |
| --- | --- |
| pic_nal_type | u(4) |
| picture_header_structure( ) |  |
| slice_header( ) |  |
| slice_data( ) |  |
| rbsp_trailing_bits( ) |  |
| } |  |

# Results

The proposed scheme has been implemented in VTM7.1 (commit 0e27375b98b629fb203c4acaff3ae892f3c79ae6) and tested using the Common Test Conditions [5]. The first set of results includes only the gain from using the Coded Picture NAL unit type as described in 3.1. The second set of results includes the optimized version described in section 3.2 and additionally the improvements provided by JVET-Q0258 and proposal 1-4 of JVET-Q0259.

## Results for Coded Picture NAL

These results includes only the gain from using the Coded Picture NAL unit type as described in 3.1.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | - |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | - |
| Class B | -0.01% | -0.01% | -0.01% | 100% | - |
| Class C | -0.02% | -0.02% | -0.02% | 100% | - |
| Class E | -0.02% | -0.02% | -0.02% | 100% | - |
| Overall | -0.01% | -0.01% | -0.01% | 100% | - |
| Class D | -0.05% | -0.05% | -0.05% | 100% | - |
| Class F | -0.02% | -0.02% | -0.02% | 99% | - |
|  |  |  | Random access |  |  |
|  |  |  | Over |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.03% | -0.03% | 100% | - |
| Class A2 | -0.03% | -0.03% | -0.03% | 98% | - |
| Class B | -0.08% | -0.08% | -0.08% | 99% | - |
| Class C | -0.19% | -0.19% | -0.19% | 99% | - |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.09% | -0.09% | 99% | - |
| Class D | -0.52% | -0.52% | -0.52% | 100% | - |
| Class F | -0.22% | -0.22% | -0.22% | 99% | - |
|  |  |  | Low delay B |  |  |
|  |  |  | Over |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.09% | -0.08% | -0.09% | 100% | 100% |
| Class C | -0.20% | -0.19% | -0.19% | 100% | 98% |
| Class E | -0.98% | -0.93% | -0.92% | 99% | 101% |
| Overall | -0.35% | -0.33% | -0.33% | 100% | 100% |
| Class D | -0.58% | -0.55% | -0.56% | 98% | 99% |
| Class F | -0.49% | -0.46% | -0.46% | 99% | 99% |
|  |  |  | Low delay P |  |  |
|  |  |  | Over |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.09% | -0.08% | -0.09% | 100% | 101% |
| Class C | -0.20% | -0.19% | -0.19% | 100% | 99% |
| Class E | -0.96% | -0.91% | -0.91% | 99% | 100% |
| Overall | -0.34% | -0.32% | -0.33% | 100% | 100% |
| Class D | -0.56% | -0.53% | -0.54% | 97% | 99% |
| Class F | -0.41% | -0.38% | -0.38% | 99% | 101% |

## Results for optimized Coded Picture NAL in combination with picture header improvements

The second set of results includes the optimized version described in section 3.2 and additionally the improvements provided by JVET-Q0258 and proposal 1-4 of JVET-Q0259.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | -0.01% | 100% | #DIV/0! |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | #DIV/0! |
| Class B | -0.01% | -0.01% | -0.01% | 100% | #DIV/0! |
| Class C | -0.03% | -0.02% | -0.03% | 100% | #DIV/0! |
| Class E | -0.03% | -0.03% | -0.03% | 100% | #DIV/0! |
| Overall | -0.02% | -0.02% | -0.02% | 100% | #DIV/0! |
| Class D | -0.08% | -0.07% | -0.07% | 100% | #DIV/0! |
| Class F | -0.02% | -0.02% | -0.02% | 100% | #DIV/0! |
|  |  |  | Random access |  |  |
|  |  |  | Over |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.07% | -0.07% | 100% | #DIV/0! |
| Class A2 | -0.07% | -0.07% | -0.06% | 101% | #DIV/0! |
| Class B | -0.16% | -0.16% | -0.16% | 98% | #DIV/0! |
| Class C | -0.36% | -0.36% | -0.35% | 99% | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | -0.18% | -0.18% | -0.18% | 99% | #DIV/0! |
| Class D | -1.01% | -1.01% | -1.02% | 100% | #DIV/0! |
| Class F | -0.37% | -0.37% | -0.37% | 100% | #DIV/0! |
|  |  |  | Low delay B |  |  |
|  |  |  | Over |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.17% | -0.17% | -0.17% | 104% | 105% |
| Class C | -0.39% | -0.37% | -0.37% | 101% | 101% |
| Class E | -1.86% | -1.76% | -1.76% | 99% | 103% |
| Overall | -0.66% | -0.63% | -0.63% | 102% | 103% |
| Class D | -1.11% | -1.07% | -1.08% | 98% | 99% |
| Class F | -0.94% | -0.88% | -0.88% | 103% | 102% |
|  |  |  | Low delay P |  |  |
|  |  |  | Over |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | -0.16% | -0.16% | 101% | 104% |
| Class C | -0.37% | -0.36% | -0.36% | 97% | 99% |
| Class E | -1.77% | -1.68% | -1.68% | 100% | 100% |
| Overall | -0.63% | -0.61% | -0.61% | 99% | 101% |
| Class D | -1.06% | -1.02% | -1.03% | 99% | 97% |
| Class F | -0.89% | -0.83% | -0.83% | 100% | 101% |

# Conclusion

It is strongly suggested to bring back support for encoding one picture in one NAL unit in VVC. The Coded Picture NAL unit type is proposed as an additional NAL unit type. Existing structure of Picture Header and Slice Header is kept in its current form.

# References

[1] HLS Authoring Specification for Apple Devices - 
https://developer.apple.com/documentation/http_live_streaming/hls_authoring_specification_for_apple_devices

[2] Vimeo Engineering Blog - https://medium.com/vimeo-engineering-blog/a-story-of-lowering-quality-for-improving-accessibility-121dcd3871cc

[3] ISO/IEC 14496-15 Carriage of NAL unit structured video in the ISO Base Media File Format

[4] IETF RFC 7798 RTP Payload Format for High Efficiency Video Coding (HEVC)

[5] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, "JVET common test conditions and software reference configurations for SDR video," JVET-N1010, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 14th Meeting: Geneva, CH, 19–27 Mar. 2019

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).