| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0464-v3 |
| --- | --- |

| Title: | On modifications of intra prediction process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Biao Wang, Semih Esenlik, Anand Meher Kotra, Han Gao, Elena Alshina | Email: | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com biao.wang@huawei.com semih.esenlik@huawei.com anand.meher.kotra@huawei.com han.gao@huawei.com elena.alshina@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

This contribution proposes to modify intra prediction process. In particular, PDPC scaling is harmonized with intra subpartitions (ISP) coding mode, so that the number of reference samples used is maximized. The simulation results for method 1 reportedly show the Y/U/V BD-rate changes of 0.00%/- 0.02%/0.00%, -0.01%/-0.01%/0.03% and -0.04%/0.01%/-0.16% for AI, RA and LDB configurations, respectively. The simulation results for method 2 reportedly show the Y/U/V BD-rate changes of 0.00%/- 0.03%/0.00% and 0.00%/-0.06%/0.07% for AI and RA configurations, respectively. More significant coding efficiency improvements (-0.12%/0.57%/-0.27% and -0.10%/-0.32%/0.24% for method 1 and method 2, respectively) are achieved for class E in LDB case.

In version 2, a symmetric intra interpolation filter selection is used, named as method 3. This method is independent from the other proposed ones. Compared to the current asymmetric interpolation filter selection approach in VTM7.0, method 3 achieves luma BD-rate changes of 0.00%/0.01%/-0.06% for AI, RA and LDB configurations, respectively. In addition, a combined test of method 1 and 3 are performed, the BD-rate changes of 0.00%/-0.01%/-0.04% for AI, RA and LDB configurations, respectively.

In version 3, a test with minimal changes to the specification (method 0) is carried out. Minor consistent improvement of coding performance on RA is observed for this method. The simulation results for method 0 reportedly show the Y/U/V BD-rate changes of 0.00%/-0.03%/-0.01%, -0.02%/-0.04%/0.01% and 0.00%/-0.08%/-0.14% for AI, RA and LDB configurations, respectively.

# Introduction and problem statement

The value of nScale parameter depends on the width or height of the predicted block. However, when a block is coded using intra subpartitions (ISP) mode, the size of this subpartition is used to determine the nScale value.

Furthermore, the current Smoothing/DCT-IF filter selection process might result into an asymmetric filtering. The following text is used to select one of these two filters, where nTbS is derived as ( Log2 ( nTbW ) + Log2 ( nTbH ) ) >> 1:

- The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) ).

- The variable intraHorVerDistThres[ nTbS ] is specified in Table 25.

- The variable filterFlag is derived as follows:

- If minDistVerHor is greater than intraHorVerDistThres[ nTbS ], filterFlag is set equal to 1.

- Otherwise, filterFlag is set equal to 0.

Table 25 – Specification of intraHorVerDistThres[ nTbS ] for various transform block sizes nTbS

|  | nTbS = 2 | nTbS = 3 | nTbS = 4 | nTbS = 5 | nTbS = 6 | nTbS = 7 |
| --- | --- | --- | --- | --- | --- | --- |
| intraHorVerDistThres[ nTbS ] | 24 | 14 | 2 | 0 | 0 | 0 |

If a mode difference to the horizontal or vertical (i.e. minDistVerHor) is larger than a correpsonding threshold value intraHorVerDistThres[ nTbS ], then filterFlag is set to 1 and Gaussian filter is selected. Considering an 8x16 block, nTbS would be 3 and the threshold intraHorVerDistThres[nTbS] equals to 14. In this case, there would be 6 modes below horizontal direction (bottom modes) and but only 2 modes above horizontal direction (top modes) will choose Gaussian filter. For bottom mode X in modes 3, -1 to -5 (after the wide angle intra mode mapping), the minDistVerHor value for mode X would choose abs(X – 18) and it is larger than the threshold value 14. However, only 2 modes (33 and 35) in the top symmetric modes (33, 35 to 39) would apply Gaussian filter, as for modes 36 to 39 their mode difference to vertical is smaller or equal to 14.

# Proposed methods

In this proposal, two methods are considered. These methods uses different approach to handle ISP split case. These methods are described in sections below.

## Method 0

In this method it is suggested to introduce minimal changes to the specification, but to align derivation of nScale parameter with ISP mode. In the specification, the value of refW and refH are defined differently for ISP and non-ISP modes. It could be noticed that when ISP is the following expressions are valid:

refW − nTbW = nTbW

refH − nTbH = nTbH,

since refW = 2 * nTbW and refH = 2 * nTbH.

When ISP is on, the same expressions would have different result:

refW − nTbW = nCbW

refH − nTbH = nCbH,

since refW = nCbW + nTbW and refH = nCbH + nTbH.

Changes to VVC specification draft [1] that correspond to this test are as follows:

The variable nScale is derived as follows:

- If predModeIntra is greater than INTRA_ANGULAR50, nScale is set equal to Min( 2, Log2( refH − nTbH ) − Floor( Log2( 3 * invAngle − 2 ) ) + 8 ), using invAngle as specified in clause 8.4.5.2.12.

- Otherwise, if predModeIntra is less than INTRA_ANGULAR18, not equal to INTRA_PLANAR and not equal to INTRA_DC, nScale is set equal to Min( 2, Log2( refW −  nTbW ) − Floor( Log2( 3 * invAngle − 2 ) ) + 8 ), using invAngle as specified in clause 8.4.5.2.12.

- Otherwise, nScale is set to ( ( Log2( nTbW ) + Log2( nTbH ) − 2 )  >>  2 ).

## Method 1

In this method, derivation of nScale parameter is performed as follows:

The variable nScale is derived as follows:

- If predModeIntra is equal to INTRA_PLANAR, INTRA_DC, INTRA_ANGULAR18, or INTRA_ANGULAR50, nScale is set to ( ( Log2( nTbW ) + Log2( nTbH ) − 2 ) >> 2 ).

Otherwise, the following is performed:

- nTbS is set equal to predModeIntra > INTRA_ANGULAR50 ? refH − nTbH : refW − nTbW and nTbS0 equal to predModeIntra > INTRA_ANGULAR50 ? nTbH : nTbW

- when nTbS==nTbS0, nScale is set equal to Min( 2, Log2( nTbS ) − Floor( Log2( 3 * invAngle − 2 ) ) + 8 ),

- otherwise (nTbS!=nTbS0),  the following is performed:

  - nScaleCb = Log2( nTbS) − Floor( Log2( 3 * invAngle − 2 ) ) + 8

  - nScale =  Log2( nTbS0) − Floor( Log2( 3 * invAngle − 2 ) ) + 8

  - nScale = nScale < 0 ? min(0, nScaleCb) : min(2, nScale);

wherein invAngle as specified in clause 8.4.5.2.12. If the value of nScale is less than 0, process 8.4.5.2.14 is terminated, and outputs of this process are set equal to the input predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

## Method 2

In this method it is suggested to modify derivation of nScale parameter in accordance with the modifications to the VVC specification draft [1] as follows:

The variable nScale is derived as follows:

- If predModeIntra is equal to INTRA_PLANAR, INTRA_DC, INTRA_ANGULAR18, or INTRA_ANGULAR50,  nScale is set equal to ( ( Log2( nTbW ) + Log2( nTbH ) − 2 ) >> 2 ).

- Otherwise, the following is performed:

    - nTbS is set equal to predModeIntra > INTRA_ANGULAR50 ? refH − nTbH : refW − nTbW and nTbS0 equal to predModeIntra > INTRA_ANGULAR50 ? nTbH : nTbW

    - if nTbS==nTbS0, nScale is set to Min(2, Log2( nTbS0) − Floor( Log2( 3 * invAngle − 2 ) ) + 8 )

    - Otherwise, if Log2( nTbS ) < Floor( Log2( 3 * invAngle − 2 ) ) + 8, nScale is set to – 1,

    - Otherwise nScale is set to 0

using invAngle as specified in clause 8.4.5.2.12. If the value of nScale is less than 0, process 8.4.5.2.14 is terminated, and outputs of this process are set equal to the input predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

## Method 3

## The method described in this section is independent from the methods described above.

To address the asymmetric filtering issue as described above, the following spec changes are proposed, with addition highlighted and deletion with strikethrough. The idea is to introduce a corner mode and derive the mode difference based on corner mode rather than the minimum mode difference to vertical or horizontal mode.

The variable nTbS is set equal to ( Log2 ( nTbW ) + Log2 ( nTbH ) )  >>  1.

The variable cornerMode is derived according to Table 8-X,

Table 8-X: cornerMode under different block aspect ratio

| Log2(nTbW/nTbH)+4 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cornerMode | 48 | 46 | 44 | 40 | 34 | 28 | 24 | 22 | 20 |

The variable filterFlag is derived as follows:

- If one or more of the following conditions is true, filterFlag is set equal to 0:

- refFilterFlag is equal to 1

- refIdx is not equal to 0

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- Otherwise, the following applies:

- The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) )

predModeIntra>cornerMode? Abs( predModeIntra − 50 ): Abs( predModeIntra − 18 ).

- The variable intraHorVerDistThres[ nTbS ] is specified in Table 25.

- The variable filterFlag is derived as follows:

- If minDistVerHor is greater than intraHorVerDistThres[ nTbS ], filterFlag is set equal to 1.

- Otherwise, filterFlag is set equal to 0.

Table 25 – Specification of intraHorVerDistThres[ nTbS ] for various transform block sizes nTbS

|  | nTbS = 2 | nTbS = 3 | nTbS = 4 | nTbS = 5 | nTbS = 6 | nTbS = 7 |
| --- | --- | --- | --- | --- | --- | --- |
| intraHorVerDistThres[ nTbS ] | 24 | 14 | 2 | 0 | 0 | 0 |

# Experimental results

The tests were carried out in accordance with [2] on the basis of VTM-7.0. Results of the tests for methods 0, 1, 2, and 3 are correspondingly shown in Table 1 to 4. In addition, a combined test (test 4) of method 1 and method 3 are performed. The obtained results for test 4 are presented in Table 5. Results for similar combination, but for methods 2 and 3 are presented in Table 6.

Table 1. Method 0 simulation results for AI, RA and LDB

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.13% | 0.07% | 100% | 100% |
| Class A2 | 0.00% | -0.04% | -0.05% | 101% | 101% |
| Class B | 0.00% | 0.01% | -0.11% | 100% | 99% |
| Class C | 0.01% | 0.09% | 0.03% | 99% | 97% |
| Class E | 0.00% | -0.13% | 0.02% | 100% | 97% |
| Overall | 0.00% | -0.03% | -0.01% | 100% | 99% |
| Class D | -0.01% | 0.05% | -0.05% | 100% | 95% |
| Class F | 0.00% | 0.02% | -0.02% | 101% | 98% |
|  | Random Access |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.19% | 0.01% | 101% | 101% |
| Class A2 | -0.03% | 0.05% | 0.07% | 101% | 101% |
| Class B | -0.02% | 0.05% | 0.08% | 101% | 100% |
| Class C | -0.01% | -0.11% | -0.11% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.04% | 0.01% | 101% | 100% |
| Class D | -0.04% | -0.23% | 0.00% | 100% | 99% |
| Class F | 0.00% | -0.02% | -0.01% | 101% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.30% | -0.06% | 102% | 100% |
| Class C | 0.01% | -0.47% | -0.24% | 101% | 101% |
| Class E | -0.03% | 0.78% | -0.14% | 101% | 101% |
| Overall | 0.00% | -0.08% | -0.14% | 101% | 101% |
| Class D | -0.02% | 0.43% | 0.25% | 101% | 101% |
| Class F | 0.01% | -0.49% | 0.43% | 102% | 101% |

Table 2. Method 1 simulation results for AI, RA and LDB

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.14% | 0.11% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | -0.07% | 100% | 100% |
| Class B | 0.00% | 0.02% | -0.07% | 100% | 100% |
| Class C | 0.00% | 0.08% | -0.04% | 99% | 100% |
| Class E | -0.01% | -0.09% | 0.15% | 100% | 100% |
| Overall | 0.00% | -0.02% | 0.00% | 100% | 100% |
| Class D | 0.00% | -0.04% | -0.09% | 100% | 101% |
| Class F | 0.00% | 0.07% | 0.03% | 100% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | -0.02% | 100% | 100% |
| Class A2 | -0.02% | 0.08% | 0.05% | 100% | 100% |
| Class B | -0.01% | 0.02% | 0.10% | 100% | 100% |
| Class C | 0.01% | -0.10% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.03% | 100% | 100% |
| Class D | 0.00% | -0.24% | 0.04% | 100% | 100% |
| Class F | -0.01% | -0.01% | -0.02% | 100% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.22% | 0.05% | 100% | 99% |
| Class C | -0.01% | -0.13% | -0.34% | 100% | 99% |
| Class E | -0.12% | 0.57% | -0.27% | 100% | 99% |
| Overall | -0.04% | 0.01% | -0.16% | 100% | 99% |
| Class D | -0.04% | 0.25% | -0.06% | 100% | 100% |
| Class F | -0.08% | 0.00% | 0.29% | 100% | 99% |

Table 3. Method 2 simulation results for AI, RA and LDB

|  | All Intra |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 | 0.00% |  | -0.10% |  | 0.16% |  | 99% |  | 99% |
| Class A2 | 0.01% |  | 0.05% |  | -0.06% |  | 99% |  | 100% |
| Class B | 0.01% |  | -0.09% |  | -0.11% |  | 99% |  | 98% |
| Class C | 0.00% |  | 0.04% |  | 0.06% |  | 99% |  | 95% |
| Class E | 0.01% |  | -0.02% |  | 0.01% |  | 100% |  | 97% |
| Overall | 0.00% |  | -0.03% |  | 0.00% |  | 99% |  | 98% |
| Class D | 0.00% |  | -0.12% |  | -0.05% |  | 100% |  | 94% |
| Class F | 0.00% |  | -0.06% |  | 0.03% |  | 101% |  | 96% |
|  | Random Access |  |  |  |  |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 | 0.02% |  | 0.00% |  | 0.01% |  | 100% |  | 100% |
| Class A2 | -0.02% |  | 0.01% |  | 0.01% |  | 101% |  | 101% |
| Class B | 0.01% |  | -0.08% |  | 0.15% |  | 100% |  | 98% |
| Class C | -0.02% |  | -0.13% |  | 0.07% |  | 100% |  | 99% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% |  | -0.06% |  | 0.07% |  | 100% |  | 99% |
| Class D | 0.00% |  | -0.20% |  | -0.08% |  | 101% |  | 99% |
| Class F | -0.07% |  | -0.03% |  | 0.00% |  | 100% |  | 99% |
|  | Low delay B |  |  |  |  |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% |  | -0.31% |  | -0.16% |  | 101% |  | 102% |
| Class C | 0.03% |  | -0.30% |  | -0.38% |  | 100% |  | 100% |
| Class E | -0.10% |  | -0.32% |  | 0.24% |  | 100% |  | 100% |
| Overall | -0.01% |  | -0.31% |  | -0.13% |  | 101% |  | 101% |
| Class D | 0.00% |  | 0.13% |  | -0.37% |  | 100% |  | 100% |
| Class F | -0.05% |  | -0.19% |  | 0.38% |  | 101% |  | 100% |

Table 4. Method 3 simulation results for AI, RA and LDB

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.11% | 0.10% | 101% | 100% |
| Class A2 | -0.01% | -0.02% | -0.08% | 101% | 99% |
| Class B | 0.00% | 0.01% | -0.07% | 100% | 100% |
| Class C | 0.04% | 0.05% | 0.09% | 100% | 100% |
| Class E | 0.00% | -0.04% | 0.15% | 101% | 99% |
| Overall | 0.00% | -0.01% | 0.03% | 101% | 100% |
| Class D | -0.01% | 0.06% | 0.04% | 100% | 99% |
| Class F | 0.05% | 0.03% | -0.01% | 99% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.07% | 0.00% | 101% | 99% |
| Class A2 | -0.01% | 0.00% | -0.02% | 100% | 99% |
| Class B | 0.00% | -0.03% | 0.13% | 101% | 100% |
| Class C | 0.03% | -0.08% | -0.03% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.04% | 0.03% | 101% | 100% |
| Class D | -0.01% | -0.16% | -0.14% | 100% | 100% |
| Class F | 0.02% | -0.01% | 0.03% | 100% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.18% | -0.36% | 98% | 96% |
| Class C | 0.01% | -0.41% | -0.17% | 99% | 96% |
| Class E | -0.22% | 0.52% | 0.27% | 99% | 96% |
| Overall | -0.06% | -0.08% | -0.14% | 99% | 96% |
| Class D | 0.05% | -0.11% | -0.37% | 100% | 99% |
| Class F | -0.02% | -0.06% | 0.39% | 99% | 98% |

Table 5. Test 4 simulation results for AI, RA and LDB

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.17% | 0.07% | 100% | 100% |
| Class A2 | -0.01% | 0.02% | -0.10% | 100% | 100% |
| Class B | 0.00% | -0.05% | -0.09% | 100% | 100% |
| Class C | 0.03% | 0.06% | 0.12% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.16% | 100% | 100% |
| Overall | 0.00% | -0.02% | 0.02% | 100% | 100% |
| Class D | 0.01% | -0.02% | 0.02% | 100% | 102% |
| Class F | 0.05% | 0.11% | 0.09% | 100% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | -0.09% | 100% | 100% |
| Class A2 | -0.01% | -0.14% | -0.03% | 100% | 100% |
| Class B | -0.02% | 0.01% | 0.12% | 100% | 100% |
| Class C | 0.01% | -0.10% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.06% | 0.01% | 100% | 100% |
| Class D | -0.03% | -0.25% | 0.09% | 100% | 100% |
| Class F | -0.01% | 0.05% | 0.01% | 100% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.21% | -0.22% | 100% | 100% |
| Class C | 0.06% | -0.53% | -0.10% | 100% | 100% |
| Class E | -0.19% | 0.52% | -0.69% | 100% | 100% |
| Overall | -0.04% | -0.13% | -0.30% | 100% | 100% |
| Class D | -0.03% | 0.04% | -0.63% | 100% | 100% |
| Class F | 0.02% | -0.24% | 0.46% | 100% | 99% |

Table 6. Test 5 simulation results for AI, RA and LDB

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.21% | 0.07% | 100% | 100% |
| Class A2 | 0.00% | 0.04% | -0.07% | 100% | 100% |
| Class B | 0.00% | -0.03% | -0.05% | 100% | 100% |
| Class C | 0.04% | 0.05% | 0.01% | 100% | 100% |
| Class E | 0.01% | 0.03% | 0.03% | 101% | 100% |
| Overall | 0.01% | -0.02% | -0.01% | 100% | 100% |
| Class D | 0.02% | 0.00% | 0.06% | 100% | 99% |
| Class F | 0.03% | 0.06% | 0.12% | 101% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.15% | -0.11% | 101% | 101% |
| Class A2 | -0.02% | -0.05% | -0.05% | 101% | 101% |
| Class B | 0.00% | 0.08% | 0.10% | 101% | 100% |
| Class C | 0.01% | 0.03% | -0.05% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | -0.01% | 101% | 100% |
| Class D | 0.03% | -0.27% | 0.01% | 101% | 100% |
| Class F | 0.02% | 0.08% | 0.01% | 101% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.01% | -0.29% | -0.19% | 101% | 100% |
| Class E | -0.02% | 0.22% | 0.79% | 101% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.00% | 0.67% | -0.27% | 101% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# Conclusion

It is proposed to adopt the modifications to the VVC 8.0 software and to the VVC specification draft 8.

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, 16th JVET meeting, Geneva, October 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET Meeting, Geneva, Switzerland, March 2019.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).