Text changes (based on JVET-Q0041-v2_DraftText.docx), to reflect the notes during meeting discussion (quoted below):

“It was agreed to have a two-flag approach. One flag that indicates the presence of intra parameters and one that indicates the presence of inter parameters. When the first flag is 0, no inter slices can be present. When the second flag is 0, no intra slices can be present. If the first flag is 0, the second flag is not present and inferred to be equal to 1. When the first flag is 0, the SH would not contain a slice_type.”

- 7.3.2.6 Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| pic_inter_slice_present_flag | u(1) |
| if( pic_inter_slice_present_flag ) |  |
| pic_intra_slice_present_flag | u(1) |
| non_reference_picture_flag | u(1) |
| gdr_pic_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| ph_pic_parameter_set_id | ue(v) |
| if( sps_poc_msb_flag ) { |  |
| ph_poc_msb_present_flag | u(1) |
| if( ph_poc_msb_present_flag ) |  |
| poc_msb_val | u(v) |
| } |  |
| if( sps_subpic_id_present_flag && !sps_subpic_id_signalling_flag ) { |  |
| ph_subpic_id_signalling_present_flag | u(1) |
| if( ph_subpics_id_signalling_present_flag ) { |  |
| ph_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| ph_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| if( !sps_virtual_boundaries_present_flag ) { |  |
| ph_virtual_boundaries_present_flag | u(1) |
| if( ph_virtual_boundaries_present_flag ) { |  |
| ph_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_ver_virtual_boundaries; i++ ) |  |
| ph_virtual_boundaries_pos_x[ i ] | u(13) |
| ph_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_hor_virtual_boundaries; i++ ) |  |
| ph_virtual_boundaries_pos_y[ i ] | u(13) |
| } |  |
| } |  |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| pic_rpl_present_flag | u(1) |
| if( pic_rpl_present_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_sps_flag[ i ] | u(1) |
| if( pic_rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| pic_poc_lsb_lt[ i ][ j ] | u(v) |
| pic_delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( pic_delta_poc_msb_present_flag[ i ][ j ] ) |  |
| pic_delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | u(1) |
| if( pic_intra_slice_present_flag ) { |  |
| if( partition_constraints_override_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( cu_qp_delta_enabled_flag ) { |  |
| pic_cu_qp_delta_subdiv_intra_slice | ue(v) |
| pic_cu_qp_delta_subdiv_inter_slice | ue(v) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) |  |
| pic_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| } |  |
| if( pic_inter_slice_present_flag ) { |  |
| if( partition_constraints_override_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| } |  |
| if( cu_qp_delta_enabled_flag ) |  |
| pic_cu_qp_delta_subdiv_inter_slice | ue(v) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| pic_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| pic_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| pic_temporal_mvp_enabled_flag | u(1) |
| if(!pps_mvd_l1_zero_idc ) |  |
| mvd_l1_zero_flag | u(1) |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |
| pic_six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| pic_five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| pic_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_pic_present_flag ) |  |
| pic_disable_bdof_flag | u(1) |
| if( sps_dmvr_pic_present_flag ) |  |
| pic_disable_dmvr_flag | u(1) |
| if( sps_prof_pic_present_flag ) |  |
| pic_disable_prof_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ) |  |
| pic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| } |  |
| if ( sps_ibc_enabled_flag ) |  |
| pic_six_minus_max_num_ibc_merge_cand | ue(v) |
| … |  |

- 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_order_cnt_lsb | u(v) |
| if( subpics_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( rect_slice_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| if( pic_inter_slice_present_flag ) |  |
| slice_type | ue(v) |
| … |  |

- 7.4.3.6 Picture header RBSP semantics

pic_inter_slice_present_flag equal to 1 specifies that one or more slice with slice_type equal to 0 (B) or 1 (P) may be present in the picture associated with the PH. pic_inter_slice_present_flag equal to 0 specifies that no slice with slice_type equal to 0 (B) or 1 (P) can be present in the picture associated with the PH.

pic_intra_slice_present_flag equal to 1 specifies that one or more slice with slice_type equal to 2 (I) may be present in the picture associated with the PH. pic_intra_slice_present_flag is equal to 0 specifies that no slice with slice_type equal to 2 (I) can be present in the picture associated with the PH. When not present, the value of pic_intra_slice_only_flag is inferred to be equal to 1.

NOTE – : The values of both pic_inter_slice_present_flag and pic_intra_slice_present_flag are set equal to 1 in the picture header associated with picture containing one or more subpicture(s) containing intra coded slice(s) which may be merged with one or more subpicure(s) containing inter coded slices(s).

- 7.4.8.1 General slice header semantics

slice_type specifies the coding type of the slice according to Table 7-5.

Table 7-5 – Name association to slice_type

| slice_type | Name of slice_type |
| --- | --- |
| 0 | B (B slice) |
| 1 | P (P slice) |
| 2 | I (I slice) |

When nal_unit_type is a value of nal_unit_type in the range of IDR_W_RADL to CRA_NUT, inclusive, and the current picture is the first picture in an access unit, slice_type shall be equal to 2.

When not present, the value of slice_type is infer to be equal to 2.

When pic_intra_slice_present_flag is equal to 0, the value of slice_type shall be in the range from 0 to 1, inclusive.