| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0824-v1 |
| --- | --- |

| Title: | Suggested Common Test Conditions and Reference Software Configurations for Lossless and Mixed Lossy/Lossless Applications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | T. Nguyen T.-C. Ma A. Nalci | Email: | tung.nguyen@hhi.fraunhofer.de tsung-chuanma@kwai.com analci@qti.qualcomm.com |
| Source: | CE3 Coordinators |  |  |

_____________________________

# Abstract

This document defines common test conditions (CTC) and reference software (VTM) configurations for lossless and mixed lossy/lossless scenarios. The test scenarios cover applications using either 4:2:0 or 4:4:4 chroma formats, and YCbCr or RGB input signals for the 4:4:4 chroma format case. The test sets include camera captured and screen content.

# Test Scenarios

The CTC described in this document are defined for conducting experiments in a controlled environment where the outcome of the experiments can be compared. The four test scenarios for this CTC are as follows:

- Lossless 4:2:0 (camera captured and screen content)

- Lossless 4:4:4 (camera captured and screen content + RGB)

- Mixed Lossy/Lossless 4:2:0

- Mixed Lossy/Lossless 4:2:0

The test scenarios are a combination between the test sets and software configurations:

- Test set (4:2:0 or 4:4:4)

- Lossless or mixed lossy/lossless

For all test scenarios, three temporal configurations are mandatory:

- All-Intra (AI)

- Random-Access (RA)

- Low-delay B (LB)

The temporal configurations and encoder parameters are according to the JVET CTC, and are overwritten by the additional encoder parameters provided in this document.

# Test Set

Test sequences are available from the following location:

- ftp://hevc@ftp.tnt.uni-hannover.de/testsequences/FrExt-candidate-sequences/ (please contact the JCT-VC chairs for login information).

- ftp://vqeg.its.bldrdoc.gov/HDTV/SVT_MultiFormat/1080p50_CgrLevels_SINC_FILTER_SVTdec05/

Note that test sequences from the EBU (i.e. with the string ‘EBU’ included in the test sequence name) are password protected.  The password can be obtained by registering at the following location:

- http://tech.ebu.ch/testsequences/uhd-1_public_form

YCbCr and RGB 4:4:4 test sequences are available from the following locations (please contact JCT-VC/JVET chairs for login information):

- (YCbCr only) ArenaOfValor 4:4:4 and GlassHalf 4:4:4:
ftp://ftp.ient.rwth-aachen.de.

- Other YCbCr and RGB 4:4:4 sequences:
ftp://hevc@ftp.tnt.uni-hannover.de/testsequences/FrExt-candidate-sequences/upload/screen_content/ScExt-TestSequences.

## 4:2:0 Content

Not all sequences are mandatory for all temporal configurations according to the JVET CTC. The following table lists the sequences and whether or not they are mandatory for a given temporal configuration, and whether or not they are included in the mixed lossy/lossless test scenario.

| Class | Sequence name | Frame count | Frame rate | Bit depth | Intra | Random access | Low-delay | Lossy Lossless |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | Tango2 | 294 | 60 | 10 | M | M | - | TBD |
| A1 | FoodMarket4 | 300 | 60 | 10 | M | M | - | TBD |
| A1 | Campfire | 300 | 30 | 10 | M | M | - | TBD |
| A2 | CatRobot | 300 | 60 | 10 | M | M | - | TBD |
| A2 | DaylightRoad2 | 300 | 60 | 10 | M | M | - | TBD |
| A2 | ParkRunning3 | 300 | 50 | 10 | M | M | - | TBD |
| B | MarketPlace | 600 | 60 | 10 | M | M | M | TBD |
| B | RitualDance | 600 | 60 | 10 | M | M | M | TBD |
| B | Cactus | 500 | 50 | 8 | M | M | M | TBD |
| B | BasketballDrive | 500 | 50 | 8 | M | M | M | TBD |
| B | BQTerrace | 600 | 60 | 8 | M | M | M | TBD |
| C | RaceHorses | 300 | 30 | 8 | M | M | M | TBD |
| C | BQMall | 600 | 60 | 8 | M | M | M | TBD |
| C | PartyScene | 500 | 50 | 8 | M | M | M | TBD |
| C | BasketballDrill | 500 | 50 | 8 | M | M | M | TBD |
| D | RaceHorses | 300 | 30 | 8 | M | M | M | TBD |
| D | BQSquare | 600 | 60 | 8 | M | M | M | TBD |
| D | BlowingBubbles | 500 | 50 | 8 | M | M | M | TBD |
| D | BasketballPass | 500 | 50 | 8 | M | M | M |  |
| E | FourPeople | 600 | 60 | 8 | M | - | M |  |
| E | Johnny | 600 | 60 | 8 | M | - | M |  |
| E | KristenAndSara | 600 | 60 | 8 | M | - | M |  |
| F | ArenaOfValor | 600 | 60 | 8 | M | M | M |  |
| F | BasketballDrillText | 500 | 50 | 8 | M | M | M |  |
| F | SlideEditing | 300 | 30 | 8 | M | M | M |  |
| F | SlideShow | 500 | 20 | 8 | M | M | M |  |
| TGM | TBD | TBD | TBD | TBD | M | M | M |  |
| TGM | TBD | TBD | TBD | TBD | M | M | M |  |
| TGM | TBD | TBD | TBD | TBD | M | M | M |  |
| TGM | TBD | TBD | TBD | TBD | M | M | M |  |

## 4:4:4 Content

For the 4:4:4 test set, all sequences are mandatory for all temporal configurations.

| Class | Sequence name | Bit depth | Frame rate | Frame count | Lossly Lossless |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p RGB444 | sc_flyingGraphics_1920x1080_60_8bit.rgb sc_desktop_1920x1080_60_8bit.rgb sc_console_1920x1080_60_8bit.rgb ChineseEditing_1920x1080_60_8bit.rgb | 8 8 8 8 | 60 60 60 60 | 0-299* 0-599 0-599 0-599 | TBD |
| TGM 720p RGB444 | sc_web_browsing_1280x720_30_8bit.rgb sc_map_1280x720_60_8bit.rgb sc_programming_1280x720_60_8bit.rgb sc_SlideShow_1280x720_20_8bit.rgb | 8 8 8 8 | 30 60 60 20 | 0-299 0-599 0-599 0-499 | TBD |
| Animation RGB444 | ArenaOfValor_1920x1080_60_8bit_444.rgb sc_robot_1280x720_30_8bit.rgb Glasshalf_3840x2160_24p_10b_444.rgb [note: do we need RGB444 for SC? These two need conversion] | 8 8 10 | 60 30 24 | 0-599 0-299 0-239 | TBD |
| Mixed Content RGB444 | Basketball_Screen_2560x1440_60p_8b444.rgb MissionControlClip2_2560x1440_60p_8444.rgb MissionControlClip3_1920x1080_60p_8b444.rgb | 8 8 8 | 60 60 60 | 322-621* 120-419* 0-599 | TBD |
| TGM 1080p YCbCr444 | sc_flyingGraphics_1920x1080_60_8bit.yuv sc_desktop_1920x1080_60_8bit.yuv sc_console_1920x1080_60_8bit.yuv ChineseEditing_1920x1080_60_8bit.yuv | 8 8 8 8 | 60 60 60 60 | 0-299* 0-599 0-599 0-599 | TBD |
| TGM 720p YCbCr444 | sc_web_browsing_1280x720_30_8bit.yuv sc_map_1280x720_60_8bit.yuv sc_programming_1280x720_60_8bit.yuv sc_SlideShow_1280x720_20_8bit.yuv | 8 8 8 8 | 30 60 60 20 | 0-299 0-599 0-599 0-499 | TBD |
| Animation YCbCr444 | ArenaOfValor_1920x1080_60_8bit_444.yuv sc_robot_1280x720_30_8bit.yuv Glasshalf_3840x2160_24p_10b_444.yuv | 8 8 10 | 60 30 24 | 0-599 0-299 0-239 | TBD |
| Mixed Content YCbCr444 | Basketball_Screen_2560x1440_60p_8b444.yuv MissionControlClip2_2560x1440_60p_8444.yuv MissionControlClip3_1920x1080_60p_8b444.yuv | 8 8 8 | 60 60 60 | 322-621* 120-419* 0-599 | TBD |

- *Note that the frames used are different from the original number of frames in the sequence

- TGM: Text and graphics with motion; M: mixed content; A: animation; CC: camera-captured content

# Software configuration

VTM-7.0 and later provides full-frame lossless operation by using a dedicated lossless configuration. It can be enabled at the encoder by using the provided configuration file “lossless.cfg” additionally to the CTC configuration files.

For all lossless and mixed lossy/lossless configurations, only 100 frames shall be encoded (e.g., by using the encoder option “--FramesToBeEncoded=100”).

According to the JVET CTC, for 4:2:0 test set, the following encoder parameters are mandatory for the class F and the TGM sequences:

- --IBC=1		### enable intra block copy

- --BDPCM=2	### enable BDPCM

- --HashME=1	### enable HashME

For the 4:4:4 test set, the following encoder parameters are mandatory:

- --IBC=1		### enable intra block copy

- --BDPCM=2	### enable BDPCM

- --HashME=1	### enable HashME

- --PLT=1		### enable palette mode

For the RGB sequences of the 4:4:4 test set, the following encoder parameters are mandatory:

- --ColorTransform-1							### enable ACT

- --Log2MaxTbSize=5						### restrict maximum transform size

- --InputColourSpaceConvert=RGBtoGBR	### encoding/decoding in GBR

The provided lossless configuration has the following encoder parameters configuration:

- --CostMode=lossless

- --ChromaTS=1

- --DepQuant=0

- --RDOQ=0

- --RDOQTS=0

- --SBT=0

- --LMCSEnable=0

- --ISP=0

- --MTS=0

- --LFNST=0

- --JointCbCr=0

- --LoopFilterDisable=1

- --SAO=0

- --ALF=0

- --DMVR=0

- --BIO=0

- --PROF=0

- --Log2MaxTbSize=5

- --InternalBitDepth= 0

For mixed lossy/lossless configuration, this document provides additional sequence specific configurations files. TBD

# Evaluation

The measurement relies on the compression ratio relative to the uncompressed raw data. A bit-rate saving or overhead is then calculated by comparing the compression ratio of the test relative to the VTM-8.0 anchor.

# Appendix

MD5 checksums for the test sequences:

| Class | Sequence name | MD5 checksum |
| --- | --- | --- |
| A1 | Tango2 | 0471a59c423b7059c5c6c8b395e864a9 |
| A1 | FoodMarket4 | a378b34190f54f688d048a9a8b46a8ac |
| A1 | Campfire | 63d3d9f9e4e8b5c344e89840e84e6428 |
| A2 | CatRobot | 03a89792693fd9ecfd72ef2590025e97 |
| A2 | DaylightRoad2 | bf1d22643afb41b1521749d2749fb5f0 |
| A2 | ParkRunning3 | e7a1d1ebff269767ec4bffd2998d1154 |
| B | MarketPlace | dc668e7f28541e4370bdbdd078e61bba |
| B | RitualDance | a3cb399a7b92eb9c5ee0db340abc43e4 |
| B | Cactus | 3fddb71486f209f1eb8020a0880ddf82 |
| B | BasketballDrive | d38951ad478b34cf988d55f9f1bf60ee |
| B | BQTerrace | efde9ce4197dd0b3e777ad32b24959cc |
| C | RaceHorses | 0a351df99f22d837bc528bd4901c6968 |
| C | BQMall | f889efea02b0c9a7d174b0f7a99cb51b |
| C | PartyScene | 4766c455665b6d228a6390e3d3ff2647 |
| C | BasketballDrill | bd215136fed04067d82c10b2e49b2c7c |
| D | RaceHorses | 290a63e86213abc4459fce1dbd39edbe |
| D | BQSquare | 713ef64958345859b9bae986c3a3f763 |
| D | BlowingBubbles | 50a520722f0e906b7884b6b9fea48699 |
| D | BasketballPass | bfd9abbdc677790130dc4023b4e409f0 |
| E | FourPeople | 4ce5d72311b32acce62614f63225fba5 |
| E | Johnny | be83259b3ccdada2213fbd8dea20bf6e |
| E | KristenAndSara | aa3931974e34deba15a1018ba3bf5e0c |
| F | ArenaOfValor | 26ca111ff83ecf4e9c9e3de3dc7d1fe4 |
| F | BasketballDrillText | 480881343081e3f4adf85075a1ed0f08 |
| F | SlideEditing | dbf1d0e765ba9f775c5208adb4dc6c0c |
| F | SlideShow | d6b28f127b9ddf088eb9db9fc5e611bd |
| TGM | TBD | TBD |
| TGM | TBD | TBD |
| TGM | TBD | TBD |
| TGM | TBD | TBD |