| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0122 |
| --- | --- |

| Title: | AHG9: On picture output for non-reference pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, it is proposed that the pic_output_flag is not signaled in picture header and inferred to be equal to 1, when the value of non_reference_picture_flag is equal to 1. A coded picture with non_reference_picture_flag equal to 1 and pic_output_flag equal to 0 should not be included in the bitstream, becaue such a picture is never outputted and never used as a reference.

# Introduction

A picture with non_reference_picture_flag equal to 1 is never used as a reference picture of the following pictures. If the pic_output_flag value of the non-reference picture is not equal to 1, the picture is not never outputted and never used for reference. Thus, a coded picture with non_reference_picture_flag equal to 1 and pic_output_flag equal to 0 is a useless picture and should not be included in the bitstream. To enforce that the non-refernce and non-output picture is not included in the bitstream and save a bit in picture header, the following modification in the picture header syntax structure is proposed.

# Proposed Text

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| if( gdr_or_irap_pic_flag ) |  |
| gdr_pic_flag | u(1) |
| ph_inter_slice_allowed_flag | u(1) |
| if( ph_inter_slice_allowed_flag ) |  |
| ph_intra_slice_allowed_flag | u(1) |
| non_reference_picture_flag | u(1) |
| … |  |
| if( output_flag_present_flag && !non_reference_picture_flag) |  |
| pic_output_flag | u(1) |
| if( rpl_info_in_ph_flag ) |  |
| ref_pic_lists( ) |  |
| … |  |
| } |  |

pic_output_flag affects the decoded picture output and removal processes as specified in Annex C. When pic_output_flag is not present, it is inferred to be equal to 1.

NOTE  – A coded picture with non_reference_picture_flag equal to 1 and pic_output_flag equal to 0 should not be included in the bitstream.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).