| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0283 |
| --- | --- |

| Title: | AhG 9: Cleanup of inter predication HLS syntax elements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Fabrice Le Léannec Michel Kerdranvat Philippe De Lagrange | Tel: Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

For intra-only configuration/profile, the draft VVC specification has several inter-related HLS syntax elements that are redundantly coded. It is asserted that the number of HLS syntax elements is more than 5 times more than in HEVC. This contribution therefore proposes a method for reducing this overhead by defining an SPS and PPS flag to indicate if intra-only is used.

# Introduction

In VVC draft 8 [1], many non-intra high-level SPS syntax elements are coded regardless if all-intra profile is used. That is, the inter-related SPS syntax elements are:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_weighted_pred_flag | u(1) |
| sps_weighted_bipred_flag | u(1) |
| … |  |
| long_term_ref_pics_flag | u(1) |
| inter_layer_ref_pics_present_flag | u(1) |
| sps_idr_rpl_present_flag | u(1) |
| rpl1_same_as_rpl0_flag | u(1) |
| for( i = 0; i < rpl1_same_as_rpl0_flag ? 1 : 2; i++ ) { |  |
| num_ref_pic_lists_in_sps[ i ] | ue(v) |
| for( j = 0; j < num_ref_pic_lists_in_sps[ i ]; j++) |  |
| ref_pic_list_struct( i, j ) |  |
| } |  |
| ... |  |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( sps_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| … |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_bdof_enabled_flag | u(1) |
| if( sps_bdof_enabled_flag ) |  |
| sps_bdof_pic_present_flag | u(1) |
| sps_smvd_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| if( sps_dmvr_enabled_flag) |  |
| sps_dmvr_pic_present_flag | u(1) |
| sps_mmvd_enabled_flag | u(1) |
| } |  |
| … |  |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| … |  |
| six_minus_max_num_merge_cand | ue(v) |
| sps_sbt_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| sps_affine_type_flag | u(1) |
| if( sps_amvr_enabled_flag ) |  |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| if( sps_affine_prof_enabled_flag ) |  |
| sps_prof_pic_present_flag | u(1) |
| } |  |
| … |  |
| sps_bcw_enabled_flag | u(1) |
| sps_ibc_enabled_flag | u(1) |
| if( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_ibc_merge_cand | ue(v) |
| sps_ciip_enabled_flag | u(1) |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| if( MaxNumMergeCand >= 2 ) { |  |
| sps_gpm_enabled_flag | u(1) |
| if( sps_gpm_enabled_flag && MaxNumMergeCand >= 3 ) |  |
| max_num_merge_cand_minus_max_num_gpm_cand | ue(v) |
| } |  |
| … |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| … |  |

That is, there are more than 40 SPS syntax elements related to inter-tools. Comparatively, in HEVC, only 7 elements are coded:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| amp_enabled_flag | u(1) |
| … |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enable_flag | u(1) |
| … |  |

That is, the number of non-intra elements is more than 5 times higher in VVC compared to HEVC. Therefore, better coding mechanism is needed where redundant information is not coded but inferred.

Apart from SPS, same redundancy is found in picture parameters set (PPS) and also in constraint flags. Specifically, for PPS, the following inter elements are coded:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| rpl1_idx_present_flag | u(1) |
| … |  |
| pps_weighted_pred_flag | u(1) |
| pps_weighted_bipred_flag | u(1) |
| … |  |
| rpl_info_in_ph_flag | u(1) |
| … |  |
| if( ( pps_weighted_pred_flag ¦ ¦ pps_weighted_bipred_flag ) && rpl_info_in_ph_flag ) |  |
| wp_info_in_ph_flag | u(1) |
| …. |  |
| pps_ref_wraparound_enabled_flag | u(1) |
| if( pps_ref_wraparound_enabled_flag ) |  |
| pps_ref_wraparound_offset | ue(v) |
| … |  |

That is, 12 inter-specific elements need to be coded even for all-intra profile.

# Proposal 1: SPS Redundancy

To remove the redundant coding in the SPS, as well as the subsequent levels, an SPS flag is added to indicate of intra-only is used, or alternatively expressed, if inter-coding is not allowed. The following modification is done to SPS:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_inter_slice_allowed_flag | u(1) |
| if (sps_inter_slice_allowed_flag){ |  |
| sps_weighted_pred_flag | u(1) |
| sps_weighted_bipred_flag | u(1) |
| } |  |
| … |  |
| if (sps_inter_slice_allowed_flag){ |  |
| long_term_ref_pics_flag | u(1) |
| inter_layer_ref_pics_present_flag | u(1) |
| sps_idr_rpl_present_flag | u(1) |
| rpl1_same_as_rpl0_flag | u(1) |
| for( i = 0; i < rpl1_same_as_rpl0_flag ? 1 : 2; i++ ) { |  |
| num_ref_pic_lists_in_sps[ i ] | ue(v) |
| for( j = 0; j < num_ref_pic_lists_in_sps[ i ]; j++) |  |
| ref_pic_list_struct( i, j ) |  |
| } |  |
| } |  |
| ... |  |
| if (sps_inter_slice_allowed_flag){ |  |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( sps_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| } |  |
| … |  |
| if (sps_inter_slice_allowed_flag){ |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_bdof_enabled_flag | u(1) |
| if( sps_bdof_enabled_flag ) |  |
| sps_bdof_pic_present_flag | u(1) |
| sps_smvd_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| if( sps_dmvr_enabled_flag) |  |
| sps_dmvr_pic_present_flag | u(1) |
| sps_mmvd_enabled_flag | u(1) |
| } |  |
| } |  |
| … |  |
| if( sps_mts_enabled_flag ) { |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| if (sps_inter_slice_allowed_flag) |  |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| } |  |
| … |  |
| if (sps_inter_slice_allowed_flag){ |  |
| six_minus_max_num_merge_cand | ue(v) |
| sps_sbt_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| sps_affine_type_flag | u(1) |
| if( sps_amvr_enabled_flag ) |  |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| if( sps_affine_prof_enabled_flag ) |  |
| sps_prof_pic_present_flag | u(1) |
| } |  |
| } |  |
| … |  |
| if (sps_inter_slice_allowed_flag){ |  |
| sps_bcw_enabled_flag | u(1) |
| sps_ibc_enabled_flag | u(1) |
| if( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_ibc_merge_cand | ue(v) |
| sps_ciip_enabled_flag | u(1) |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| if( MaxNumMergeCand >= 2 ) { |  |
| sps_gpm_enabled_flag | u(1) |
| if( sps_gpm_enabled_flag && MaxNumMergeCand >= 3 ) |  |
| max_num_merge_cand_minus_max_num_gpm_cand | ue(v) |
| } |  |
| } |  |
| … |  |
| if (sps_inter_slice_allowed_flag) |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| … |  |

Clearly, this method avoids coding several SPS flags in intra-only configuration. These flags shall be inferred as zero when not present.

The semantic of this flag is:

sps_inter_slice_allowed_flag equal to 0 specifies inter slices are not allowed. sps_inter_slice_allowed_flag equal to 1 specifies inter slices maybe allowed.

This added SPS flag is further used to improve the picture header coding. That is, if inter-slices are not allowed, there is no need to signal at PH level if inter is allowed. Therefore, the following modifications are made:

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| if (sps_inter_slice_allowed_flag) |  |
| ph_inter_slice_allowed_flag | u(1) |
| … |  |

Its semantic is modified as follows:

ph_inter_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have slice_type equal to 2. ph_inter_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have slice_type equal to 0 or 1. If not present, the value of ph_inter_slice_allowed_flag is inferred to be equal to zero.

Finally, the constraint information flag related to intra-only can be modified as follows:

intra_only_constraint_flag equal to 1 specifies that slice_type sps_inter_slice_allowed_flag shall be equal to I 0. intra_only_constraint_flag equal to 0 does not impose such a constraint.

# Proposal 2: PPS Redundancy

Similar to SPS, PPS has also redundant information of inter mode. It is proposed here to add a PPS flag to indicate if inter slices are allowed:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_inter_slice_allowed_flag | u(1) |
| if (pps_inter_slice_allowed_flag) |  |
| rpl1_idx_present_flag | u(1) |
| … |  |
| if (pps_inter_slice_allowed_flag){ |  |
| pps_weighted_pred_flag | u(1) |
| pps_weighted_bipred_flag | u(1) |
| } |  |
| … |  |
| if (pps_inter_slice_allowed_flag) |  |
| rpl_info_in_ph_flag | u(1) |
| … |  |
| if( ( pps_weighted_pred_flag ¦ ¦ pps_weighted_bipred_flag ) && rpl_info_in_ph_flag && pps_inter_slice_allowed_flag ) |  |
| wp_info_in_ph_flag | u(1) |
| …. |  |
| if (pps_inter_slice_allowed_flag) |  |
| pps_ref_wraparound_enabled_flag | u(1) |
| if( pps_ref_wraparound_enabled_flag ) |  |
| pps_ref_wraparound_offset | ue(v) |
| … |  |

The values of non-coded flags shall be inferred to zero.

The semantic of the added flag is:

pps_inter_slice_allowed_flag equal to 0 specifies inter slices are not allowed. pps_inter_slice_allowed_flag equal to 1 specifies inter slices maybe allowed. It is requirement of bitstream conformance that pps_inter_slice_allowed_flag value is equal to sps_inter_slice_allowed_flag.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).