| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0231-v1 |
| --- | --- |

| Title: | On Signaling of TU Luma Coded Flag for CU with ACT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lien-Fei Chen, Xiang Li, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A | Email: | lienfeichen@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

It is reported that there is a mismatch on the condition of tu_y_coded_flag signaling between VVC draft JVET-R2001-v10 and VTM-9. In this contribution, it is proposed to either align VTM to draft, or to align draft to VTM.

After aligning VTM to draft, the performance impact on RGB sequences over VTM-9 is reported as follows.

- AI: 0.00%/0.02%/0.0%

- RA:

- LB: -0.01%/-0.01%/0.04%

- LP: 0.02%/-0.03%/0.01%

And the performance impact on RGB SCC content is reported as follows.

- AI: 0.11%/0.10%/0.11%

- RA:

- LB: 0.01%/0.01%/0.03%

- LP: 0.05%/0.00%/0.06%

Align draft to VTM reportedly shows identical results to VTM-9.0.

# Problem Statement

In current VVC draft spec [1], the TU coded flag of luminance component is signaled for the intra block when the TU coded flags of the chrominance components are both zero no matter whether the CU is ACT or not as shown in Table 1. However, ACT mode should not be signalled if the CU doesn’t have any coefficient. Therefore, in VTM-9.0 software, the TU coded flag of luminance is inferred to 1 when the TU coded flag of chrominance channels are both zero and the current CU is an intra block and is also coded with ACT mode. The corresponding software description is shown in Table 2.

Table 1 TU coded flag for three color channels in VVC draft spec.

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && ChromaArrayType != 0 && ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cb_coded_flag[ xC ][ yC ] | ae(v) |
| tu_cr_coded_flag[ xC ][ yC ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) && ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ ( chromaAvailable && ( tu_cb_coded_flag[ xC ][ yC ] ¦ ¦ tu_cr_coded_flag[ xC ][ yC ] ) ) ¦ ¦ CbWidth[ chType ][ x0 ][ y0 ] > MaxTbSizeY ¦ ¦ CbHeight[ chType ][ x0 ][ y0 ] > MaxTbSizeY ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex < NumIntraSubPartitions − 1 ¦ ¦ !InferTuCbfLuma ) ) ) |  |
| tu_y_coded_flag[ x0 ][ y0 ] | ae(v) |
| if(IntraSubPartitionsSplitType != ISP_NO_SPLIT ) |  |
| InferTuCbfLuma = InferTuCbfLuma && !tu_y_coded_flag[ x0 ][ y0 ] |  |
| } |  |

Table 2 TU coded flag for luminance component in VTM-9.0.

if (!isChroma(partitioner.chType)) {

if (!CU::isIntra(cu) && trDepth == 0 && !chromaCbfs.sigChroma(area.chromaFormat)) {

TU::setCbfAtDepth(tu, COMPONENT_Y, trDepth, 1);

}

else if (cu.sbtInfo && tu.noResidual) {

TU::setCbfAtDepth(tu, COMPONENT_Y, trDepth, 0);

}

else if (cu.sbtInfo && !chromaCbfs.sigChroma(area.chromaFormat)) {

assert(!tu.noResidual);

TU::setCbfAtDepth(tu, COMPONENT_Y, trDepth, 1);

}

else {

bool lumaCbfIsInferredACT = (cu.colorTransform && cu.predMode == MODE_INTRA &&

trDepth == 0 && !chromaCbfs.sigChroma(area.chromaFormat));

bool lastCbfIsInferred    = lumaCbfIsInferredACT; // ISP and ACT are mutually exclusive

bool previousCbf          = false;

bool rootCbfSoFar         = false;

if (cu.ispMode) {

...

}

bool cbfY = lastCbfIsInferred ? true : cbf_comp(cs, tu.Y(), trDepth, previousCbf, cu.ispMode);

TU::setCbfAtDepth(tu, COMPONENT_Y, trDepth, (cbfY ? 1 : 0));

}

}

According to the above two tables, there is a mismatch behavior about the TU coded flag for luminance component between the SW and draft text.

# Proposed Solution

## Alignment of SW to spec

In VTM-9.0, the “lastCbfIsInferred” signal in Table 2 is forced to zero according to the table syntax in Table 1. Therefore, the SW fix for the alignment of SW to spec is shown as Table 3. The modification part is highlighted in yellow, and deleted statement is marked by red strikethrough.

if (!isChroma(partitioner.chType)) {

if (!CU::isIntra(cu) && trDepth == 0 && !chromaCbfs.sigChroma(area.chromaFormat)) {

TU::setCbfAtDepth(tu, COMPONENT_Y, trDepth, 1);

}

else if (cu.sbtInfo && tu.noResidual) {

TU::setCbfAtDepth(tu, COMPONENT_Y, trDepth, 0);

}

else if (cu.sbtInfo && !chromaCbfs.sigChroma(area.chromaFormat)) {

assert(!tu.noResidual);

TU::setCbfAtDepth(tu, COMPONENT_Y, trDepth, 1);

}

else {

bool lumaCbfIsInferredACT = (cu.colorTransform && cu.predMode == MODE_INTRA &&

trDepth == 0 && !chromaCbfs.sigChroma(area.chromaFormat));

bool lastCbfIsInferred    = false;

bool previousCbf          = false;

bool rootCbfSoFar         = false;

if (cu.ispMode) {

...

}

bool cbfY = lastCbfIsInferred ? true : cbf_comp(cs, tu.Y(), trDepth, previousCbf, cu.ispMode);

TU::setCbfAtDepth(tu, COMPONENT_Y, trDepth, (cbfY ? 1 : 0));

}

}

Table 3 SW fix for the alignment of SW to draft spec

## Alignment of spec to SW

Another method is trying to align the spec text to SW behavior. In order to achieve the behavior that the ACT mode should not be signaled anymore if the coded CU block doesn’t have any coefficient. For the coded block with zero TU coded flags for the chrominance channels, the TU coded flag for luminance channel should be inferred to 1 if the current CU is coded with ACT mode. The corresponding added text is highlighted in yellow.

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && ChromaArrayType != 0 && ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cb_coded_flag[ xC ][ yC ] | ae(v) |
| tu_cr_coded_flag[ xC ][ yC ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) && ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && !cu_act_enabled_flag ) ¦ ¦ ( chromaAvailable && ( tu_cb_coded_flag[ xC ][ yC ] ¦ ¦ tu_cr_coded_flag[ xC ][ yC ] ) ) ¦ ¦ CbWidth[ chType ][ x0 ][ y0 ] > MaxTbSizeY ¦ ¦ CbHeight[ chType ][ x0 ][ y0 ] > MaxTbSizeY ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex < NumIntraSubPartitions − 1 ¦ ¦ !InferTuCbfLuma ) ) ) |  |
| tu_y_coded_flag[ x0 ][ y0 ] | ae(v) |
| if(IntraSubPartitionsSplitType != ISP_NO_SPLIT ) |  |
| InferTuCbfLuma = InferTuCbfLuma && !tu_y_coded_flag[ x0 ][ y0 ] |  |
| } |  |
| if( ( CbWidth[ chType ][ x0 ][ y0 ] > 64 ¦ ¦ CbHeight[ chType ][ x0 ][ y0 ] > 64 ¦ ¦ tu_y_coded_flag[ x0 ][ y0 ] ¦ ¦ ( chromaAvailable && ( tu_cb_coded_flag[ xC ][ yC ] ¦ ¦ tu_cr_coded_flag[ xC ][ yC ] ) ) && treeType != DUAL_TREE_CHROMA && pps_cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta_abs | ae(v) |
| if( cu_qp_delta_abs ) |  |
| cu_qp_delta_sign_flag | ae(v) |
| } |  |

if (!isChroma(partitioner.chType)) {

if (!(CU::isIntra(cu) && !cu.colorTransform) && trDepth == 0 && !chromaCbfs.sigChroma(area.chromaFormat)) {

TU::setCbfAtDepth(tu, COMPONENT_Y, trDepth, 1);

}

else if (cu.sbtInfo && tu.noResidual) {

TU::setCbfAtDepth(tu, COMPONENT_Y, trDepth, 0);

}

else if (cu.sbtInfo && !chromaCbfs.sigChroma(area.chromaFormat)) {

assert(!tu.noResidual);

TU::setCbfAtDepth(tu, COMPONENT_Y, trDepth, 1);

}

else {

bool lumaCbfIsInferredACT = (cu.colorTransform && cu.predMode == MODE_INTRA &&

trDepth == 0 && !chromaCbfs.sigChroma(area.chromaFormat));

bool lastCbfIsInferred    = false;

bool previousCbf          = false;

bool rootCbfSoFar         = false;

if (cu.ispMode) {

...

}

bool cbfY = lastCbfIsInferred ? true : cbf_comp(cs, tu.Y(), trDepth, previousCbf, cu.ispMode);

TU::setCbfAtDepth(tu, COMPONENT_Y, trDepth, (cbfY ? 1 : 0));

}

}

In order to improve the readability the SW may be revised as follows, where the modifications are highlighted in yellow, and deleted statement is marked by red strikethrough. Note that the changes have no impact on coding performance

# Experimental Results

The proposed methods have been implemented on top of VTM-9.0, and the experimental results are simulated based on the common test conditions defined in JVET-Q2013[2] and JVET-R0468[3].

## Alignment of SW to spec

For the RGB444 natural scene, the RD performance result is shown as follows

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | -0.03% | 0.02% | -0.02% | 99% | 101% |
| Kimono | 0.01% | 0.01% | -0.04% | 100% | 97% |
| EBULupoCandlelight | 0.03% | 0.06% | 0.03% | 101% | 96% |
| EBURainFruits | -0.03% | 0.00% | -0.09% | 104% | 100% |
| VenueVu | 0.09% | 0.05% | 0.07% | 106% | 110% |
| DucksAndLegs | 0.03% | -0.01% | 0.04% | 102% | 103% |
| OldTownCross | -0.06% | 0.02% | 0.00% | 102% | 102% |
| ParkScene | -0.02% | 0.00% | 0.03% | 100% | 103% |
| Overall | 0.00% | 0.02% | 0.00% | 102% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | VTM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | -0.01% | 0.01% | -0.03% | 100% | 99% |
| Kimono | 0.04% | -0.05% | -0.13% | 100% | 98% |
| EBULupoCandlelight | -0.10% | -0.09% | -0.07% | 101% | 97% |
| EBURainFruits | 0.01% | -0.09% | -0.03% | 102% | 102% |
| VenueVu | #VALUE! | #VALUE! | #VALUE! | 101% | 103% |
| DucksAndLegs | -0.11% | -0.01% | -0.03% | 101% | 99% |
| OldTownCross | #VALUE! | #VALUE! | #VALUE! | 100% | 99% |
| ParkScene | -0.09% | -0.01% | 0.00% | 100% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | VTM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | -0.22% | -0.08% | -0.11% | 99% | 98% |
| Kimono | -0.02% | 0.01% | -0.01% | 100% | 101% |
| EBULupoCandlelight | -0.07% | -0.08% | 0.19% | 100% | 99% |
| EBURainFruits | 0.03% | -0.02% | -0.02% | 100% | 97% |
| VenueVu | -0.03% | 0.09% | 0.09% | 100% | 98% |
| DucksAndLegs | -0.04% | 0.01% | 0.02% | 99% | 99% |
| OldTownCross | 0.28% | 0.00% | 0.15% | 98% | 104% |
| ParkScene | -0.02% | 0.00% | 0.00% | 100% | 100% |
| Overall | -0.01% | -0.01% | 0.04% | 101% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | VTM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | -0.04% | -0.06% | 0.03% | 100% | 100% |
| Kimono | 0.06% | 0.04% | -0.02% | 101% | 101% |
| EBULupoCandlelight | -0.32% | -0.20% | 0.00% | 101% | 97% |
| EBURainFruits | 0.01% | -0.02% | 0.01% | 101% | 99% |
| VenueVu | 0.05% | 0.05% | 0.00% | 102% | 100% |
| DucksAndLegs | 0.17% | 0.03% | 0.06% | 100% | 99% |
| OldTownCross | 0.26% | -0.07% | 0.07% | 99% | 102% |
| ParkScene | -0.03% | -0.01% | -0.04% | 101% | 96% |
| Overall | 0.02% | -0.03% | 0.01% | 101% | 101% |

In the case of RGB444 SCC content, the RD performance result is shown as follows

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.04% | 0.05% | 0.01% | 100% | 99% |
| TGM 720p | 0.13% | 0.15% | 0.16% | 101% | 103% |
| Animation | 0.26% | 0.22% | 0.24% | 102% | 101% |
| Mixed content | 0.02% | -0.01% | 0.03% | 101% | 96% |
| Overall | 0.11% | 0.10% | 0.11% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| TGM 720p | 0.02% | 0.05% | 0.03% | 101% | 102% |
| Animation | -0.01% | 0.08% | 0.04% | 101% | 101% |
| Mixed content | 0.01% | 0.04% | 0.12% | 101% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.02% | 0.03% | -0.01% | 102% | 102% |
| TGM 720p | 0.11% | 0.00% | 0.05% | 102% | 104% |
| Animation | -0.01% | 0.00% | 0.00% | 101% | 101% |
| Mixed content | -0.05% | -0.01% | 0.06% | 103% | 101% |
| Overall | 0.01% | 0.01% | 0.03% | 102% | 102% |
|  | Low delay P Main10 |  |  |  |  |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.01% | -0.02% | 0.02% | 102% | 98% |
| TGM 720p | 0.10% | 0.03% | 0.14% | 102% | 105% |
| Animation | 0.00% | 0.02% | 0.01% | 101% | 102% |
| Mixed content | 0.07% | -0.06% | 0.05% | 103% | 101% |
| Overall | 0.05% | 0.00% | 0.06% | 102% | 101% |

## Alignment of spec to SW

For the RGB444 natural scene, the RD performance result is shown as follows

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Kimono | 0.00% | 0.00% | 0.00% | 99% | 94% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 101% | 100% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 104% | 102% |
| VenueVu | 0.00% | 0.00% | 0.00% | 103% | 107% |
| DucksAndLegs | 0.00% | 0.00% | 0.00% | 101% | 102% |
| OldTownCross | 0.00% | 0.00% | 0.00% | 101% | 99% |
| ParkScene | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | VTM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Kimono | 0.00% | 0.00% | 0.00% | 101% | 99% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 100% | 100% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 102% | 102% |
| VenueVu | 0.00% | 0.00% | 0.00% | 102% | 104% |
| DucksAndLegs | 0.00% | 0.00% | 0.00% | 100% | 98% |
| OldTownCross | 0.00% | 0.00% | 0.00% | 101% | 100% |
| ParkScene | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | VTM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Kimono | 0.00% | 0.00% | 0.00% | 100% | 101% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 100% | 97% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 100% | 98% |
| VenueVu | 0.00% | 0.00% | 0.00% | 101% | 98% |
| DucksAndLegs | 0.00% | 0.00% | 0.00% | 100% | 98% |
| OldTownCross | 0.00% | 0.00% | 0.00% | 100% | 102% |
| ParkScene | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | VTM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Kimono | 0.00% | 0.00% | 0.00% | 100% | 101% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 100% | 96% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 101% | 98% |
| VenueVu | 0.00% | 0.00% | 0.00% | 101% | 100% |
| DucksAndLegs | 0.00% | 0.00% | 0.00% | 100% | 99% |
| OldTownCross | 0.00% | 0.00% | 0.00% | 100% | 98% |
| ParkScene | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |

In the case of RGB444 SCC content, the RD performance result is shown as follows

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 100% | 99% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 100% | 105% |
| Animation | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Mixed content | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| TGM 720p | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Animation | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Mixed content | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 102% | 100% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 102% | 105% |
| Animation | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Mixed content | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 102% | 100% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 102% | 106% |
| Animation | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Mixed content | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 102% |

# Conclusion

In this contribution, it is reported there is a mismatch behavior of TU coded flag for luminance component between SW and spec. It is proposed to either align software to spec., or align spec. to software. However, align software to spec. may lead to a case that a CU is signaled as ACT but has no coefficient, which is not consistent with transform design. Therefore, it is recommended to align spec. to software.

# References

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 9)”, JVET-R2001, 18th meeting, by teleconference, 15–24 April 2020.

- T.-H. Chao, Y.-C. Sun, J. Xu, and X. Xu, “JVET common test conditions and software reference configurations for non-4:2:0 colour formats”, JVET-Q2013, 17th meeting, Brussels, BE, 7–17 January 2020.

- Y.-H. Chao, W.-J. Chien, M. Karczewicz, X. Xiu, Y.-W. Chen, and X. Wang, “AHG13: on RGB common test condition“, JVET-R0468, 18th meeting, by teleconference, 15–24 April 2020.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).