| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0239 |
| --- | --- |

| Title: | AHG9: Clean-up on derivation of POC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Hendry Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, hm.jang, jaehyun.lim, dr.hendry, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In current VVC working draft, the MSB of POC is derived according to whether the flag, ph_poc_msb_cycle_present_flag is equal to 0 or not. However, ph_poc_msb_cycle_present_flag is not present when sps_poc_msb_cycle_flag is equal to 0 and currently there is no inference rule is specified when for ph_poc_msb_cycle_present_flag when it is not present. This issue has been reported in the VVC spec ticket #1118. To solve the above asserted bug, this contribution suggests to infer ph_poc_msb_present_flag to be equal to 0 when not present.

Problem Statement

In current VVC working draft, ph_poc_msb_cycle_present_flag is used to derive the MSB of POC as follows.

When ph_poc_msb_cycle_present_flag is equal to 0 and the current picture is not a CLVSS picture, the variables prevPicOrderCntLsb and prevPicOrderCntMsb are derived as follows: - Let prevTid0Pic be the previous picture in decoding order that has nuh_layer_id equal to the nuh_layer_id of the current picture, has TemporalId equal to 0, and is not a RASL or RADL picture. - The variable prevPicOrderCntLsb is set equal to ph_pic_order_cnt_lsb of prevTid0Pic. - The variable prevPicOrderCntMsb is set equal to PicOrderCntMsb of prevTid0Pic. The variable PicOrderCntMsb of the current picture is derived as follows: - If ph_poc_msb_cycle_present_flag is equal to 1, PicOrderCntMsb is set equal to ph_poc_msb_cycle_val * MaxPicOrderCntLsb. - Otherwise (ph_poc_msb_cycle_present_flag is equal to 0), if the current picture is a CLVSS picture, PicOrderCntMsb is set equal to 0. - Otherwise, PicOrderCntMsb is derived as follows: if( ( ph_pic_order_cnt_lsb < prevPicOrderCntLsb ) && ( ( prevPicOrderCntLsb − ph_pic_order_cnt_lsb ) >= ( MaxPicOrderCntLsb / 2 ) ) ) PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb (201) else if( ( ph_pic_order_cnt_lsb > prevPicOrderCntLsb ) && ( ( ph_pic_order_cnt_lsb − prevPicOrderCntLsb ) > ( MaxPicOrderCntLsb / 2 ) ) ) PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb else PicOrderCntMsb = prevPicOrderCntMsb PicOrderCntVal is derived as follows: PicOrderCntVal = PicOrderCntMsb + ph_pic_order_cnt_lsb (202)

However, since ph_poc_msb_cycle_present_flag is not present when sps_poc_msb_cycle_flag is equal to 0 as described in the snippet of syntax table below, decoder may derive the value of PicOrderCntMsb incorrectly.

<In picture header structure syntax>

| ph_pic_order_cnt_lsb | u(v) |
| --- | --- |
| if( sps_poc_msb_cycle_flag ) { |  |
| ph_poc_msb_cycle_present_flag | u(1) |
| if( ph_poc_msb_cycle_present_flag ) |  |
| ph_poc_msb_cycle_val | u(v) |
| } |  |

Proposed Solution

To solve the above asserted problem, it is proposed to infer the value of ph_poc_msb_cycle_present_flag to be equal to 0 when it is not present. Furthermore, it is suggested to editorially update the semantics of ph_poc_msb_cycle_present_flag by adding “... and used for deriving the value of PicOrderCntMsb”.

ph_poc_msb_cycle_present_flag equal to 1 specifies that the syntax element ph_poc_msb_cycle_val is present in the PH and used for deriving the value of PicOrderCntMsb. ph_poc_msb_cycle_present_flag equal to 0 specifies that the syntax element ph_poc_msb_cycle_val is not present in the PH. When not present, ph_poc_msb_cycle_present_flag is inferred to be equal to 0. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 and there is a picture in the current AU in a reference layer of the current layer, the value of ph_poc_msb_cycle_present_flag shall be equal to 0.

Reference

- B. Bross, et al., “Versatile Video Coding (Draft 9)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-R2001(JVET-Q2001-vA), 18th Meeting: teleconference, 15–24 April 2020.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).